User Manual for C64-Copy

========================

I ) General Information

-----------------------

Now your Dreams become reality !

With the software tool C64-Copy you are able to transfer files between the

ATARI ST and the C64. Both transfer directions are possible.

There are many possibilities of usage for such a tool. For example you can

transfer old text files wich had been written on the C64 in order to print

them out with a word-processor for the ATARI ST. With a word-processor

like Signum II you get a printing quality which is not possible on a C64 !

You are also able to run your old software on a C64 emulator, so you don't

have do by new ones.

If you own special hardware for your C64 like an EPROM burner you won't

have to buy another one for the ATARI ST. Just transmit the data to the

C64 and burn them with your old EPROMmer.

The whole ATARI ST program and all transmit procedures of the C64 program

are written in assembly language. This makes the transmission running so

fast.

II ) Hardware Configuration

---------------------------

First of all you need an ATARI ST with a floppy disc drive and screen and

a C64 also with floppy disk drive and screen. And as the most important

thing you need a transmission wire.

By the way : You can get the instructions for building this wire for a

little fee of DM 20,- ( about 10 $ or 5 ) from me ( see the address at

the end of this file ).

ATTENTION :

-----------

C64-Copy will work in high and medium resolution. So you can use either a

monocrome or a colour screen. But during the transmisson all interrupts

are switched off. Even the interrupt which causes a reset when the screen

is changed won't work. It's important because many colour screens genetate

their high-voltage from the computer's syncronization and the automatical

reset while changing the screen will prevent the colour screen from the

higer syncronization frequency of the monocrome mode. So if you change the

screen during data transmisson your colour screen will be burn out !

III ) Software Configuration

----------------------------

You need for the ATARI ST the program 64_COPY.PRG with the resource files

64_COPYM.RSC ( monocrome ) and 64_COPYF.RSC ( colour ). And for the C64

you need the Program ST COPY. Unfortunately it doesn't make any sense to

copy this program onto the ATARI ST disk. There are two possibilities. You

can copy it from a friend or you print out the file BASICLST.C64. This

file is a listing of ST COPY so you can type it in your C64 ( It's only

104 BASIC lines ! ).

IV ) Instructions for the ATARI ST program

------------------------------------------

Please start the program 64_COPY.PRG on your ST. The manu bar at the top

contains some menu titles.

There is only the entry ABOUT C64 COPY in the menu DESK. If you enter this

you will get some information about my person.

In the menue FILE are some more entries. With LOAD FILE you are able to

load a file into the internal buffer. This buffer has only a size of 64

KByte so you can't load files which are larger than 64 Kbyte. With SAVE

FILE the contents of the buffer is written onto disc. You have to select

the filename with the TOS internal File-Selector-Box.

Remember : You can only put data into the buffer if it is empty and to get

data out of the buffer it must be full. To clear the buffre use simpy the

function CLEAR BUFFER.

The menu entry END will leave the program ( of cause ).

With the menu title DATA TRANSMISSON the connection to the C64 will be

done. You will get data from the C64 into the buffer with RECEIVE DATA and

SEND DATA sends the contents of the buffer to the C64.

The menu title OPTIONS contains two entries. BUFFER INFO shows how many

byte of the buffer are used.

And with ASCII CONVERT you can adapt the contents of the buffer to the two

different ASCII standards.

V ) Instructions for the C64 program

------------------------------------

Using the C64 program is very easy too. But there is no buffer because

with 64 KByte memory you can't make such a big buffer. So the floppy disc

will work during data transmisson. Because of this the speed of data

transmisson depends on the speed of the floppy disk drive. You should use

a floppy speeder.

With SEND FILE and RECEIVE FILE you can send data to the ST or get data

from the ST. The first two bytes of an C64 PRG file are the starting

address of this program. The ATARI ST can't use this address so you have

to say if you want to transmit these two bytes or not.

To quit the program choose EXIT PROGRAM.

If you have choosen RECEIVE DATA on the ATARI ST the ATARI awaits some

data from the C64. The ST won't react till it gets an EOF from the C64.

Using SEND EOF TO ST will make the ST able to get new instructions.

VI ) Apendix

------------

If you have questions about C64 Copy write them to :

Thomas Grube

Heidkamp 11

D-2200 Klln-Reisiek

West Germany

Send the DM 20,- for the intructions for building the transmisson wire

also to this address. By the way : You can also get the Sources from me.

All programs are public domain and it is allowded to copy them.

Thomas Grube