DECcomm

A DEC Rainbow implementation of a FOSSIL driver

USER'S  MANUAL

Version 5.01

August 22, 1988

IMPORTANT NOTICE

--------------------------------------------------------------------

DecComm Asynchronous Communications Driver Revision 5.01

Copyright (C) 1988 by VEP Software, ALL RIGHTS RESERVED

You are  free  to use and distribute this software in its unmodified

form for ANY  NONCOMMERCIAL  USE  WHATSOEVER.   Modified versions of

this software are NOT  to be distributed without the express written

consent of the author.   You  are  not  allowed to sell or otherwise

make money from the use of  this  software  without specific license

from the author.  You are invited  to  offer  contract  work  to the

author, who can be reached at:

Vincent E. Perriello

VEP Software

111 Carroll Street

Naugatuck, CT 06770

or via Fidonet mail to Network 141 Node 491. Information or comments

about the functioning of this driver are most welcome.

DECcomm - a Rainbow implementation of a FOSSIL driver              Page 1

This   document  describes  the  function  and  usage  of  the    DECcomm

comm/keyboard/ TSR driver for the DEC Rainbow.  It is  intended  for  use

with Opus,  SEAdog,  BinkleyTerm  or  other  communications packages.  It

implements all the  machine-specific  code that either Opus or the SEAdog

mailer needs.  It's  a  FOSSIL driver.  That stands for "Fido-Opus-SEAdog

Standard Interface Layer".  Please feel free to contact me if you want to

know more about how to write  a  FOSSIL driver, or code that makes use of

FOSSIL services.

Starting  with  Version  3.0,  DECcomm's  version  numbering  method  has

changed.  Now the "Major Version" (the number  to the left of the decimal

point)  will  always  refer  to  the  Revision  Level  of    the   FOSSIL

specification  that  is  supported  by  that version of DECcomm, and  the

"Minor  Version"  will  refer  to rev level of DECcomm within that  spec.

Version  5.0  is the first version of DECcomm to claim full adherence  to

the Revision 5 FOSSIL specification.

You should run DECcomm just once before starting up Opus, SEAdog or other

FOSSIL communications package.  It's OK to do so more than once, however.

DECcomm is capable of determining whether  or  not  a  copy  of itself is

already in memory and will not clutter  up your system with "billions and

billions" of copies.  When you run DECcomm,  it  should  come  up  with a

title  screen,  tell  you  either  that  it  is loaded  or  that  it  was

unnecessary  to  do  so (one was already there), and exit  back  to  DOS.

DECcomm  should  normally  be  invoked  without  a command line argument.

However, should  you  wish  to run DECcomm at a fixed baud rate of 19,200

with CTS/RTS (this works well for USR HST's and Telebit TrailBlazers) you

should invoke DECcomm with  an  argument.    Any argument.  I invoke mine

with "DECcomm HST".

DECcomm in  its  present  form implements an extended form of the IBM INT

14H communications calls.   These extensions have been done carefully, in

such a way as  to preserve compatibility with Tom Jennings' Generic Fido.

It has been tested with  FIDO_GEN and works fine with it (though timeouts

can occur on 300 baud downloads  due  to  the output buffering).  It also

seems to work on Rainbow 100A's (don't  go  trying  it with FIDO_GEN 11w,

though.  The problem with Fido 11w on  100A's is too nasty to be fixed by

software alone).

DECcomm - the Rainbow implementation of a FOSSIL driver            Page 2

DECcomm 5.01 User's Guide

The  extensions  to  the  "basic" FIDO_GEN spec implement fully  buffered

input  and  output  with large buffers, for use in high-performance  file

transfer  operations (ZMODEM and SEAlink, for example) and to make a  BBS

look  much  faster than it actually is by hiding disk I/O behind  output.

Some  basic  screen  primitives  (get and set cursor location and write a

character to  screen)  are  also  included.  Finally, there is a keyboard

routine that includes  much of the IBM PC keyboard scan codes, for use by

Opus and other packages, such as BinkleyTerm and the SEAdog mailer.

The key code mapping supported is:

1. (CTRL+SHIFT+any alpha or number key) maps to the same codes as

(ALT+key) on IBM, e.g. CTRL-SHIFT-A = ALT A, etc.

2. Function keys:

(DEC)		(IBM)		(DEC)		(IBM)

Interrupt  =	  F1		AddtnlOptions =   F6

Remove	   =	  F2		F17	      =   F7

Cancel	   =	  F3		F18	      =   F8

MainScreen =	  F4		F19	      =   F9

Exit	   =	  F5		F20	      =  F10

Both the CTRL+function key and ALT (CTRL+SHIFT) + function key map

the same as on IBM.

Find	   = 	 Home		Remove	      = Delete

Select     = 	 End 		PrevScreen    = PageDown

InsertHere = 	 Insert		NextScreen    = PageUp

Also, arrow keys map the same as IBM.

DECcomm - the Rainbow implementation of a FOSSIL driver            Page 3

DECcomm 5.01 User's Guide

The numeric keypad is mapped to work with any terminal package

that maps the VT100 keypad like BinkleyTerm or ProComm. This is

what it looks like:

+--------+--------+--------+--------+

|        |        | Shift- | Shift- |

|   F1   |   F2   |   F1   |   F2   |

|        |        |        |        |

+--------+--------+--------+--------+

|        |        | Shift- | Shift- |

|   F3   |   F4   |   F3   |   F4   |

|        |        |        |        |

+--------+--------+--------+--------+

|        |        | Shift- | Shift- |

|   F5   |   F6   |   F5   |   F6   |

|        |        |        |        |

+--------+--------+--------+--------+

|        |        | Shift- |        |

|   F7   |   F8   |   F7   |        |

|        |        |        | Shift- |

+--------+--------+--------+  F10   |

|                 | Shift- |        |

|       F10       |   F9   |        |

|                 |        |        |

+-----------------+--------+--------+