Notes on Windows Kernel Debugging

---------------------------------

Windows implements a low-level debugging interface called Kernel

Debugging (WKD) which obtains control very early in the startup

sequence of Windows.  The program WDEB386 shipped with the Windows SDK

is an example of one such program; 386SWAT is another.

When Windows sees that a kernel debugger is present, it informs the

debugger about a number of events as they occur, principally parameter

errors made by running Windows programs as they call Windows API

functions.	For example, if a program calls EnableWindow with an

invalid window handle (say, zero), this error is passed on to the WKD

for processing before returning to the calling program.  In this way,

a great many errors can be caught before they can cause any harm.

By default, 386SWAT is a WKD.  If, for some reason, you wish to

disable this feature, place the keyword NOWINK into your 386SWAT

profile.

At the current time, you must have a monochrome adapter and monitor

attached to your system in order for WKD services to be allowed by

386SWAT.  386SWAT tests for a mono card and disables WKD if not found.

New Command Line Actions

------------------------

* To enable or disable WKD services entirely, use

WKD [ON|OFF]

This option is available outside Windows only.  If you have

specified NOWINK in the 386SWAT profile, this option has no effect

-- you must remove that profile option and reboot the system to

enable WKD services.

* To change the response to a parameter error, use

WKD [NOISY|QUIET]

If NOISY is specified, then each parameter error is followed by the

prompt "Break, Ignore, Quiet?" to which you must respond B, I, or

Q.

A response of Q, is equivalent to setting WKD QUIET after which

parameter errors are logged to the screen, but execution does not

stop for any more parameter errors until you break into 386SWAT and

type WKD NOISY on 386SWAT's command line.  This is the default

state, so you'll see parameter errors fly past on the mono screen

without pause.  I recommend that when you are debugging your own

code, use WKD NOISY; otherwise, use WKD QUIET.  This is because many

commercial packages contain so many parameter errors that you'll

constantly be interrupted with a BIQ prompt.

A response of I, ignores this one error only and continues

execution.

A response of B, triggers a single step breakpoint at an IRETD

inside 386SWAT.  Tracing over this instruction returns to the

instruction inside Windows immediately following the call to

386SWAT (typically an INT 41h).  Tracing from this point on

eventually gets you back to a call to LogError which was called by

the Windows API function which encountered the invalid parameter,

and then back to the application which called the Windows API with

an invalid parameter.  If the application is Win32, then you'll have

to trace back through a thunking layer before you get to the

application.	This can be quite tricky.  At some point, I'll try to

put in some stack tracing to aid this process.

* Sometimes Windows and/or applications make direct calls to LogError

as opposed to indirect calls as part of a parameter error.  These

calls are uncommon, so there is a separate switch to control

386SWAT's response to such an event.  Use

WKD LOGERROR [ON|OFF]

to change the response of stopping at an IRETD inside 386SWAT or

not.	The default setting is OFF.

* Sometimes an application is bad enough that it triggers an

unrecoverable fault (typically a GP or Page Fault) and it must be

terminated.  Windows gives the WKD a crack at it first to which

386SWAT responds by stopping in the application at the faulting

instruction, thus giving you the opportunity to debug the problem.

If you wish to tell Windows to handle the fault itself (typically

by terminating the application), use

WKD FAULT SKIP

If you do not want to receive any more fault messages from Windows,

use

WKD FAULT OFF

To restart receiving fault messages, use

WKD FAULT ON

* The keystroke Ctrl-K brings up a menu from which you can choose to

display various internal Windows structures.	At the moment, the

only one which has been fleshed out is the display of the Windows

Global Heap (WGH).  The WGH contains one entry for each globally

allocated region of memory.  For example, the various code and data

segments of KRNL386.EXE are global allocations and have entries in

this table.  To scroll through this table, use the usual up and down

arrow keys as well as Page Up and Page Down.

To search through the WGH for a specific entry, use the SGH command:

SGH [/b|/s|/h|/o] [/c] expr

The expression (expr) entered is interpreted as a base address if /b

is specified, size if /s, handle if /h, and owner if /o.  If /c is

specified, the search continues from the currently displayed entry;

otherwise, the search starts at the top of the heap.	If the value

is found in the WGH, the WGH is displayed with the matching entry at

the top of the screen.  If you wish to repeat the search (perhaps

becayse there are multiple entries with the same owner), either

retype the command (and specify /c so the search continues with the

next entry after the one at the top of the screen), or start the

command with a slash so it is not erased after successful execution.

If the value is not found in the WGH, an error message is displayed.

* If Invalid Page Faults are being trapped by SWAT's VxD (see

SWATVXD.DOC for more details), use the IPF command to control how

these events are to be handled.

IPF [/d] [/s] [/r] expr

where /d tells SWAT not to display a message on the mono screen

describing this event, /s tells SWAT not to stop when this event

occurs, /r tells SWAT to remove this entry from its local tables,

and expr is an expression which evaulates to a linear address

corresponding to the Invalid Page Fault.

* To display the memory pointed to by a selector as a Module Database,

use

MDB expr

* To display the memory pointed to by a selector as a Task Database,

use

TDB expr

* Certain commands usually preceded with a dot are automatically

passed to Windows for processing by WIN386, assuming Windows is

active and WIN386 services are available.  These commands include

?		Display a Help screen with the available commands

.?		Display a list of registered dot commands (e.g., .M)

.R [#]	Display the registers of the current thread (or thread #)

.VM [#]	Display the complete VM status of the current VM (or #)

.VC [#]	Display the current VM's (or #) control block

.VH [#]	Display a VMM linked list, given list handle

.VR [#]	Display the registers of the current VM (or #)

.VS [#]	Display the current VM's (or #) virtual stack

.VL		Display list of all valid VM handles

.DS		Display protected mode stack with labels

.VMM		Display menu of VMM state info

.<dev_name>	Display device-specific info

Profile Options

---------------

Because 386SWAT is a WKD, it is given much more information to

process.  Much of that data is stored in its internal log file

(acessible via Ctl-F10) which means that you should increase the log

file's default size.  You might experiment with different values, but

I suggest using something on the order of LOGSIZE=100000.

During Windows startup while still in RM/VM, Windows loads the real

mode portion of all of the VxDs to be used this Windows session.  At

this time, Windows tells the WKD about these VxDs and where they will

be loaded in memory when in PM, and 386SWAT logs this information in

two ways.

The first way is to write a line to the mono display with the data

about each segment (code or data) of the form

Loaded VxD <VxDname>, module <Modname> at <address> len <length>

The second way is to append a symbol to 386SWAT's symbol table of the

form <VxDname>_Code<nnnn> or <VxDname>_Data<nnnn>, where <nnnn> is a

decimal sequence number starting at 1 and is used to distinguigh one

code/data segment from another.  By convention in Windows, code

segments are in selector 0028h and data segments are in selector

0030h.	These symbols are useful when debugging VxDs.

In order to save this information, you need to tell 386SWAT how much

space to reserve.  The profile option WKDLS (Load Segments) is used

for this purpose.  The actual value depends upon how many VxD code and

data segments are loaded on your system.  I use WKDLS=700.	If you

have no interest in debugging VxDs, omit this profile option as the

default value is zero which reserves no space.	You might also want to

increase the profile value for SYMSIZE.  I use SYMSIZE=700000 (you do

have 32MB of RAM, don't you?).

Debugging DLLs

--------------

The Windows SDK (3.1x or 95) contains separate copies of various

system DLLs each of which which contain more messages and checks on

Windows execution.  I haven't played with these much, so you're on

your own, but it may be very worthwhile.  They also contain symbol

files which I haven't put in code to load as yet.

SWAT VxD

--------

This version of 386SWAT no longer requires that you load SWATVXD.EXE

as a resident program (thus saving you a few bytes of DOS memory), but

the VxD must be in same directory as 386SWAT.LOD.  The VxD is now

loaded by 386SWAT when Windows starts up.  See the file SWATVXD.DOC

for more information on how to configure the VxD.

Loading 386SWAT

---------------

If you are running 386MAX as your memory manager, you could load

386SWAT from the 386MAX profile with the LOAD= keyword.  However,

because this mechanism doesn't provide for 386SWAT to have a DOS

footprint and such a footprint is needed for WKDing, I do not

recommend loading SWAT this way anymore.  Instead, load 386SWAT as a

device driver on the line which immediately follows the line which

loads the memory manager.  386SWAT must load in low DOS memory, so

don't try to load it high.  The low DOS memory footprint of 386SWAT is

about 2KB.