PRINTGL 1.16 REFERENCE

(C) Copyright Ravitz Software Inc. 1990,1991

Ravitz Software Inc.

P.O. Box 25068

Lexington, KY  40524-5068

USA

p-1

Contents -------------------------------------------------------------

Introduction ..................................................... 2

License and Registration                                           3

What Is New ...................................................... 4

Installation                                                       5

Operation ........................................................ 6

Syntax                                                             7

Messages ......................................................... 8

Overview of Options                                                9

/A - Location Option ............................................. 9

/C - Pen Color Option                                             10

/D - Output Destination Option .................................. 10

/F - Output Format Option                                         11

/I - Scaling Point Location Option .............................. 13

/J, /K - Printer Code Prefix, Suffix Options                      13

/L - Page Layout Option ......................................... 14

/M - Magnification Option                                         15

/N - Page Number Option ......................................... 15

/O - Origin and Orientation Option                                15

/P - Plotfile Option ............................................ 16

/R - Internal Resolution Option                                   16

/S - Pen Shading Option ......................................... 16

/T - Temporary File Option                                        16

/W - Pen Width Option ........................................... 17

/X - Suppress Messages Option                                     17

/Y - Y/D Switch Setting Option .................................. 17

/Z - Chord Angle Option                                           17

PrintGL Menu Interface (PMI) .................................... 18

PMI Specific Functions                                            18

PMI Input and Output Choices .................................... 19

PMI Plot Options                                                  19

PMI Pen Options ................................................. 19

PMI Page Layout Options                                           20

PMI Other Options ............................................... 20

Display Output                                                    21

Canon BJ Printers ............................................... 21

Canon LBP Printers                                                21

Epson 9 Pin Printers ............................................ 22

Epson, NEC 24 Pin Printers                                        22

HP-GL Output .................................................... 22

HP LaserJet, DeskJet                                              23

HP PaintJet ..................................................... 23

HP QuietJet                                                       23

IBM ExecJet ..................................................... 23

IBM LaserPrinter                                                  23

IBM Proprinter .................................................. 23

IBM Proprinter X24 and Compatible Printers                        24

IBM Quietwriter 2 and 3 ......................................... 24

Kodak Diconix Color 4                                             24

PostScript Printers ............................................. 24

GEM .IMG Bit Map Output                                           25

ZSoft .PCX Bit Map Output ....................................... 25

HP-GL Commands                                                    26

HP-GL Character Sets ............................................ 35

Compatibility and Technical Information                           37

User Support .................................................... 38

p-2

Introduction ---------------------------------------------------------

PrintGL is a shareware pen plotter emulator for IBM/DOS systems.  It

prints  or displays an HP-GL (Hewlett-Packard Graphics Language - HP

7475  and 7440/17440 subset) plotfile  on most PC graphics  devices.

HP-GL  is  widely  supported by CAD,  drawing,  plotting, and  other

graphics programs including AutoCAD, Generic CADD, MathCAD, SAS, and

Schema.  PrintGL will print on  Epson, IBM, and NEC compatible 9 and

24  pin  printers,  HP  LaserJet,  DeskJet,  PaintJet, QuietJet, and

plotters,  IBM  ExecJet, Quietwriter  2  and  3,  LaserPrinter,  and

plotters, Canon LBP and  BJ printers,  Kodak Color 4, and PostScript

printers.  It will also display plots with a CGA, EGA, VGA, enhanced

VGA, or HGC, and output a ZSoft .PCX or GEM .IMG bit map file.

Even  if  your  graphics program supports  your  printer,  you  will

probably  find that PrintGL  is faster,  gives better print quality,

and/or  gives more  formatting flexibility.  PrintGL  uses  the best

graphics modes available for each printer that  it supports and uses

transfer  data compression  to improve print speed on many printers.

And it gives options  such as orientation, magnification,  position,

and pen  width, color, and  shading.  If  you need  printed graphics

output from personal software, you can output  HP-GL and use PrintGL

to do the printing.  This gives immediate support to a wide range of

printers.  And it lets you use the HP 17440 command  set while still

supporting lower function plotters.

PrintGL interprets all of the HP 7475  and 7440/17440 (ColorPro with

GEC)  command  set  that  is  applicable  to  plotfiles  except  for

character set 8 (Katakana).  None of the digitize, output, or device

control commands (except  plotter on and off) are supported as these

are only  used with direct control of a plotter.  And the error mask

command is not supported.  In addition to the HP 7475 and 7440/17440

commands, PrintGL handles paper feed commands and proportional fonts

from the HP 7550 command set.

PrintGL  can be  run  from the DOS command line (and  .BAT files) or

from PrintGL Menu Interface (PMI). PMI is a menu driven program that

lets you choose PrintGL options, select a list of plotfiles, and run

PrintGL. This simplifies the selection of PrintGL's many options and

lets you  run multiple plotfiles without intervention.  PMI lets you

save three  configurations,  so  after initial  setup  you  can  use

PrintGL with minimal effort.

This program is produced by a member of the Association of Shareware

Professionals  (ASP).  ASP  wants  to  make sure  that the shareware

principle  works  for   you.   If  you  are  unable  to  resolve   a

shareware-related  problem  with  an ASP  member  by contacting  the

member directly, ASP may be able to help. The ASP Ombudsman can help

you resolve a  dispute or problem  with an ASP member, but  does not

provide technical support for members' products. Please write to the

ASP Ombudsman  at  P.O.  Box  5786,  Bellevue, WA  98006 or  send  a

Compuserve message via easyplex to ASP Ombudsman 70007,3536.

p-3

License and Registration ---------------------------------------------

PrintGL (including PrintGL Menu Interface) is provided as is.  There

are no warranties expressed or implied.

PrintGL is distributed as  shareware.  You may  use PrintGL  without

charge on a trial basis to determine its suitability for you. If you

continue  to use it  after  your  evaluation, a $40  registration is

requested.  This registration  covers  use  by a  single person  (on

multiple computers) or  installation  on  a  single computer (may be

used by  a group of people).  There is  no cost  to use a later 1.**

version after registering any 1.** version.

You may distribute PrintGL  as a stand alone product if you keep the

entire   package   together,  unchanged,   including   this  license

explanation, and do not charge more than $10. You may not distribute

PrintGL as part of another product.

To  register your copy of PrintGL,  send your name, address, and $40

check, and specify PrintGL 1.16, to:

Ravitz Software Inc.

P.O. Box 25068

Lexington, KY  40524-5068

USA

You will be sent a printed manual and current disk (specify 3 1/2 or

5  1/4).  If you previously registered PrintGL, you may send  $15 to

get a current disk and manual.

Multi-user  licenses are  $40  for the  first  copy (with  disk  and

manual) and  thereafter $30 per copy with disk and manual or $20 per

copy without disk or manual.

If you are interested in using PrintGL  as  a  printer  driver  in a

commercial program,  send  a  note.  Special  license agreements are

available for this.

For users outside the USA, please make checks payable in US dollars,

and except for Canada, please  use a check or money  order from a US

bank if possible.

p-4

What Is New ----------------------------------------------------------

For version 1.16:

clarification of Canon BJ / IBM ExecJet drivers

fixes bug in BJ130 driver (/FJ)

HP QuietJet, ZSoft .PCX, Kodak Color 4 output formats

compression mode drivers for 24 pin printers

the PaintJet driver now uses data transfer mode 1

new LaserJet 2P/3, DeskJet driver (/FI) uses transfer mode 2

= symbol replaced by ! for alternate drivers (= still works)

~ in /F option suppresses color processing on color printers

/J and /K option lets you send codes to the printer before and

after the usual data is sent

/FB (flat bit map) has been dropped

/FV+ (720x540 EVGA) has been dropped - /FV+ is now 800x600 EVGA

For version 1.14:

new license agreement, printed manual

HP-GL, PostScript, Canon LBP, Canon BJ, IBM ExecJet output

HP 7550 polygon commands, proportional fonts are supported

destination files can be appended

improved clipping window handling

improved line dash patterns

improved margin precision

revised colors (generally backward compatible)

For version 1.12:

HP 7475 command set support

GEM .IMG output format

alternate LaserJet driver is faster and uses less printer memory

For version 1.07:

plotfile masks are supported

/X[S[F]][U] option suppresses messages

PrintGL Menu Interface is included with the PrintGL package

For version 1.05:

DOS environment variable PRINTGL can be used to set defaults

For version 1.04:

/F9* supports NEC 24 pin printers in 360x360 dpi mode

For version 1.02:

any 800x600 VGA is supported with /FV*modenum

The latest version of PrintGL is available from the support bulletin

board at  606-268-0577  (1200/2400,N,8,1  24  hours/day,  XMODEM  or

YMODEM (1K XMODEM) download), packed in .ZIP format.

p-5

Installation ---------------------------------------------------------

The PrintGL package includes these files:

PRINTGL.EXE  program

PRINTGL.DOC  documentation

CHARSET.PLT  HP-GL character set plotfile

PMI.EXE      menu interface program

SAMPLE.PLT   sample plotfile

README.*     quick information

To  install PrintGL copy the distribution files to your working disk

or directory.

PRINTGL.EXE is a stand alone executable  file.  There  are no  extra

printer driver files or overlays.

PRINTGL.DOC is this manual.  You can print it with "COPY PRINTGL.DOC

PRN" (39 pages).  The  HP-GL character sets may be printed with  the

command  PRINTGL CHARSET.PLT /M1 /AO0,0 and a /F option  specific to

your printer, and inserted in the manual.

PMI.EXE is the executable file for PrintGL Menu Interface. PMI needs

to be able to  find  itself  (PMI.EXE)  and  PRINTGL.EXE, so both of

these files should be in a directory that is in your DOS PATH.

SAMPLE.PLT is a sample plotfile to help you try out PrintGL.

The README.* files  are not needed  to  run PrintGL,  but  they must

accompany the rest of the files if you distribute the package.

p-6

Operation ------------------------------------------------------------

PrintGL reads a plotfile, does arc,  character, fill, etc.  to  line

conversion,  stores the data in  a  coordinate list, and  if needed,

puts the list on disk. It then reorients the data and determines the

plot size. Next the data is resized and relocated. For vector output

formats (HP-GL, PostScript) data is output from here. For bit mapped

devices  (most printers, displays, bit maps) the coordinate list  is

rasterized and output in swaths.  You are likely to  see the printer

print a swath and stop while PrintGL is processing the next swath.

When you display a plot, press Esc to return to the DOS text screen.

Ctrl-Break  stops  PrintGL at the next break  point.  Ctrl-Break  is

ignored when PrintGL is waiting for user input.

PrintGL works within a printer's current margins  and starts at  the

current  vertical print  position.  If  you decline  the  form  feed

suboption  of the  /L option then PrintGL leaves the  printer at the

start of the  line directly below the plot.  All printing is done in

printer lines that cover the full height of the printhead.

PrintGL will not automatically change a printer's operating mode. On

multi-mode printers, the  right  operating  mode must be  set before

running PrintGL or with the /J option.

Internally PrintGL  uses two byte  coordinates  on  a 1016 dots/inch

grid. This allows for plots with coordinates of over 30 inches - far

larger than an HP 7475 can handle. However, if you are using PrintGL

to emulate a large plotter, you may overflow the internal coordinate

system.  This condition  is  not checked, and  it will cause garbage

output.  You can change the internal resolution to 508 dots per inch

(giving 60 inch maximum plot sizes) with the /R option (/R508). This

only helps  store the  original plot  internally.  The  next step in

PrintGL converts  these  coordinates  to the  requested size  in the

resolution of the output device, and these coordinates must also fit

in the two byte coordinate list.

p-7

Syntax ---------------------------------------------------------------

The calling syntax of PrintGL is:

PRINTGL plotfile /option1 /option2 ...

Spaces between parameters  are  not required.  The  plotfile  may be

specified with  the /P option if it  is inconvenient to make it  the

first parameter.

If a given option is entered more  than once, only the last entry is

used.  And if  an  option  is invalid  (but  not  incorrect),  it is

ignored.  This  lets  you set  up a  batch  file with your preferred

options  but override them as desired.  For example,  create PHP.BAT

with the line:

PRINTGL /FL /M1 /P %1 %2 %3 %4 %5 %6 %7 %8 %9

Then  PHP can be called just  like PRINTGL  but  it  defaults to  HP

LaserJet format with magnification 1.

The DOS environment variable PRINTGL  is examined by PrintGL and  if

it  is  nonblank  then  it  is  put  in  front  of  the command line

parameters.  If you  put "SET PRINTGL=/FL/M1"  in your  AUTOEXEC.BAT

then  PrintGL will default to LaserJet format with  magnification 1.

This is ignored when running under PrintGL Menu Interface.

If you do not enter any parameters or if any parameter is incorrect,

PrintGL displays its option reminder screen.

Below, [ and  ] enclose optional suboptions and | separates mutually

exclusive suboptions. The options and suboptions may be in any order

except that multiple numeric suboptions are ordered. "c" refers to a

suboption character, "x" and "y"  to  suboption numbers, "i" and "j"

to  suboption integers, "b"  to  a suboption  byte, and "f" to a DOS

file or device name.

For numbers (x, y), a decimal point is allowed but  is not required,

and scientific notation is not allowed.  "-" is allowed but not "+".

For integer input (i, j), only base ten integers from 0 to 32767 are

allowed.  A  byte (b) must be a base ten integer from 0 to 255  or a

hexadecimal number  prefixed with $  from $00 to $FF.  Further range

checking is done for most options to flag unreasonable input.  If an

option has multiple numeric suboptions, these  may be separated with

commas or blanks.

********************************************************************

It  is very important to choose the right output format (/F option).

It is likely  that  the  default  output  format will  work on  your

printer, but it is equally likely  that it  will not produce optimal

results.  Please look carefully at the entire list of output formats

before  choosing one, and do not assume that because a  format works

that it is the best one to use.

********************************************************************

p-8

Messages -------------------------------------------------------------

PrintGL  displays a number of messages to  keep you informed of what

it is doing. The output looks like this.

Reading SAMPLE.PLT

Plotfile has 1 page(s)    Processing page 1    2283 data records

Overall magnification    0.9964

Print window   Horizontal   0.00   7.54    Vertical   0.00  10.14

Plot area      Horizontal   0.42   7.13    Vertical   0.61   9.54

Output format /FN    120x72 dots/inch (HxV)

Writing to device LPT3#

The first line  is the plotfile  that is  currently being processed.

The second line tells how many pages are in the plotfile, which page

is being processed, and the number of data points in that page.  The

number  of data points is the number of pen moves in the page and is

shown to give you an indication of the plot's complexity.

The  overall magnification is  either  the  magnification  that  you

specified  with the /M option or the  effective  magnification after

fitting to the print  window with /MF.  The  print window line gives

the horizontal  and vertical coordinates of the print window, always

starting  at 0,0 and in inches.  The plot area  gives the horizontal

and vertical coordinates of a box that just covers the plotted data.

This uses the same coordinate system as the print window, so you can

tell where the plot is relative to the print window.

The  output format  and resolution  are  given and  then the  output

device is  noted (except for screen plots).  If three plane color is

being used, the word "color" is displayed.  If you have a black only

printer, but the output format allows  color (this is  common with 9

and 24 pin  printers), using color  will triple the PrintGL run time

with no improvement in print quality.

During  processing, PrintGL informs  you when  it is  processing and

writing data,  and you can watch the progress from 0% to 100%, along

with the elapsed time.

p-9

Overview of Options --------------------------------------------------

PrintGL's options fall into several categories.  The  first is input

and output choices.  The input plot is chosen with /P, the plotfile,

and  /N, the page number.  The output  options  are  /F,  the output

format  (printer model), and  /D, the destination port,  device,  or

file.

The  second  category is  the page layout, which is all contained in

the /L option.  This  includes  the  size and  location of the print

window (which defines the  hard clip  limits of  the plot), and  the

option to box the window, and send a form feed.

The third category defines  how the  plot  looks  within  the  print

window.  /M,  the  magnification,  sets  the size  of  the plot.  /A

determines the location of the plot within the print window. /O sets

the  origin  and  orientation, and  is used to rotate or  mirror the

plot. /I sets the initial scaling points as if you had set them from

the front panel of the plotter.

The  fourth category defines the pens, /W for the width,  /C for the

color, and /S for the shade.

And  the last  category determines how PrintGL  processes the  plot.

This includes the temporary file /T,  the default and minimum  chord

angle  /Z,   the  plotter  Y/D  switch  position  /Y,  the  internal

resolution  /R, /X which  suppresses output messages, and /J  and /K

which let you set up and reset the printer for special situations.

/A - Location Option -------------------------------------------------

/A[O][x,y] - location - default /A

The A option determines the location of the plot in the print window

by specifying a point on the plot that is to be aligned with a point

in the print window.  You can specify the  plot alignment point with

x,y in inches from the plot's origin (before PrintGL's magnification

is  applied),  or let it default to the plot  center.  For the print

window alignment point, you can default to the center or specify the

origin with O. By default, the plot center is aligned with the print

window center.

/A      puts the center of the plot at the center of the print

/AO0,0  puts the plot 0,0 at the print origin

p-10

/C - Pen Color Option ------------------------------------------------

/Cc.. - color - default /CK

The  C option specifies the color  of  each  of the eight pens.  The

colors are  B for blue, C for cyan, G for green, K or L for black, M

for magenta, R for red, W for white, and  Y for yellow.  Unspecified

pens  use the last selected color (/CRGB is the same as /CRGBBBBBB).

If the output format does not allow  color or color is suppressed in

the /F option then every color except white is converted to black.

All colors are ored into the plot, starting with a white background.

White is useful for ignoring a pen and with shade mixed colors.

Additional colors can be created with shade mixing. These colors use

shading patterns  to  mix  the  eight pure colors.  The format for a

mixed color is  .??  where each ?  represents any of  the eight pure

colors.  The first  color is plotted  with the pen's shading pattern

(/S option)  and the  second  is plotted  with  the  inverse shading

pattern.  If  the shade value is  0 (solid) it is replaced by 2 (one

pel checkerboard) which produces the most uniform mixed colors.

The mixed colors need line widths of at least 2 (4 for 2 pel shading

patterns) to look good, and they may  have visual artifacts on edges

near 45 degree angles.  Multipass dot  matrix formats may not handle

color mixing very well.

Only a few of the printers that  PrintGL supports can handle  color.

These are marked with ~ in the printer list (see /F option). Shading

may be used to emulate color on black only printers (see /S option).

/CK         sets pen 1..8 to black

/CKR.MB.KY  sets pen 1    to black

pen 2    to red

pen 3    to magenta on blue

pen 4..8 to black on yellow

/D - Output Destination Option ---------------------------------------

/D[+]f - destination - default /D1

The D option specifies the output printer port, device, or file. The

+ suboption causes files to be appended  rather than overwritten (it

has no effect on printer port or device output).  1,  2, or 3 causes

PrintGL to  use the BIOS printer routines,  bypassing DOS.  This  is

normally faster than using the DOS device (LPT1, LPT2, LPT3), but it

may cause problems with print spoolers or multitasking systems.

If your  printer  is  attached  to  a  serial port, specify  /DCOM1,

/DCOM2, etc. PrintGL cannot set up the port. This is usually done in

the AUTOEXEC.BAT with MODE (probably MODE COM1:9600,N,8,1,P).

/D is ignored for display output.

/D2          sends output to the second parallel port via BIOS

/DLPT3       sends output to the DOS device LPT3

/D+TEMP.PRN  appends output to file TEMP.PRN

p-11

/F - Output Format Option --------------------------------------------

/Fc[-�+[b]|*|i,j][!][~] - output format - default /FN~

The F option specifies the output format. -, +, and * are resolution

modifiers,  !  selects a compression mode  driver, and  ~  turns off

color processing on  color printers.  -, +,  *, !, and ~ are allowed

even where  they have no effect.  The display  mode  numbers  may be

entered in hex by prefixing them with $. For bit maps you can choose

a  specific  resolution.  Resolutions  are  given  in  horizontal  x

vertical dots/inch.

Drivers marked below with ! have a compression mode driver, selected

with  "!".  This may be very useful, but it depends on your  printer

and computer. These drivers compress the output data by breaking bit

map strips  with  printhead  positioning commands.  This saves  data

transmission time  but uses more processing time.  So  you  may find

that on a slow computer they take more time than the regular driver.

Some  printers may spend more time positioning the printhead than is

saved by not sending the data.  And some printers  may  not position

the printhead as accurately as needed for good results.

Drivers marked below with ~  are  color devices.  Using ~ suppresses

color processing, substituting black for all colors except white.

/F1       IBM Proprinter .................................. 120x72

/F1+      IBM Proprinter (2 pass)                           120x144

/F2       IBM Quietwriter 2 ............................... 240x240

/F3       IBM Quietwriter 3                                 240x240

/F4-      IBM LaserPrinter (PPDS mode) .................... 150x150

/F4    !  IBM LaserPrinter (PPDS mode)                      300x300

/F5    !  IBM X24 compatibles alternate 1 ................. 180x180

/F5+   !  IBM X24 compatibles alternate 1 (2 pass)          360x180

/F6    !  IBM Proprinter X24 .............................. 180x182

/F6+   !  IBM Proprinter X24 (2 pass)                       360x182

/F7-   !  IBM ExecJet, Canon BJ (Proprinter mode) ......... 180x180

/F7    !  IBM ExecJet, Canon BJ (Proprinter mode)           360x360

/F8       HP-GL .......................................... 1016x1016

/F9    !~ NEC 24 pin printers                               180x180

/F9+   !~ NEC 24 pin printers (2 pass) .................... 360x180

/F9*   !~ NEC 24 pin printers (4 pass)                      360x360

/FC       CGA display ...................................... 62x25

/FD-      Canon LBP (ISO mode)                              150x150

/FD    !  Canon LBP (ISO mode) ............................ 300x300

/FE     ~ 128K EGA color display                             62x45

/FG     ~ GEM .IMG bit map ................................ 100x100

/FGi,j  ~ GEM .IMG bit map                                    ixj

/FH       Hercules graphics card display ................... 70x45

/FI-      HP LaserJet 2P/3, DeskJet                         150x150

/FI       HP LaserJet 2P/3, DeskJet ....................... 300x300

/FJ-      Canon BJ (BJ130 mode)                             180x180

/FJ       Canon BJ (BJ130 mode) ........................... 360x360

/FK     ~ Kodak Diconix Color 4                             192x192

/FL-      HP LaserJet compatible printers ................. 150x150

/FL    !  HP LaserJet compatible printers                   300x300

p-12

/FM       128K EGA monochrome display ...................... 62x45

/FN     ~ Epson 9 pin compatible printers                   120x72

/FN+    ~ Epson 9 pin compatible printers (3 pass) ........ 120x216

/FN*    ~ Epson 9 pin compatible printers (6 pass)          240x216

/FP    !~ HP PaintJet ..................................... 180x180

/FQ       HP QuietJet (PCL mode)                            192x192

/FS       PostScript printers ............................ 1016x1016

/FT    !~ Epson 24 pin compatible printers                  180x180

/FT+   !~ Epson 24 pin compatible printers (2 pass) ....... 360x180

/FT*   !~ Epson 24 pin compatible printers (4 pass)         360x360

/FV     ~ VGA display ...................................... 62x62

/FV+    ~ 800x600 VGA display - mode $64                     78x78

/FV+b   ~ 800x600 VGA display - mode b ..................... 78x78

/FX    !  IBM X24 compatibles alternate 2                   180x180

/FX+   !  IBM X24 compatibles alternate 2 (2 pass) ........ 360x180

/FZ     ~ ZSoft .PCX                                        100x100

/FZi,j  ~ ZSoft .PCX ........................................ ixj

Most  9 pin printers, including the IBM Graphics Printer,  work best

with the  /FN  drivers, even if they claim  Proprinter  rather  than

Epson compatibility.  9 pin  printers with 1/144 inch indexing, such

as the IBM Proprinter, may work better with the /F1 drivers.

Most   Proprinter  X24  compatible  printers,   including   the  IBM

Quickwriter, work best  with /F5  or /FX.  /F6  is only  for the IBM

Proprinter X24, XL24, X24E, and XL24E.

The  Canon  BJ130E, BJ10E, BJ300,  and BJ330 are compatible with the

IBM ExecJet and work best with the /F7 driver.

p-13

/I - Scaling Point Location Option -----------------------------------

/IA|R|W[N] - IP point location - default /IA

The I option sets the initial scaling points  (P1, P2 - the  scaling

points may  be set  in the  HP-GL  file  with  the  IP  command).  A

(absolute) sets  the  points to the default for the paper  size (see

below).  R fits P1 and P2 to the print window with at least .25 inch

margins  and an x/y ratio of 10/7.2 (the  same ratio as  the HP 7475

with  small paper).  W fits P1 and P2 to the print window with a .25

inch margin. N makes the margin 0 for R or W and is ignored with A.

The default scaling points depend on the paper size  (/L option) and

the plot orientation (/O option).

paper  options              default scaling pts  with RO90 command

A      /O1 /L 7.84,10.20    250,596 10250, 7796  154,244  7354,10244

A4     /O1 /L 7.60,10.88    603,521 10603, 7721    0,610  7200,10610

B      /O1 /L10.20,16.38    522,259 15722,10259  283,934 10283,16134

A3     /O1 /L10.88,15.90    170,602 15370,10602  607,797 10607,15997

A      /O2 /L10.20, 7.84    250,596 10250, 7796  154,244  7354,10244

A4     /O2 /L10.88, 7.60    603,521 10603, 7721    0,610  7200,10610

B      /O2 /L16.38,10.20    522,259 15722,10259  283,934 10283,16134

A3     /O2 /L15.90,10.88    170,602 15370,10602  607,797 10607,15997

anything else               250,279 10250,7479   279,250  7479,10250

/O3 and /O4 gives the same default scaling points as /O1 and /O2.

The default paper size is 7.54 by 10.14, the HP 7470 standard  paper

size.  This causes "anything else" values.  This is because a number

of  printers  can't handle the slightly larger HP 7475 A size plots.

You must use the /L option to get HP 7475 standard paper sizes.

/IA   uses the default scaling points

/IWN  sets the scaling points to fit the print window

/J, /K - Printer Code Prefix, Suffix Options -------------------------

/J[b,b,..] - printer code prefix - default /J

/K[b,b,..] - printer code suffix - default /K

/J lets you send codes to the printer before PrintGL's usual data is

sent and /K lets you  send codes after PrintGL's usual data is sent.

This lets  you  set up and reset the printer for special situations.

These are not needed in the normal use of PrintGL.

The  codes  are  specified  with  byte  values  (0..255  or $0..$FF)

separated with blanks or commas.  There is  a limit of 64 characters

for any option, so very long  sequences must  be handled outside  of

PrintGL.

/J27 79   sets skip perforation off  to allow plots  larger than one

page on most continuous form printers

/K10 10   sends two line feeds after the plot

p-14

/L - Page Layout Option ----------------------------------------------

/L[B][F][R][x,y[,x,y]] - page layout - default depends on printer

The L option defines the page layout.

B boxes the plot at the print window edge using pen 8.

F  causes a form feed after the plot.  This  is ignored for bit maps

and display output.

R reverses  the colors  on displays and bit  maps  and is  otherwise

ignored. This gives the equivalent of a photographic negative.

The first optional x and y are the  print window width and height in

inches  rounded to  .01.  These define the hard  clip limits.  These

values are not checked against your printer's capabilities.  If  you

specify  a print window that exceeds the  output  device limits, the

results are unknown.  You may need  to reset your printer's margins,

maximum  line  length, or page length to get large  plots.  The size

needed  for a given width  and height exceeds x and  y  by 8 pels to

allow for line widths.

The  second optional x and y are the left and top  margins, measured

from the  printer's left margin and current vertical position to the

print window.  The  top  margin is modified  to match  the printer's

native indexing, but is accurate to .02 inch.  The margins for HP-GL

and PostScript  are  measured from  the device's 0,0  (left, bottom)

since that  is the starting point for  these  devices.  Margins  are

ignored for display output and bit maps.

Each printer has a default page layout.  The default print window is

7.54x10.14  for printers  and  bit maps and 10.14x7.54 for HP-GL and

displays. (150 dpi page printer formats default to a height of 10.11

to keep the picture on one page.) These correspond to HP 7470 A size

paper. This is the done because some printers can't handle the extra

.06  inch  length of the HP 7475  A  size.  The margins are  printer

specific to compensate for different default margins.  In evaluating

user  options,  PrintGL  processes  each  option and  when  finished

reprocesses the default page layout for the last chosen  printer and

then reprocesses the last user /L option.  This procedure gives good

default values so that your /L option can  change the letter or size

suboptions and still have good margins.

/LB7.5,3,.5,0   makes the  print window 7.5 inches wide and 3 inches

high,  with a .5 inch left margin and no top margin,

boxes the window, and does not do a form feed

/LF7.54,10.14   sets HP 7470 A size paper

/LF7.54,10.74   sets HP 7470 A4 size paper

/LF7.84,10.20   sets HP 7475 A size paper

/LF7.60,10.88   sets HP 7475 A4 size paper

/LF10.20,16.38  sets HP 7475 B size paper

/LF10.88,15.90  sets HP 7475 A3 size paper

p-15

/M - Magnification Option --------------------------------------------

/M[F]x - magnification - default /MF.9

The M option  specifies  the magnification.  F fits the plot  to the

printer  window size.  The default is /MF.9 which makes the plot 90%

of the largest  size that will fit in the printer window.  /M1 gives

the same size plot as an HP 7475 plotter.

/MF.9  fits the plot to the print window and shrinks it 10%

/M1    plots at the same size as an HP 7475

/M.5   plots at half the size of an HP 7475

/N - Page Number Option ----------------------------------------------

/N[i] - page number - default /N

Ni says to print only page i of the plotfile.  If you do not specify

i then all pages in the plot will be printed (this is the default).

/N   prints all pages in the plotfile

/N3  prints only page 3

/O - Origin and Orientation Option -----------------------------------

/O[1|2|3|4[L]] - origin, orientation - default /O

The O  option sets the printer's origin and orientation.  The origin

is one  of  four  corners numbered 1..4 for upper  left, lower left,

lower right, and upper  right.  The  L  suboption  specifies  a left

handed coordinate system (this should never be needed).

If no origin is specified then printers  use  /O1  and  displays and

plotters  use /O2.  This  best matches  the  plotter's  width/height

ratio.

Here are pictures of PrintGL's view of  the output media compared to

an HP 7475's view, to help with choosing a /O option.

HP 7475                                PrintGL

display or

A size           B size                printer        plotter

y��������Ŀ    0,0��������Ŀy        ul������Ŀur   ul��������Ŀur

�        �       �        �          1�      �4     1�        �4

�        �       �        �           �      �       �        �

0,0����������x      �        �           �      �     ll����������lr

�        �           �      �      2          3

�        �         ll��������lr

�        �          2        3

x����������

/O1  puts the print window's origin in the upper left corner

/O2  puts the print window's origin in the lower left corner

p-16

/P - Plotfile Option -------------------------------------------------

/Pf - plotfile - no default

The plotfile may be specified as the first parameter with no prefix,

or  it may be specified anywhere in the option list with /P.  If you

use a  file name  mask with "*"  and/or  "?",  each matching file is

processed, but the first failure or user break stops all processing.

/R - Internal Resolution Option --------------------------------------

/Ri - internal resolution - default /R1016

/R sets the internal grid resolution  in dots/inch.  It is  normally

1016 and rarely needs  to be changed.  To fit very large plots  into

the   -32768..32767  coordinate  limit,  use   /R508.   Setting  the

resolution to a multiple  of the output device  resolution times the

magnification may yield a small improvement in print quality.

/S - Pen Shading Option ----------------------------------------------

/Sc.. - pen shading patterns - default /S0

/S  assigns a shading pattern to each pen.  Unspecified pens use the

last specified shade.  The shade values are 0..8.  0  is the default

solid line.  1..4 are 1 dot patterns (75%, 50%, 25%, 12.5%) and need

line widths of at least three to give good looking  lines.  5..8 are

2x2 dot patterns  (75%, 50%, 25%,  12.5%) and need line widths of at

least  five to give good looking  lines.  Some printers may wash out

shading because they have  a dot size  that is large relative to the

dot spacing.  This is often the  case  with  dot  matrix printers in

multipass modes.

1 ��������         2 ��������          3 � � � �          4 �   �

��������           ��������            � � � �              �   �

��������           ��������            � � � �            �   �

��������           ��������            � � � �              �   �

5 ��������         6 ��  ��            7 ��  ��           8 ��

��  ��               ��  ��

��������           ��  ��              ��  ��                 ��

��  ��               ��  ��

/S023  sets pen 1    to solid

pen 2    to 1 pel checkerboard

pen 3..8 to 1 pel dots

/T - Temporary File Option -------------------------------------------

/T[+]f - temporary file - default /TPRINTGL.TMP

PrintGL uses a temporary file when it does not have enough memory to

handle a plotfile.  You can use /T to put it on a RAM disk for extra

speed.  If  you  have 256K of free memory, it is likely that PrintGL

will  never  use  a  temporary file.  You  can  force  the use of  a

temporary file, if  the plot has  over 1024 data points, with +.  If

you use a RAM disk this may speed printing.

p-17

/W - Pen Width Option ------------------------------------------------

/Wc.. - pen widths - default /WA

The W option assigns a line width to each pen (1..8). Each width may

be 1..8 pels or A..H for the default width of the output format plus

0..7 (maximum 8). Unassigned pens use the last assigned width.

The default line width is 1 for all output formats except non-impact

printers of a least than 240 dpi, which default to 2.

/WA       sets pen 1..8 to the default width for the chosen printer

/WABC123  sets pen 1    to the default width for the chosen printer

pen 2    to the default width + 1 pel

pen 3    to the default width + 2 pel

pen 4    to 1 pel

pen 5    to 2 pels

pen 6..8 to 3 pels

/X - Suppress Messages Option ----------------------------------------

/X[S[F]][U] - suppress messages - default /X

The  X option  lets  you  suppress  some  PrintGL  messages.  The  S

suboption suppresses location and size messages. With S you can also

specify   F,   which  suppresses  file  names   and  page   numbers,

substituting "Plot  1", "Plot  2", etc.  The  U suboption suppresses

unsupported command messages.

/X   gives all the normal messages

/XS  gives only the most important messages

/Y - Y/D Switch Setting Option ---------------------------------------

/YD|Y - Y/D switch setting - default /YD

The Y option sets the plotter's rear panel Y/D switch to D  or Y.  Y

starts with the plotter off and responds to esc.(, esc.Y, esc.), and

esc.Z  commands.  D  starts with  the plotter  on and  ignores these

commands.

/YD  puts the YD switch in the usual D position

/Z - Chord Angle Option ----------------------------------------------

/Zi,j - default and minimum chord angle - default /Z10,5

The HP 7475 defaults  to a  chord  angle of  5  and has  no minimum.

Because  PrintGL stores each  chord  in a  circle  in  its  internal

database, this can be  very  time consuming.  And because of the low

resolution   of  printers  it  does   not  produce  a  corresponding

improvement in print quality. The Z option specifies the default and

minimum  chord angles.  The default is /Z10,5.  For better emulation

use /Z5,2.

/Z5,2  sets the default chord angle to 5 degrees, and the minimum to

2 degrees

p-18

PrintGL Menu Interface (PMI) -----------------------------------------

PrintGL Menu Interface  (PMI) is a menu driven program that lets you

select PrintGL options, choose a list of plotfiles, and run PrintGL.

To  run  PrintGL Menu Interface,  enter PMI  at  the DOS prompt, and

press Enter. There are no command line options. The PMI main menu is

displayed at the top of the screen, and the bottom of the  screen is

used differently for each main menu option.  The  PMI  main  menu is

divided into six groups - the top row  and five columns.  Each group

is described below.

When  the  cursor is on  a main menu item,  the current value of the

corresponding  option is displayed below the main  menu.  To  modify

that value, press Enter (or the left mouse button).  The options are

modified with cursor movement and sometimes entering numeric values.

The Enter key  returns you to  the  main  menu  with the  new option

values. Esc (or the right mouse button) returns you to the main menu

with the option values unchanged.

Whenever a cursor bar has a notch, the value within the notch may be

edited.

PMI Specific Functions -----------------------------------------------

The top row of  main  menu  options is  dedicated to PMI  functions.

These  include  running PrintGL,  switching  to  alternate settings,

exiting  PMI, saving  the  current  PMI  settings, and  setting  PMI

options.

R  runs  PrintGL from  any main  menu cursor  location, but  PrintGL

messages will not be saved unless the cursor is on "run PrintGL".

The "alt settings"  item rotates  between three  PMI configurations.

The current configuration number is on line 2 column 1.

The "save settings" item saves the current PMI  configurations.  All

of the current  PrintGL and PMI options  for  each configuration are

saved.  The plotfile list is not saved.  PMI saves its configuration

by writing to  PMI.EXE.  To  find PMI.EXE  it  looks in  the current

directory and  then  searches  the  DOS PATH, saving  to  the  first

PMI.EXE that it finds.

The  "menu options"  item lets you configure  PMI mouse sensitivity,

screen colors, and up to five initial plotfile masks.  Screen colors

are specified with the standard IBM BIOS attributes.

For color displays, the left hex  digit is the  background color and

the  right hex digit is the foreground color.  Bright colors are not

available in the background.

0 black       4 red           8 gray (bright)     C bright red

1 blue        5 magenta       9 bright blue       D bright magenta

2 green       6 brown         A bright green      E bright yellow

3 cyan        7 white         B bright cyan       F bright white

p-19

PMI Input and Output Choices -----------------------------------------

Column  one of the main menu lets you choose the  plotfiles  to plot

(the /P option  in PrintGL), the plot page  number (/N),  the output

destination (/D), and the output format (/F).

You may enter up to 99 plotfiles in the "plotfile" main menu option.

To  choose from a  list of files, enter a  mask name and press Enter

(the cursor must  be on the  mask).  PMI will then display a list of

corresponding  files.  Select  any number of the files by moving the

cursor  to the desired file and pressing Space  (or the middle mouse

button). If no files have been selected, then pressing Enter selects

the file under the cursor.  The selected files are added to the file

list at  or below  the  cursor.  Files that would overflow the  99th

position on the plotfile list are dropped.

When you run PrintGL, PMI runs PrintGL for each file on the plotfile

list.  Masks are ignored.  For any  plotfiles that PrintGL does  not

successfully process, PMI inserts an arrow in front of the  plotfile

name.  You  can view the results from the "plotfile" main menu item.

The arrow is not considered  part of the file name by PMI, so you do

not need to remove it to retry PrintGL.

A number of printers in the output format list are marked  with "!".

These  are compression  mode drivers  that may run faster  than  the

normal drivers (see the ! suboption of the /F option).

Printers that  support  color  have  a color option.  This  lets you

specify a black/white printer even though you may have set up colors

with the "pen color" menu.  Output formats that do not support color

do not have this option and  always convert the  selected  colors to

black and white.

PMI Plot Options -----------------------------------------------------

The second column  of  main menu options define  how  the  plot will

appear  on  the  page.   The  options  include  magnification  (/M),

orientation (/O), position (/A), and initial scaling points (/I).

Left handed orientations are not available from PMI.

PMI Pen Options ------------------------------------------------------

The   third   column  of   main  menu   options   covers   the   pen

characteristics:  color  (/C), shade (/S), and width (/W).  Each  of

these menus use the up and down cursor keys to select a pen, and the

left and right cursor keys to choose an attribute.

For  color  selection,  the  one  character  colors  (l,b,c,g,m,r,y)

represent  pure  colors,  and  the  two  character  colors are shade

pattern mixes.  The  shade  pattern  mixed  colors use  the  current

shading option for  that  pen,  unless  the  shade pattern  is  100%

(solid), in which case a one pel checkerboard is used.

p-20

PMI Page Layout Options ----------------------------------------------

The fourth column of main menu options covers the page layout.  Each

item is a suboption of the /L option - box, form feed, print window,

and left and top margins.

Reverse image displays and bit maps are not available from PMI.

PMI Other Options ----------------------------------------------------

Options that  are  generally  set once and then never touched are in

column 5.  These include the  temporary  file  name  (/T), and under

"other",  the  default  and  minimum chord  angle (/Z), plotter  D/Y

switch setting  (/Y),  internal resolution  (/R), and  printer  code

prefix (/J) and suffix (/K) options.

p-21

Display Output -------------------------------------------------------

/FC drives a  CGA, but does  not give  color.  The  CGA card  has no

graphics mode that supports the eight colors used by PrintGL.

The EGA drivers (/FE, /FM) require a 128K or 256K EGA. If you have a

64K EGA, the CGA (/FC) driver will work.

/FM is  for  an  EGA  card with  a digital (TTL) monochrome  monitor

attached.  If your display adaptor supports Hercules modes, /FH will

give better resolution.

/FH supports the  Hercules  monochrome graphics card.  There  is  no

additional support for the Hercules InColor card.

/FV works with any VGA card.  /FV+ requires a VGA card and multisync

monitor  that  support  800x600 mode.  PrintGL sets up the screen by

setting BIOS display mode $64  ($ means hex).  If your VGA card uses

another mode number  to  select 800x600 mode, you  can  specify this

number  in  the /F option.  Here is a list of /FV options for common

VGA cards.

ATI    /FV+$54      Paradise     /FV+$58      Trident      /FV+$5B

Genoa  /FV+$29      Renaissance  /FV+$64      Video Seven  /FV+$62

For MCGAs, use  the  VGA driver without color - /FV~.  The MCGA does

not support high resolution color.

Display  output cannot  be redirected to a file - the /D  option  is

ignored when a display output format is chosen.

Canon BJ Printers ----------------------------------------------------

/FJ gives 360 dpi graphics for any Canon BJ printer in BJ130 mode.

For the BJ130E, BJ10E, BJ300, and BJ330 in Proprinter mode, /F7 (IBM

ExecJet) gives 360 dpi graphics with better speed than /FJ. /F7 does

not work with the BJ130.

The BJ  printers have  an image density option  (switch  1-9 on  the

BJ130).  Low density eliminates  every second pel (to save ink).  To

get the full resolution of the printer, you must use high density.

Canon LBP Printers ---------------------------------------------------

/FD  works with any  Canon LBP printer  in ISO  mode.  At 300 dpi it

takes about 1 meg of memory to print a full page of graphics. At 150

dpi (/FD-) 512K handles a full page.

The compression mode driver (/FD!) will probably be preferred.

You may need  to set the printer memory mode with /J 27  59 27 91 50

38 122.

If the printer  is in Diablo mode, PrintGL switches to ISO mode.  To

switch back to Diablo mode on completion, use /K 27 58.

p-22

Epson 9 Pin Printers -------------------------------------------------

/FN covers a wide range  of  9  pin dot matrix printers.  You need a

cyan/magenta/yellow ribbon  to  get  color  prints.  /FN+ uses 1/216

indexing and /FN* uses 1/216 indexing and 1/240 graphics.

The  IBM  9 pin  Proprinters  do not  do  1/216 indexing and  do not

correctly handle 1/240 graphics. /F1 is a special Proprinter driver.

The /FN drivers use the following printer escape codes:

esc J - index

esc L - graphics command for /FN and /FN+

esc Z - graphics command for /FN*

esc r - set ribbon color (only if color is specified)

Epson, NEC 24 Pin Printers -------------------------------------------

/FT covers  all  Epson  LQ  printers  and many  compatible  printers

(Panasonic, etc.).  You  need  a  cyan/magenta/yellow ribbon  to get

color prints.  /FT+ and /FT* use 1/360 graphics  mode, and /FT* uses

1/360  indexing  and  resets  the line feed distance to 1/6  inch on

completion.  A clean  paper path with equal tension on each  side is

needed for good 360x360 graphics.

/F9 and  /F9+, for NEC  24 pin printers,  are  identical to /FT  and

/FT+. /F9* is identical to /FT* except for the esc + printer code.

These  drivers will not  work with IBM  Proprinter X24s in alternate

graphics mode because they use the absolute tab command.

The /FT drivers use the following printer escape codes:

esc $   - absolute tab

esc * ' - graphics command for /FT

esc * ( - graphics command for /FT+, /FT*

esc +   - set n/360 indexing for /FT*

esc 2   - start esc A indexing for /FT*

esc A   - set n/72 indexing for /FT*

esc J   - index

esc r   - set ribbon color (only if color is specified)

The compression mode drivers (!  suboption) may  or  may  not  be an

improvement with these printers.

HP-GL Output ---------------------------------------------------------

/F8 outputs low level HP-GL, converting all characters,  arcs, fill,

and clips to vectors.  The output is  HP 7470 compatible, containing

only IN, SP, IW, LT, PU, and PD commands.  PG is added  if form feed

is  specified (the  default).  If you decline  the form  feed,  then

multiple plots may be appended to the same page.

PrintGL uses its width parameter (/W) as a pen selector, so for each

pen in the original  HP-GL, you may specify  a new pen in the output

HP-GL.  The default /W  option is /W12345678 for this case.  Shading

and color parameters are ignored.

p-23

HP LaserJet, DeskJet -------------------------------------------------

/FL works  with any LaserJet  or DeskJet in portrait mode.  Portrait

mode may be set with /J 27 38 108 48 79 or you can reset the printer

with /J 27 69.

At 300 dpi (the  default), if you have  just 512K in a LaserJet then

your plot can  cover  about half  the  area  of  the  page.  1.5M is

sufficient for a full  page  of  graphics.  At  150  dpi (/FL-) 512K

handles a full page. All DeskJets can handle a full page of graphics

at 300 dpi.

The  compression  mode driver (/FL!)  will probably be  preferred on

LaserJets, but it does not work on DeskJets.

/FI is the  same as /FL except it uses transfer  mode 2 for improved

speed. This is supported on the LaserJet 2P and 3 and all DeskJets.

HP PaintJet ----------------------------------------------------------

/FP drives PaintJet printers. PaintJet printers are sold with either

a serial or parallel interface. The parallel interface is faster.

The  compression  mode  driver  (/FP!)  uses  data  transfer  mode 2

(instead of 1) and works only with a PaintJet XL.

HP QuietJet ----------------------------------------------------------

/FQ works with QuietJets in PCL mode.  /J 27  37 65 /K 27 37 64 sets

PCL mode and resets to the DIP switch selected mode on completion.

IBM ExecJet ----------------------------------------------------------

/F7 drives the ExecJet. The image density (switch 1-4) should be set

to high to get the full 360 dpi resolution.

IBM LaserPrinter -----------------------------------------------------

/F4 drives the LaserPrinter in PPDS (native) mode.  /FL will work if

the LaserPrinter is in HP LaserJet mode. At 300 dpi it takes about 1

meg of memory to print a full page of  graphics.  At 150 dpi  (/F4-)

512K handles a full page.

The compression mode driver (/F4!) will probably be preferred.

IBM Proprinter -------------------------------------------------------

The /F1 driver  is  for all 9 pin  Proprinters.  The high resolution

driver (/F1+) uses  1/144  inch indexing  and it does  not work with

Epson 9 pin compatible printers that have 1/216 inch indexing.

There  is no  /F1*  resolution,  which  would  be  240x144,  because

Proprinters do not handle 1/240 graphics correctly.

Use /FN for the IBM Graphics Printer.

p-24

IBM Proprinter X24 and Compatible Printers ---------------------------

PrintGL  has  three  drivers  that  cover  the  Proprinter  X24  and

compatible printers. The first, /F6, is for all IBM Proprinter X24s.

These printers do 1/144 indexing, and using them at 180  dpi results

in  horizontal  white  streaks  every  2/3  inch.  The  /F6  drivers

compensate for this  indexing by defining the vertical resolution as

182 dots/inch and indexing 19/144 inch per line.  To  work properly,

the  printer  must  be  on  a  1/72 boundary  when the graphics  are

printed. If you keep the line feed distance a multiple of 1/72 (1/6,

1/8, 1/9) then this will always be the case.

The second  X24 compatible driver  is /F5 and is referred to as  X24

alternate  1.  This  is for printers that are X24  compatible except

that they do 1/180 indexing. This is probably the case with most X24

compatible printers, including the IBM Quickwriter.

The  third X24  compatible driver is  /FX and is referred to  as X24

alternate 2.  This is the same as /F5 except that the vertical units

command uses a different format that is not strictly IBM compatible.

This driver works with Panasonic 24 pin printers in X24 mode.

Proprinter X24 and XL24 printers are very slow with graphics, making

many passes  per  line.  This  limitation cannot  be  overcome  with

software. The X24E and XL24E are significantly faster.

IBM Quietwriter 2 and 3 ----------------------------------------------

/F2 and /F3 drive the Quietwriter 2 and 3.  Line widths should be at

least 2 as Quietwriters do  not print consistent 1 pel lines.  These

drivers reset the line feed distance to 1/6 inch upon completion.

Kodak Diconix Color 4 ------------------------------------------------

/FK drives the Color 4.

PostScript Printers --------------------------------------------------

/FS  outputs  Encapsulated  PostScript,   useful   for  printing  or

importing into publishing programs.  All characters, arcs, fill, and

clips are converted to  simple vectors,  so PrintGL is not a general

purpose HP-GL to PostScript converter.

PrintGL's pen width parameters are assumed to be in  units of 4/1016

inch.  Drawing  is  ordered so that each shade  covers  all  lighter

shades and the /S shading parameters are mapped as follows:

shade   gray level        shade   gray level

1,5     0.2               3,7     0.6

2,6     0.4               4,8     0.8

If the form feed suboption  of  the /L option is declined then there

is no  showpage command  in  the  file, and more  PostScript may  be

appended to the same page.

p-25

GEM .IMG Bit Map Output ----------------------------------------------

Here is the .IMG format created with /FG.  The header is composed of

8 word values, high byte first:

1

8

number of color planes - 1 or 3

1

pel width in microns - round(25400 / horizontal dots/inch)

pel height in microns - round(25400 / vertical dots/inch)

scan line width in pels

number of scan lines

The scan lines follow, in top to bottom order. For color plots, each

each complete scan line consists of a red, green, and then blue scan

line. Data for the scan line uses these commands:

$01..$7F          - 1 to 127 * 8 pels 0 (only used for background)

$81..$8F          - 1 to 127 * 8 pels 1 (only used for background)

$80 n b1 b2 .. bn - n bytes of bit mapped data

1 is the background and 0 the foreground for b/w files. 1,1,1 is the

background for color files, with 0 representing the absence of  red,

green, or blue. The bit usage may be reversed with /LR.

The  bit map is the size specified by the /L option plus  eight pels

in each direction. The width is an even number of bytes.

ZSoft .PCX Bit Map Output --------------------------------------------

/FZ gives .PCX output  with this format.  The  file  starts  with  a

header composed of 128 byte values:

10,2,1,1,0,0,0,0,

max_x,max_y,                            (2 bytes each, in pels   )

dpi_x,dpi_y,                            (2 bytes each, dots/inch )

0,0,0,   0,0,127,   0,127,0,   0,127,127,    (color palette      )

127,0,0, 127,0,127, 127,127,0, 127,127,127,

0,0,0,   0,0,255,   0,255,0,   0,255,255,

255,0,0, 255,0,255, 255,255,0, 255,255,255,

0,

color_planes,                           (1 or 3                  )

width                                   (2 bytes, in bytes       )

1,0,..,0

The  scan  lines,  top  to bottom, follow.  For  color  plots,  each

complete  scan line  consists of  a red,  green, and blue scan line.

Each scan line is composed of these data commands:

$00..$BF    - 1 byte of bit mapped data

$C1 b       - 1 byte of bit mapped data

$C2..$FF b  - 2..63 repeated bytes (only used for background)

1 or 1,1,1 is used for the background (0 or 0,0,0 with /LR). The bit

map is the size specified by the /L option  plus eight pels in  each

direction, and the width is an even number of bytes.

p-26

HP-GL Commands -------------------------------------------------------

PrintGL's HP-GL specification is taken  from the programming manuals

for  the  HP 7470A,  7475A,  ColorPro (7440/17440), and  7550A.  The

closest match to  the  PrintGL command set  is  the  7440/17440,  HP

manual 07440-90001.  PrintGL supports the 7470, 7475, and 7440/17440

command  sets  except  for  character  set  8  (Katakana),  digitize

commands, output commands, the error  mask command, and some  device

control commands. Supported commands are listed below. Commands that

are not applicable to printing, such as pen velocity, are considered

supported  but  are  completely  ignored.   Commands  that  are  not

supported are flagged and then ignored.

In addition, form feed  commands (PG, AF)  and  proportional  spaced

fonts, from  the 7550, are  supported.  And fifteen pens are allowed

instead of eight.

In reading a plotfile, PrintGL ignores carriage returns, line feeds,

and nulls  except where  they may be valid characters (LB,  SM,  DT,

esc.), and ASCII EOF (026) is interpreted as an end of file mark.

AA x_center , y_center , arc_angle [, chord_angle]

arc absolute

AA draws an arc starting at the current pen position  with the pen

in the current up/down state.

AF

advance full page (7550 extension)

AF starts a new plot. PrintGL starts every page with the pen up at

0,0.

AP

automatic pen pickup - IGNORED

AR x_center_delta , y_center_delta , arc_angle [, chord_angle]

arc relative

AR draws an arc starting at the current pen position  with the pen

in the current up/down state.

CA character_set

designate alternate character set

See HP-GL CHARACTER SETS for a list of character sets.

p-27

CI radius [, chord_angle]

circle

CI draws a  circle centered at the current position.  The  current

up/down state and position are unchanged by a CI command.

CP [spaces , lines]

character plot

CP  moves  the  pen by character spaces (+ is  along the text) and

lines  (+  is  up),  using  the current up/down  state.  Using  no

parameters is the same as a carriage return/line feed.

CS character_set

designate standard character set

See HP-GL CHARACTER SETS for a list of character sets.

DF

default values

This sets default values for the following commands:  CA,  CS, DR,

DT, FT, IW, LT, PA, PT, SC, SM, SL, SR, SS, TL.

DI [run , rise]

absolute character direction

DI sets the baseline direction for labels.  Using no parameters is

the same as DI1,0.

DR [run , rise]

relative character direction

DI  sets the baseline  direction  for labels.  Run  and rise are a

percentage of P2X-P1X and P2Y-P1Y. Using no parameters is the same

as DR1,0.

DTt

define label terminator

The label terminator marks  the end of an LB command.  The default

is ASCII 003.

EA x_opposite_corner , y_opposite_corner

edge rectangle absolute

EA draws a rectangle with corners defined  by the current position

and the command  parameters.  The current pen position and up/down

state are unchanged.

p-28

EC

enable cutter - IGNORED

EP

edge polygon

EP draws  the  current  polygon  edge.  Any  pen  up/down used  in

creating the polygon is respected in drawing the edge.

ER x_opposite_corner_delta , y_opposite_corner_delta

edge rectangle relative

ER draws a rectangle with corners defined  by the current position

and the command  parameters.  The current pen position and up/down

state are unchanged.

EW radius , start_angle , sweep_angle [, chord_angle]

edge wedge

EW draws a wedge centered at the current position. The current pen

position and up/down state are unchanged.

FP

fill polygon

FP  fills  the  current  polygon.  Pen up/down  commands  used  in

creating the polygon are ignored.

There is a limit of 2000 fill line segments per polygon.

FT [type [, spacing [, angle]]]

fill type

These are  the  fill types.  Type  2  is identical to  type 1  for

PrintGL.

1  solid lines spaced at the pen thickness (see PT command)

2  unidirectional solid lines spaced at the pen thickness

3  lines at the specified spacing

4  cross hatched lines at the specified spacing

5  type ignored

A spacing parameter of 0 sets  the spacing  to  1%  of  the  P1-P2

diagonal. Spacing is in x axis units (this is only relevant if the

axes have different  scaling).  PrintGL supports any integer angle

for  fill  (the HP 7475 requires multiples of 45  degrees).  Cross

hatched lines are at angle + 90.

IN

initialize

p-29

IP [p1x , p1y [, p2x , p2y]]

sets scaling points P1 and P2

IP sets the scaling points (P1 and P2) in plotter units. P1 and P2

are used  by a number  of  commands  to help with size independent

plots.

IW [x_lower_left , y_lower_left , x_upper_right , y_upper_right]

input window

IW sets a  clipping window in plotter units.  With  no parameters,

the clipping window is turned off.

LB..text..terminator

label

Labels use  the current character set, based on CA, CS, SA, and SS

commands. See HP-GL CHARACTER SETS for font specifics.

LT [pattern_number [, pattern_length]]

line type

PrintGL's line type  patterns are close to, but not  identical  to

HP's.  Line  type patterns  are dynamically  limited to a  minimum

length of 16 pels.

no parameter  ������������������������������������������������

0             dots at endpoints only

1             �               �               �

2             ��������        ��������        ��������

3             ������������    ������������    ������������

4             ������������  � ������������  � ������������  �

5             ����������  ��  ����������  ��  ����������  ��

6             ������  ��  ��  ������  ��  ��  ������  ��  ��

Pattern  length  is a  percentage  of the  P1-P2  diagonal with  a

default of 4%.

Line  type is  a property  of the line that is being drawn - it is

not a property of the pen that is being used to draw the line. You

cannot specify that a certain pen should use a specific line type.

(Color, shading, and width are pen  properties and can be assigned

to pens.)

PA [x , y] [, x , y] ...

plot absolute

PA moves the pen in current up/down state.

p-30

PD [x , y] [, x , y] ...

pen down

PD sets  the pen  down and optionally  moves  it with  the current

absolute/relative mode.

PG [i]

new page (7550 extension)

PG starts  a new  plot.  The parameter is ignored.  PrintGL starts

every page with the pen up at 0,0.

PM i

polygon mode

PM 0 starts polygon mode.  The current position is the first point

in the polygon.

PM 1 starts  a new  subpolygon.  The  position after the next  pen

movement is the first point in the subpolygon.  The first pen move

of a subpolygon is done with the pen up, regardless of the current

up/down status.

PM 2 ends polygon mode.

PM 1 and PM  2 close the current  subpolygon with the pen up if it

is  open.  If  used without  first starting  polygon mode  (PM 0),

PrintGL adds an implied PM 0 before processing the PM 1 or PM 2.

The following  commands are accepted in polygon  mode: PM, PA, PR,

PU, PD,  AA,  AR,  CI  (circles  are  automatically  prefixed  and

suffixed by PM 1), and IN (ends polygon mode).

The polygon buffer is limited to 500 data points.

PR [x_delta , y_delta] [, x_delta , y_delta] ...

plot relative

PR moves the pen with the current up/down state.

PS

paper size - IGNORED

PrintGL ignores this command. You must set the paper size with the

/L option.

PT [thickness]

pen thickness

The pen  thickness is the line spacing, from .1 to 5.0 mm, used in

solid fills. (PT does not cause a pen to be a certain width.)

p-31

PU [x,y] [,x,y] ...

pen up

PU  raises  the  pen  and  optionally  moves it with  the  current

absolute/relative mode.

RA x_opposite_corner , y_opposite_corner

shade rectangle absolute

RA  fills a rectangle with corners defined by the current position

and the command parameters.  The current pen  position and up/down

state are unchanged. The rectangle edge is not drawn.

RO [angle]

rotated coordinate system

RO or RO 0 sets  up a normal  coordinate system.  RO  90, the only

other  allowed value, sets up a system that is rotated 90 degrees.

The method of rotation depends on the paper  size.  The RO command

does not change the scaling  points,  so an IP  command  is useful

after an RO.

The HP 7475 specifies two different meanings for RO90 depending on

the  paper  size (A/A4 or B/A3).  PrintGL  decides  which  type of

rotation to do based on the sum of  the  print  window  width  and

height.  If this sum  is  greater than 24 inches  then large paper

rotation is used. Otherwise, small paper rotation is used.

Small paper rotation:              Large paper rotation:

RO 0             RO 90             RO 0             RO 90

y��������Ŀ   0,0��������Ŀy     0,0��������Ŀy    y��������Ŀ

�        �      �        �         �        �      �        �

�        �      �        �         �        �      �        �

0,0����������x    x����������         �        �      �        �

�        �      �        �

�        �      �        �

�        �      �        �

x����������   0,0����������x

RR x_opposite_corner_delta , y_opposite_corner_delta

shade rectangle relative

RR  fills a rectangle with corners defined by the current position

and the command parameters.  The current pen  position and up/down

state are unchanged. The rectangle edge is not drawn.

p-32

SA

select alternate character set

SA specifies the alternate character set as current. The alternate

character set is chosen with CA.

SC [xmin , xmax , ymin , ymax]

scaling

SC  sets user units for subsequent drawing.  The units are defined

by  user values for P1 (xmin,ymin)  and  P2 (xmax,ymax).  Using no

parameters  turns  scaling off, and  subsequent drawing is done in

plotter units (1/1016 inch).

SI [width , height]

character size absolute

SI sets the capital letter  box  width and height  in centimeters.

Letter  spacing  is  1.5  *  width and line spacing is 2 * height.

Using no parameters with B or A3 paper is the  same as SI.285,.375

and any other paper is the same as SI.187,.269.

SL [tan_angle]

character slant

SL sets the character slant.  The parameter is  interpreted as the

tangent  of  the angle from vertical.  Using no parameter sets the

slant to 0 degrees.

SM[c]

symbol mode

SM  sets symbol  mode, which centers  the symbol mode character at

any plotted point.  Any character from ASCII 033 to 126 except ";"

sets symbol mode. Any other character turns symbol mode off.

SP pen_number

select pen

SP selects a pen.  Using 0 or no parameter stores the pen  without

selecting a new pen.  This is usually done at the  end of a  plot.

PrintGL ignores any drawing with pen 0.

SR [width , height]

character size relative

SR sets the capital letter box width and height as a percentage of

P2X-P1X  and P2Y-P1Y.  Letter  spacing is 1.5  *  width  and  line

spacing  is  2  *  height.  Using no  parameters  is the  same  as

SR.75,1.5.

p-33

SS

select standard character set

SS specifies the standard character  set as current.  The standard

character set is chosen with CS.

TL [positive_length [, negative length]]

tick length

TL sets the tick lengths  used by XT and YT.  The  parameters  are

lengths above  and  below the current position as a  percentage of

P2Y-P1Y for x-axis ticks and  a percentage of  P2X-P1X  for y-axis

ticks.

UC [[pen_control ,] x_delta , y_delta] ...

user character

UC specifies  a user defined character.  The pen control number is

99 for pen down  or -99  for pen  up.  Pen movements are relative,

with x_delta in  units  of 1/4 the character  width and y_delta in

units of 1/8 the character height.

VA

adaptive velocity - IGNORED

VN

normal velocity - IGNORED

VS

pen velocity - IGNORED

WG radius , start_angle , sweep_angle [, chord_angle]

shade wedge

WG fills a wedge centered at the current position. The current pen

position and up/down  state are unchanged.  The wedge  edge is not

drawn.

XT

x-axis tick

XT draws a vertical line, with length specified by the TL command,

at  the current position.  The pen position  and up/down state are

unchanged.

p-34

YT

y-axis tick

YT draws  a  horizontal  line,  with length  specified  by  the TL

command, at the  current position.  The  pen position  and up/down

state are unchanged.

esc.(

esc.Y

plotter on

Esc.(  and esc.Y turn the plotter on.  This is only relevant  with

the /YY option. It is not flagged as unsupported if /YD is set.

esc.)

esc.Z

plotter off

Esc.) and esc.Z turn the plotter off. They are only effective with

the /YY option.  The plotter on  and off commands let you  include

comments  or  data  to  pass through  to  another  device  in  the

plotfile. These are flagged as unsupported commands if /YD is set,

as an indication that you should set /YY.

esc.@, esc.H, esc.I, esc.M, esc.N, esc.R

RS-232 device control commands - IGNORED with /YY

p-35

HP-GL Character Sets -------------------------------------------------

The  HP  7475  and 7440/17440 include  the following character sets.

PrintGL supports all of these except Katakana.  PrintGL's characters

are similar to but not identical to HP's.

0  ANSI ASCII

1  9825 character set

2  French/German

3  Scandinavian

4  Spanish/Latin American

6  JIS ASCII

7  Roman Extensions

8  Katakana - NOT SUPPORTED BY PRINTGL

9  ISO IRV (International Reference Version)

30  ISO Swedish

31  ISO Swedish For Names

32  ISO Norway Version 1

33  ISO German

34  ISO French

35  ISO United Kingdom

36  ISO Italian

37  ISO Spanish

38  ISO Portuguese

39  ISO Norway Version 2

The character sets specify a font for codes 033..127. Codes 000..032

and 128..255 are independent of the character set and are all no ops

except these.

003  default label terminator

008  backspace

009  half backspace

010  line feed

011  reverse line feed

013  carriage return

014  select alternate character set

015  select standard character set

032  space

Each of the  character  sets is available as  a proportional font by

adding  10  to  the  set  number.  Unlike  the  HP  7550,  PrintGL's

proportional fonts are identical to  their corresponding fixed pitch

fonts in shape, and differ only in positioning.  The character width

to user specified width ratio is 1 for all characters except these:

I l i j          width ratio .75

1 ( ) [ ] { } "  width ratio .5

. , : ; ` ' | !  width ratio .25

The space between  characters is always one half the  user specified

character width.

p-36

The  HP-GL characters cannot  be  printed  with  the  standard ASCII

character set.  The file CHARSET.PLT  is an HP-GL file  that can  be

used  to  print a  replacement for this page, showing  the character

sets.  Use the  command  PRINTGL CHARSET.PLT  /M1 /AO0,0 with  a  /F

option specific to your printer to do this.

p-37

Compatibility and Technical Information ------------------------------

PrintGL will run on any IBM compatible MS-DOS computer. It will work

in a region  as  small  as 96K  but  may  need  more memory for high

resolution or  wide  prints.  120K should suffice for the worst case

black/white prints -  13.5 inches  wide, 360x360  dpi.  Color output

requires  an  additional  32K or more  for high  resolution or  wide

prints.  200K is needed for the worst case.  A larger region usually

makes things go faster by  allowing larger  buffers and averting the

use of a spill file.

By default, PrintGL  uses the BIOS printer interrupt to send data to

a printer.  If a file or device is specified for output then DOS 2.0

file handling  is used  instead.  Messages are written to the screen

with  standard  DOS  CON  output.   Graphics  screen  modes  (except

Hercules) are set up with BIOS, and graphics screen updates are done

by writing directly to the display buffer.

PMI will run on any IBM compatible MS-DOS computer.  It will work in

a  region as small as 160K.  When running  PrintGL,  all but 40K  is

given to PrintGL. More memory may allow PrintGL to run more quickly.

PMI runs PrintGL via a DOS shell.  It searches the current directory

and then the DOS PATH, calling the first copy of PRINTGL.EXE that it

finds.  When saving the current settings, PMI makes the  same search

for PMI.EXE.

PMI  can use any text or graphics display  mode that allows at least

80 columns and 20 lines of text.  In text modes, PMI writes directly

to  the  display  buffer, and  in  graphics modes, BIOS  is  used to

display text.  When under DESQview, PMI should be started in a  text

mode, and it writes directly to the DV selected screen buffer.

The Microsoft mouse interface (interrupt  hex 33) is  used for mouse

handling.

PRINTGL.EXE  may be compressed with PKLITE, LZEXE, or other EXE file

compressors.  PMI should not be  compressed because it rewrites  the

EXE file when saving the user settings.

PrintGL and PMI  were  written  by  Cary Ravitz  and  compiled  with

Borland's Turbo Pascal 5.5 and Turbo Assembler 1.01.

p-38

User Support ---------------------------------------------------------

The  quickest  way to  resolve  problems with PrintGL  is to use the

support bulletin board.  The Ravitz Software support  bulletin board

is at:

606-268-0577  1200/2400,N,8,1  24 hours/day.

To ask a question (or make a comment or suggestion), first prepare a

file with the details of your question. Please be very specific, and

include  any related information  on  your  hardware,  software, and

operating environment.  Then call the bulletin board  and choose the

upload question  option.  XMODEM file  transfer protocol is required

for this.  Your question will be assigned  a number (such as 1015) -

remember this number. An answer file (with the assigned number) will

be posted, usually within 24 hours.

To  get an  answer,  call the  bulletin  board, choose the  download

option, then the answer option, and then enter  the  answer  number.

XMODEM is also required for downloading answers.

All question and answer files are public.

The latest versions of PrintGL and Ravitz Editor are available  from

the  bulletin board.  These  are  packed in  .ZIP format and  may be

downloaded with XMODEM or YMODEM (1K XMODEM).

Anyone  may use the bulletin board, however priority may be given to

registered users of PrintGL and RE.

You may send questions by mail to:

Ravitz Software Inc.

P.O. Box 25068

Lexington, KY  40524-5068

USA

It is  best to  send questions  on diskette  (360K, 1.2M,  720K,  or

1.44M, 720K  is  best) in a standard ASCII file.  The answer will be

put on your diskette and  it will be mailed back  to you.  This will

make  it  easy  to include  example files  or a  new version  of the

software with the answer.

Return postage is appreciated.