Microcontroller Simulator
Web Site  http://www.samphire.demon.co.uk/

General	Tutorials	Reference

The Simulator	Using the Simulator	ASCII Codes

Architecture	0) All The Tasks	Glossary

To Install	1) First Program	Hexadecimal and Binary

To Un-install	    99nasty.asm	Instructions Detailed

To Register	2) Traffic Lights	Instructions Summarised

Bugs	3) Data Moves	The List File

PC Support Handbook	4) Inc and Jmp	Negative Numbers

5) Keys CMP and JZ	Pop Up Help

Use Alt+Tab	6) Delay Procedure	Truth Tables and Logic

to switch back	7) Text IO	Using the Editor

and forth between	8) Data Table

the help window	9) Parameters

and your program.	10) SW Interrupts

11) HW Interrupts	(c) C Neil Bauers 1997

Who Should Use the Simulator

The simulator is intended for any student studying low level programming, control or machine architecture for the first time.

The simulator can be used by students aged 14 to 16 to solve less complex problems such as controlling the traffic lights and snake.

More advanced students typically 16 or older can solve quite complex low level programming problems involving conditional jumps, procedures, software and hardware interrupts and Boolean logic. Although programs will be small, there is good scope for modular design and separation of code and data tables.

The simulator is suitable for courses such as

An Introduction to Microprocessor Based Systems.

Computer Systems.

Electronics Courses.

EdExcel GNVQ Low Level Programming Additional Unit 21.

Courses involving microcontrollers.

Courses involving control systems.

Advanced Level Computing.

Description of the Simulator

In the shareware version the following instructions are not included. CALL, RET, INT and IRET. The hardware timer interrupt does not function because IRET can not be used either. The registered version includes these features. To Register click on this link.

This simulator emulates an eight bit CPU that is similar to the low eight bits of the 80x86 family of chips. 256 bytes of RAM are simulated. It is surprising how much can be done with only 256 bytes or RAM.

Features

8 bit CPU

16 Input Output ports. Not all are used.

Simulated peripherals on ports 0 to 5.

An assembler.

On-line help.

Single step through programs.

Continuously run programs.

Interrupt 02 triggered by a hardware timer (simulated).

CPU Clock Speed can be altered.

Peripherals	Example Programs

Keyboard Input	99keyb.asm

Traffic Lights	99tlight.asm

Seven Segment Display	99sevseg.asm

Heater and Thermostat	99hon.asm   99hoff.asm

Snake and Maze	99snake.asm

Stepper Motor	99step.asm

Memory Mapped VDU	99keyb.asm

Documentation

On-line hypertext help is stored in a Windows Help file.  It is possible to copy from windows help and paste into word processor or text editor programs.  The Rich Text Format source for the help file is also available. Many word processors including MS Word can load RTF files. Registered users have permission to modify help files for use by students and to print and or make multiple photocopies.

Disclaimer

This simulation software is not guaranteed in any way.  It may differ from reality.  It might not even work at all.  Try it out and if you like it, please register.

System Architecture

This simulator enables you to put into practice most of the topics discussed below.

The simulator consists of a central processing unit (CPU), 256 bytes of random access memory (RAM) and 16 input output (IO) ports.  Only five are used.  There is a hardware timer that triggers interrupt 02 at regular time intervals that you can pre-set using the configuration tab.

The simulator is programmable in that you can run many different programs.  In real life, the RAM would be replaced by read only memory (ROM) and the system would only ever run one program hard wired into the ROM.  There are hundreds of examples of systems like this controlling traffic lights, CD players, simple games consoles, many children's games, TV remote controls, microwave oven timers, clock radios, car engine management systems, central heating controllers, environmental control systems and the list goes on.

The Central Processing Unit

The central processing unit is the "brain" of the computer.  All calculations, decisions and data moves are made here.  The CPU has storage locations called registers.  It has an arithmetic and logic unit (ALU) where the processing is done.  Data is taken from the registers, processed and results go back into the registers.  Move (MOV) commands are used to transfer data between RAM locations and the registers.  There are many instructions, each with a specific purpose.  This collection is called the instruction set.

General Purpose Registers

The CPU has four general-purpose registers called AL, BL, CL and DL.  These are eight bits or one byte wide.  Registers can hold unsigned numbers in the range 0 to +255 and signed numbers in the range –128 to +127.  These are used as temporary storage locations.  Registers are used in preference to RAM locations because it takes a relatively long time to transfer data between RAM and the CPU.  Faster computers generally have more CPU registers or memory on the CPU chip.

The registers are named AL, BL, CL and DL because the 16-bit version of this CPU has more registers called AH, BH, CH and DH.  The 'L' means Low and the 'H' means High.  These are the low and high ends of the 16-bit register.

Special Purpose Registers

The special purpose registers in the CPU are called IP, SR and SP.

IP is the Instruction pointer.

This register contains the address of the instruction being executed.  When execution is complete, IP is increased to point to the next instruction.  Jump instructions alter the value of IP so the program flow jumps to a new position.  CALL and INT also change the value stored in IP.  In the RAM displays, the instruction pointer is highlighted red with yellow text.

SR is the Status Register.

This register contains flags that report the CPU status.

The 'Z' zero flag is set to one if a calculation gave a zero result.

The 'S' sign flag is set to one if a calculation gave a negative result.

The 'O' overflow flag is set if a result was too big to fit in a register.

The 'I' interrupt is set if interrupts are enabled.  See CLI and STI.

SP is the Stack Pointer.

The stack is an area of memory organised using the LIFO last in first out rule.  The stack pointer points to the next free stack location.  The simulator stack starts at address BF just below the RAM used for the video display.  The stack grows towards address zero.  Data is pushed onto the stack to save it for later use.  Data is popped off the stack when needed.  The stack pointer SP keeps track of where to push or pop data items.  In the RAM displays, the stack pointer is highlighted blue with yellow text.

Random Access Memory

The simulator has 256 bytes of ram.  The addresses are from 0 to 255 in decimal numbers or from [00] to [FF] in hexadecimal.  RAM addresses are usually given in square brackets such as [7C] where 7C is a hexadecimal number.  Read [7C] as "the data stored at location 7C".

Busses

Busses are collections of wires used to carry signals around the computer.  They are commonly printed as parallel tracks on circuit boards.  Slots are sockets that enable cards to be connected to the system bus.  An 8-bit computer typically has registers 8 bits wide and 8 wires in a bus.  A 16-bit computer has 16 bit registers and 16 address and data wires and so on.  The original IBM PC had 8 data wires and 20 address wires enabling one megabyte of RAM to be accessed.  32 bit registers and busses are now usual (1997).

The Data Bus	is used to carry data between the CPU, RAM and IO ports.  The simulator has an 8-bit data bus.

The Address Bus	is used to specify what RAM address or IO port should be used. The simulator has an 8-bit address bus.

The Control Bus	This has a wire to determine whether to access RAM or IO ports.  It also has a wire to determine whether data is being read or written.  The CPU reads data when it flows into the CPU.  It writes data when it flows out of the CPU to RAM or the IO ports.

The System Clock	wire carries regular pulses so that all the electronic components can function at the correct times.  Clock speeds between 100 and 200 million cycles per second are typical (1997).  This is referred to as the clock speed in MHz or megahertz.  The simulator runs in slow motion at about one instruction per second.  This is adjustable over a small range.

Hardware Interrupts	require at least one wire.  These enable the CPU to respond to events triggered by hardware such as printers running out of paper.  The CPU processes some machine code in response to the interrupt.  When finished, it continues with its original task.  The IBM PC has 16 interrupts controlled by 4 wires.

Simulator Installation

Important

FIRST make a back-up copy of the distribution disk or downloaded file.

System Requirements

Sms16V23.Exe requires Windows 3.1. A mouse or other pointing device is highly recommended to access the hypertext help pages.

To Install

Make a folder for all the simulator files and copy them from the distribution disk to this folder. If you have downloaded a Zip file, unzip it into this folder.

Suggestion	In a School College or University setting, the exercise files should be made available to students. These files are numbered as in this example - 02tlight.asm  and  99snake.asm. The other example files demonstrate the capabilities of the simulator and are typical of the sorts of programs students might write for assignments. DEMO.ASM could be made available. The other example files should be kept by the teacher for reference.

Note	Userinfo.reg, if available, should be in the same folder as Sms16V23.Exe. Userinfo.reg contains the registration key that you need to run the simulator as a registered program. When Userinfo.reg is missing or faulty, the simulator runs in shareware mode. When you register, you will get instructions relating to Userinfo.reg.

Note	The on line help - Sms16V23.Hlp should be in the same folder as Sms16V23.Exe

Network Installation

Follow the directions above. Make all the program, help and example files read only to normal users. Users should run the simulator in a folder set up for the purpose. If the simulator starts in this folder, it will save an INI file in the user's folder that holds information about window positions and preferences. If the INI file is missing, the simulator will start with default settings and will attempt to create the INI file. If SMS starts in a read only folder, it will not be able to save its INI file. If possible, correct this problem.

To Run

If you have set up a short cut, double click the icon or select the icon and press enter. You can double click on Sms16V23.Exe within the Windows File Manager. You can drag and drop an assembly code file onto the program icon. Assembly code file names should end in .asm. You can drop any type of file into the simulator but the result is not likely to be useful.

Simulator Un-Install

Important

FIRST make a back-up copy of the distribution disk or downloaded file.

To Un-install

NOTE Make sure you have a back up copy before un-installing.

The simulator is completely self contained and can be deleted without upsetting any other applications. Using Windows File Manager delete all the files in the simulator folder. Remove any icons, shortcuts or other references to the simulator from your system.

Simulator Registration

Please fill in this form to register.

When You Register

You will be sent a software key that unlocks all the features in the simulator.  You should enter this key into the place provided.  The menu choice for entering your key is

Help - Register    Alt+H   R

Type or paste your key into the space provided and press the Register button.  If there are no spelling errors, the simulator will be registered.

Shareware

The Microcontroller Simulator is shareware. In the shareware version the following features are not included. CALL, RET, INT, IRET and the simulated hardware timer interrupt 02.

Distribution

The complete unaltered shareware package may be distributed freely without restriction. Re-packaging to suit different distribution media is permitted. Please give a copy to any interested friend, colleague or student.

Copyright

C Neil Bauers 1997 applies to all parts of this product.

EMail nbauers@samphire.demon.co.uk

Student Registration

Individual students may use this software free. There is no need to register. Students may also use registered copies free of charge. These copies should, whenever possible, be obtained from your school, college or university.

Educational Institutions

Schools, Colleges, Universities and other Educational Institutions are asked to register. When you register, you will receive a key that unlocks the full version of this software. The prompts to register will go away and CALL, RET, INT, IRET and simulated hardware timer interrupts will be made available.

Payment

You are asked to register this software by paying 15.00 Pounds Sterling (UK Currency). Please use a method of payment that will not incur a large bank fee. A good method is a cheque written in pounds sterling issued by a bank based in the UK. Most banks in most countries can do this via their London or UK office or by arrangement with a UK bank. Cash is OK as a last resort. I hope to cover my software and Internet access costs. Please make cheques payable to C NEIL BAUERS. I am not able to process credit card payments.

Registered Sites

Registered sites will be kept informed of any upgrades to this software. Registered sites may photocopy manuals and help pages as required. You may give registered copies of the software to your students. These copies should be free. You may run unlimited instances of the software on a single site. If you have multiple sites in different villages, towns or cities, you are asked to register and pay for each site separately.

Please fill in this form to register.

Either print and post it or EMail it to
nbauers@samphire.demon.co.uk

C Neil Bauers

33 Low Road

Grimston

KINGS LYNN

Norfolk

PE 32 1 AF

United Kingdom

SIMULATOR REGISTRATION

Please fill in this form and post or EMail it to the address below.  These details will not be passed to any junk mail organisation.

Educational institutions should include a cheque for £15.00 in UK currency.  Such cheques can be obtained from most banks in most countries.

Cheques should be made payable to C Neil Bauers.

nbauers@samphire.demon.co.uk

C Neil Bauers

33 Low Road

Grimston

KINGS LYNN

Norfolk

PE 32 1 AF

UK

Please provide your details below ...

Contact Person ___________________________________________

Institution Name _________________________________________

Institution Address ______________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

Your EMail address _______________________________________

Bugs

Here are some bugs and features that I know about.  If you discover other problems, please send me an E Mail.  I will cure the easy bugs and list the hard ones here.

nbauers@samphire.demon.co.uk

Windows Font Problems

The simulator was developped using the default windows fonts, colours  and borders.  Some combinations of colours, fonts and window borders cause problems.

The symptoms include invisible text, text that won't fit inside the window, labels that don't line up with the item they are supposed to label and stretched bitmap images that look untidy.

The cure is to use Windows default settings or compatible settings.

My windows have vanished!

Beginners and some experts might hit another problem. With the display in a high resolution mode, all the simulator windows are moved far towards the bottom right. The display is restored to a lower resolution and the simulator re-starts with all its windows off screen where thay can't be controlled. The cure is to delete Sms16V23.INI. The simulator will restart with its windows in default positions.

The PC Support Handbook

Details are at	http://www.dumbreck.demon.co.uk/

This book covers PC architecture in some detail and makes excellent further reading in conjunction with the use of this simulator.

The book’s ISBN reference is 09521484-4-7 and it is available in the following ways:

From all major book distributors.

From the Maplin chain of stores or Maplin mail order.

Directly from the publishers, Dumbreck Publishing. This is often the quickest method.

Dumbreck Publishing,

24 Brandon Gardens,

Prestwick,

KA9 1RY,

Scotland.

Phone/Fax : 01292-470310

E-Mail : sales@dumbreck.demon.co.uk

PC Support Handbook Contents

Computer Basics

Computer Software

Operating Systems & Enviroments

Computer Architecture

Computer Video

P.C. Configuration

Batch Files

Computer Memory

Windows Configuration

Discs & Drives

Computer Viruses

P.C. Support

Upgrading a Computer

System Selection

Data Communications

Local Area Networks

Multimedia

Using the Simulator
Getting Started

On Line Help

Press F1 to get on line help.

Switch back and forth between the help system and the simulator using Alt+Tab.

While typing in your program, if you highlight an assembly code command like MOV or INT and you press F1, a help page will pop up with descriptions of the machine command.  If you mis-spell a command and try to get help, you will see a list of all the available commands.

Use Alt+Tab to get back to the simulator.

Writing a Program

To write and run a program using the simulator, select the text editor by pressing

µ §

Alt+D

and type in the program. It is best to get small parts of the program working rather than typing it all in at once. To get started, look at the tutorial example programs. Base your first programs on these.  You can switch backwards and forwards between help and your program by pressing Alt+Tab

Running a Program

To run a program, you can step through it one command at a time by pressing

µ §

Alt+P

repeatedly. You can run a program without single stepping by pressing

µ §

Alt+R or F9

Assembly Code

The code you type is called assembly code. You can translate the assembly code into machine code understood by the CPU by pressing

µ §

Alt+A

If necessary, this is done automatically when you press Run or Step.

Viewing Machine Code

The machine code stored in RAM can be viewed in three modes by selecting the Tab of your choice.

µ §

The Hexadecimal display corresponds exactly to the binary executed by the CPU.  The ASCII display is convenient if your program is processing text.  The text is readable but the machine codes are not.  The source code display shows how the assembly code commands are placed in memory.  In normal use, these displays work quite well.  It is possible to deliberately confuse the display.

For example	MOV  AL,15	translates into	D0 00 15.

Instead of typing in the MOV command as above try this.

DB	D0

DB	00

DB	15

The same machine codes will be generated.  The program will work in the same way.  The displays will no longer recognise that it was a MOV command.

Tutorial Examples

The tutorial examples provide a step by step introduction to the commands and techniques of low level programming.  Each program has one or more learning tasks associated with it.  Some of the tasks are simple.  Some are real brain teasers.

All The Tasks

Here are all the learning tasks grouped together with pointers to the example programs and explanatory notes.

Simple Arithmetic

Notes	1) First Program

Program	01first.asm

1)	Write a program that subtracts using SUB

2)	Write a program that multiplies using MUL

3)	Write a program that divides using DIV

4)	Write a program that divides by zero.  Make a note to avoid doing this in real life.

Using Hexadecimal

Notes	2) Traffic Lights

Program	02tlight.asm

5)	Use the help page on Hexadecimal and Binary numbers.  Work out what hexadecimal numbers will activate the correct traffic lights.  Modify the program to step the lights through a realistic sequence.

Using MOV and ASCII

Notes	3) Data Moves

Program	03move.asm

6)	Look up the ASCII codes of H, E, L, L and O and copy these values to memory locations C0, C1, C2, C3 and C4.  This is a simple and somewhat crude way to display text on a memory mapped display.

INC and JMP - Counting

Notes	4) Inc and Jmp

Program	04incjmp.asm

7)	Rewrite the program to count backwards using DEC BL.

8)	Rewrite the program to count in threes using ADD BL,3.

9)	Rewrite the program to count 1 2 4 8 16 using MUL BL,2

10)	Here is a more difficult task.  Count 0 1 1 2 3 5 8 13 21 34 55 98 overflow.  Here each number is the sum of the previous two.  You will need to use two registers and two RAM locations for temporary storage of numbers.  If you have never programmed before, this is a real brain teaser. Remember that the result will overflow when it goes above 127.

This number sequence was first described by Leonardo Fibonacci of Pisa (1170_1230)

Character Input Output

Notes	5) Keys CMP and JZ

Program	05keybin.asm

11)	Easy!  Input characters and display each character at the top left position of the VDU by copying them all to address [C0].

12)	Harder  Use BL to point to address [C0] and increment BL after each key press in order to see the text as you type it.

13)	Harder!  Store all the text you type in RAM when you type it in.  When you press Enter, display the stored text on the VDU display.

14)	Difficult  Type in text and store it.  When Enter is pressed, display it on the VDU screen in reverse order.  Using the stack makes this task easier.

Procedures

Notes	6) Time Delay Procedure

Program	06proc.asm

15)	Re-do the traffic lights program and use this procedure to set up realistic time delays.  02tlight.asm

16)	Re-do the text input and display program with procedures.  Use one procedure to input the text and one to display it.

Text IO and Procedures

Notes	7) Text IO

Program	07textio.asm

17)	Write a program using three procedures.  The first should read text from the keyboard and store it in RAM.  The second should convert any upper case characters in the stored text to lower case.  The third should display the text on the VDU screen.

Data Tables

Notes	8) Data Table

Program	08table.asm

18)	Improve the traffic lights data table so there is an overlap with both sets of lights on red.

19)	Use a data table to navigate the snake through the maze.  This is on port 04.  Send FF to the snake to reset it.  Up, down left and right are controlled by the left four bits.  The right four bits control the distance moved.

20)	Write a program to spin the stepper motor. Activate bits 1, 2, 4 and 8 in sequence to energise the electromagnets in turn.  The motor can be half stepped by turning on pairs of magnets followed by a single magnet followed by a pair and so on.

21)	Use a data table to make the motor perform a complex sequence of forward and reverse moves.  This is the type of control needed in robotic systems, printers and plotters.  For this exercise, it does not matter exactly what the motor does.

Parameters

Notes	9) Parameters

Program	09param.asm

22)	Write a procedure that doubles a number.  Pass the single parameter into the procedure using a register.  Use the same register to return the result.

23)	Write a procedure to invert all the bits in a byte.  All the zeros should become ones.  All the ones should become zeros.  Pass the value to be processed into the procedure using a RAM location.  Return the result in the same RAM location.

24)	Write a procedure that works out Factorial N.  This example shows one method for working out factorial N.  Factorial 5 is 5 * 4 * 3 * 2 * 1 = 120.  Your procedure should work properly for factorial 1, 2, 3, 4 or 5.  Factorial 6 would cause an overflow.  Use the stack to pass parameters and return the result.  Calculate the result.  Using a look up table is cheating!

25)	Write a procedure that works out Factorial N.  Use the stack for parameter passing.  Write a recursive procedure.  Use this definition of Factorial.  

Factorial ( 0 )  is defined as 1.
Factorial ( N ) is defined as N * Factorial (N - 1).

To work out Factorial (N), the procedure first tests to see if N is zero and if not then re-uses itself to work out N * Factorial (N - 1).  This problem is hard to understand in any programming language.  In assembly code it is harder still.

Software Interrupts

Notes	10) SW Interrupts

Program	10swint.asm

26)	Write a new interrupt 02 that fetches a key press from the keyboard and stores it into RAM.  The IBM PC allocates 16 bytes for key press storage.  The 16 locations are used in a circular fashion.

27)	Create a new interrupt that puts characters onto the next free screen location.  See if you can get correct behaviour in response to the Enter key being pressed (fairly easy) and if the Back Space key is pressed (harder).

Hardware Interrupts

Notes	11) HW Interrupts

Program	11hwint.asm

28)	Write a program that controls the heater and thermostat whilst at the same time counting from 0 to 9 repeatedly, displaying the result on one of the seven segment displays.  If you want a harder challenge, count from 0 to 99 repeatedly using both displays.  Use the simulated hardware interrupt to control the heater and thermostat.

29)	A fiendish problem.  Solve the Tower of Hanoi problem whilst steering the snake through the maze.  Use the text characters A, B, C Etc. to represent the disks.  Use three of the four rows on the simulated screen to represent the pillars.

One - First Program.  2 + 2

Program uses

CLO, MOV, ADD, SUB, MUL, DIV and END

Click on the link below.  When doing the learning exercises, add to and modify your own copy of the program.  You can copy from the help page and paste into the program source code editor.

Here is the example program.  01first.asm

To run the program press the Step button repeatedly until the program ends. Watch the registers change as you step through the program. The altered registers at the top of the window are highlighted yellow.

What you need to know

Comments

Any text after a semicolon is not part of the program and is ignored by the simulator. These comments are used for explanations of what the program is doing. Good programmers make extensive use of comments. Good comments should not just repeat the code. Good comments should explain why things are begin done.

CLO

The CLO command is unique to this simulator. It closes any window that is not needed while a program is running. This command makes it easier to write nice demonstration programs. It also avoids having to close several windows manually.

MOV

The MOV command is short for Move. In this example numbers are being copied into registers where arithmetic can be done. Mov copies data from one location to another. The data in the original location is left intact by the mov command.

Registers

Registers are storage locations where 8 bit binary numbers are stored. The central processing unit in this simulator has four general purpose registers called AL, BL, CL and DL. These registers can, with a few exceptions, be used for any purpose.

Newer central processing unit (CPU) chips have 16, 32 or even 64 bit registers. These work in the same way but more data can be moved in one step so there is a speed advantage.

Wider registers can store larger integer (whole) numbers. This simplifies many programming tasks. The other three registers SP, IP and SR are described elsewhere.

Hexadecimal Numbers

In the command   MOV AL,2   the 2 is a hexadecimal number. The hexadecimal number system is used in low level programming because there is a very convenient conversion between binary and hex. Study the Hexadecimal and Binary number systems.

Arithmetic

ADD is used to add two registers together. Another version of add is used to add a number to a register. You can get help on ADD, SUB, MUL and DIV by

highlighting the command and pressing F1.  This works both here and within the program source code editor.

END

The last command in a program should be END. Any text after the END keyword is ignored.

Your Task

Most of the example programs include a learning task. You should study the example and if you can complete the task, it is likely that your understanding of the example is good.  In this case your task is to use all the registers AL, BL, CL and DL and to use ADD, SUB, MUL and DIV.  Find out what happens if you try to divide by zero.

01First.Asm

Copy this and paste it into the simulator source code editor.

; _____ WORK OUT 2 PLUS 2 ______________________________________

CLO		; Close unwanted windows.

MOV	AL,2	; Copy a 2 into the AL register.

MOV	BL,2	; Copy a 2 into the BL register.

ADD	AL,BL	; Add AL to BL.  Answer goes into AL.

END		; Program ends

; _____ Program Ends __________________________________________

YOUR TASKs

1)	Write a program that subtracts using SUB

2)	Write a program that multiplies using MUL

3)	Write a program that divides using DIV

4)	Write a program that divides by zero.  Make a note to avoid doing this in real life.

How NOT to write a program
99Nasty.Asm

Copy this and paste it into the simulator source code editor.

; ----- Here is how NOT to write a program -----

_: Mov BL,C0 Mov AL,3C Q: Mov [BL],AL CMP  AL,7B

JZ Z INC AL INC BL JMP Q Z: MOV CL,40  MOV AL,20

MOV BL,C0 Y: MOV [BL],AL INC BL DEC CL JNZ Y JMP

_ END ; Look at the list file.  It comes out OK!

; Press  Escape  to  stop  the program  running.

; ----------------------------------------------

Two - Traffic Lights

Program uses

CLO, MOV, OUT, JMP and END

The traffic lights are controlled by sending data to input output (IO) port One.

There are six lamps to control.  Red, Amber and Green for a pair of lights.  This can be achieved with a single byte of data where two bits are unused.

By setting the correct bits to One, the correct lamps come on.

Here is the example program. 02tlight.asm

Click on the link above.  Copy the program from the help page and paste it into the program source code editor.

To run the program press the Step button repeatedly or press the Run button.

To stop the program, press Stop.  When the program is running, click the RAM-Source or RAM-Hex or RAM-ASCII tabs. These give alternative views of the contents of random access memory (RAM).

Also click the List File tab to see the machine code generated by the simulator.

What you need to know

Labels and the JMP command

Labels mark positions that are used by Jump commands. All the commands in this program are repeated for ever or until Stop is pressed. Label names must start with a letter or _ character. Label names must not start with a digit. The line

JMP Start

causes the program to jump back and re-do the earlier commands.

Destination labels end in a colon. For example

Start:

Controlling the Lights

If you look carefully at the traffic lights display, you can see which bit controls each light bulb. Work out the pattern or noughts and ones needed to turn on a sensible set of bulbs. Use the Hexadecimal and Binary numbers table to work out the hexadecimal equivalent. Move this hexadecimal number into AL.

OUT 01

This command copies the contents of the AL register to Output Port One. The traffic lights are connected to port one. A binary one causes a bulb to switch on. A nought causes it to turn off.

Traffic Lights - 02Tlight.Asm

Copy this and paste it into the simulator source code editor.

; _____ CONTROL THE TRAFFIC LIGHTS _____________________________

CLO		; Close unwanted windows.

Start:

; Turn off all the traffic lights.

MOV	AL,0	; Copy 00000000 into the AL register.

OUT	01	; Send AL to Port One (The traffic lights).

; Turn on all the traffic lights.

MOV	AL,FC	; Copy 11111100 into the AL register.

OUT	01	; Send AL to Port One (The traffic lights).

JMP	Start	; Jump back to the start.

END		; Program ends.

; _____ Program Ends __________________________________________

YOUR TASK

5)	Use the help page on hexadecimal and Binary numbers.  Work out what hexadecimal numbers will activate the correct traffic lights.  Modify the program to step the lights through a realistic sequence.

Three - Data Moves

Program uses

Five variants of MOV, INC and JMP

Here is the example program. 03move.asm

Step through the program and watch the register values changing. In particular, look at the RAM-Hex display and note the way that values in RAM change. Addresses [50] and [A0] are altered.  You can copy the example program from the help page and paste it into the source code editor.

TASK.

Look up the ASCII codes of the letters in H,E,L,L,O and move these ASCII codes to RAM addresses [C0], [C1], [C2], [C3] and [C4]. Run the program and watch how the text appears on the simulated VDU display. This is very much the same as what happens in the IBM PC running MS DOS. The program you write should work but if you continue to study low level programming, you will find much more efficient and flexible ways of solving this problem.

There are several ADDRESSING MODES available with move commands.

Immediate Addressing.

A hexadecimal number is copied into a register. Examples

MOV	AL,15	; Copy 15 HEX into the AL register

MOV	BL,40	; Copy 40 HEX into the BL register

MOV	CL,50	; Copy 50 HEX into the CL register

MOV	DL,60	; Copy 60 HEX into the DL register

Indirect Addressing

A value is moved to or from RAM. The ram address is given as a number like [22] in square brackets. Examples

MOV	[A0],AL	; Copy value in AL to RAM location [40]

MOV	BL,[40]	; Copy value in RAM location [A0] into BL

Register Indirect Addressing

Copy a value from RAM to a register or copy a value from a register to RAM. The RAM address is contained in a second register enclosed in square brackets like this [CL].

MOV	[CL],AL	; Copy the value in AL to the RAM

; location that CL points to.

MOV	BL,[CL]	; Copy the RAM location that CL points

; to into the BL register.

Register Moves - Not available in this simulator.

A register move looks like this

MOV AL,BL

To do this using simulator commands, use

PUSH BL

POP AL

Push and Pop are explained later.

Calculated Addresses - Not available in this simulator.

Copy a value from RAM to a register or copy a value from a register to RAM. The RAM address is contained in square brackets and is calculated. This is done to simplify access to record structures. For example a surname might be stored 12 bytes from the start of the record. This technique is shown in the examples below.

MOV	[CL + 5],AL	; Copy the value in AL to the RAM

; location that CL + 5 points to.

MOV	BL,[CL + 12]	; Copy the RAM location that

; CL + 12 points

; to into the BL register.

Implied Addresses - Not available in this simulator.

In this case, memory locations are named. Address [50] might be called 'puppy'. This means that moves can be programmed like this.

MOV	AL,puppy	; Copy the value in RAM at position

; puppy into the AL register.

MOV	puppy,BL	; Copy BL into the RAM location

; that puppy refers to.

Using Mov - 03Move.Asm

Copy this and paste it into the simulator source code editor.

; ---------------------------------------------------------------

;   A program to demonstrate MOV commands. Mov is short for move.

; ---------------------------------------------------------------

CLO			; Close unwanted windows.

;	IMMEDIATE MOVES

MOV	AL,15		; Copy 15 HEX into the AL register

MOV	BL,40		; Copy 40 HEX into the BL register

MOV	CL,50		; Copy 50 HEX into the CL register

MOV	DL,60		; Copy 60 HEX into the DL register

Foo:

INC	AL		; Increment AL for no particular reason.

;	INDIRECT MOVES

MOV	[A0],AL	; Copy value in AL to RAM location [40]

MOV	BL,[40]	; Copy value in RAM location [A0] into BL

;	REGISTER INDIRECT MOVES

MOV	[CL],AL	; Copy the value in AL to the RAM

; location that CL points to.

MOV	BL,[CL]	; Copy the RAM location that CL points

; to into the BL register.

JMP	Foo		; PRESS ESCAPE TO STOP THE PROGRAM

END

; ---------------------------------------------------------------

TASK

6)	Look up the ASCII codes of H, E, L, L and O and copy these values to memory locations C0, C1, C2, C3 and C4.  This is a simple and somewhat crude way to display text on a memory mapped display.

Four - INC and JMP - Counting

Program Uses

MOV, INC, JMP and END

Here is the example program. 04incjmp.asm

The program counts up in steps of one until the total is too big to be stored in a single byte. At this point the calculation overflows. Watch the values in the registers. In particular, watch IP and SR. These are explained below.

Although this program is very simple, some new ideas are introduced.

MOV BL,40

This line initialises BL to 40.

Rep:

Rep: is a label. Labels are used with Jump commands. It is possible for programs to jump backwards or forwards. Because of the way numbers are stored, the largest jumps are -128 backwards and + 127 forwards. Labels must begin with a letter or the _ character. Labels may contain letters, digits and the _ character. Destination labels must end with a Colon:

INC BL

This command adds one to BL. Watch the BL register. It will count up from 40 in hexadecimal so after 49 comes 4A, 4B, 4C, 4D, 4E, 4F, 50, 51 and so on.

Overflow

When BL reaches 7F hex or 127 in decimal numbers the next number ought to be 128 but because of the way numbers are stored in binary, the next number is minus 128. This effect is called an OVERFLOW.

Status Register (SR)

The status register labelled SR contains four flag bits that give information about the state of the CPU. There are three flags that indicate whether a calculation overflowed, gave a negative result or gave a zero result. Calculations set these flags

S	The sign flag indicates a negative result.

O	The overflow flag indicates overflows.

Z	The zero flag indicates a zero result.

The fourth flag ...

I	Interrupts enabled.	STI turns this on.

CLI turns this off.

These flags are described in more detail later.

JMP Rep

This command causes the central processing unit (CPU) to jump back and repeat earlier commands or jump forward and skip some commands.

Instruction Pointer (IP)

The instruction pointer labelled IP contains the address of the instruction being executed. This is indicated by a highlighted position in the simulator. Each CPU command causes the IP to be increased by 1, 2 or 3 depending on the size of the command.  In the RAM displays, the instruction pointer is highlighted red with yellow text.

NOP		; Increase IP by 1

INC	BL	; Increase IP by 2

ADD	AL,BL	; Increase IP by 3

JMP	Rep	; Add or subtract a value from IP to

; jump to a new part of the program.

Fetch Execute Cycle

Fetch the instruction that IP points to.  This is called the operator.

If necessary, fetch data at IP + 1.  This is the first operand.

If necessary, fetch data at IP + 2.  This is the second operand.

Execute the command.  This may involve more fetching or putting of data.

Increase IP to point to the next command.

Repeat this cycle.

Every machine cycle has one operator or instruction.  There could be zero, one or two operands depending on the instruction.  OP Codes are the machine codes that correspond to the operators and operands.

04IncJmp.Asm

Copy this and paste it into the simulator source code editor.

; _____ Counting _______________________________________________

MOV	BL,40	; Initial value stored in BL

Rep:			; Jump back to this label

INC	BL	; Add ONE to BL

JMP	Rep	; Jump back to Rep

END		; Program Ends

; _____ Program Ends __________________________________________

TASK

7)	Rewrite the program to count backwards using DEC BL.

8)	Rewrite the program to count in threes using ADD BL,3.

9)	Rewrite the program to count 1 2 4 8 16 using MUL BL,2

10)	Here is a more difficult task.  Count 0 1 1 2 3 5 8 13 21 34 55 98 overflow.  Here each number is the sum of the previous two.  You will need to use two registers and two RAM locations for temporary storage of numbers.  If you have never programmed before, this is a real brain teaser. Remember that the result will overflow when it goes above 127.

This number sequence was first described by Leonardo Fibonacci of Pisa (1170_1230)

Five - Keyboard Input

Program Uses

CLO, IN, CMP, JNZ, END

Here is the example program. 05keybin.asm

You can copy the example program from the help page and paste it into the source code editor.

IN  00

Input from port zero. In this simulator, port zero is wired to the keyboard hardware. The simulator waits for a key press and copies the ASCII code of the key press into the AL register.

CMP  AL,0D

Compare the AL register with the ASCII code of the Enter key. The ASCII code of the Enter key is 0Dhex.

CMP  AL,BL   works as follows. The processor calculates AL - BL. If the result is zero, the 'Z' flag in the status register SR is set. If the result is negative, the 'S' flag is set. If the result is positive, no flags are set. The 'Z' flag is set if AL and BL are equal. The 'S' flag is set if BL is greater then AL. No flag is set if AL is greater than BL.

JNZ  Rep

JNZ stands for Jump Not Zero. Jump if the 'Z' flag is not set. The program will jump forwards or back to the address that Rep marks.

A related command is JZ. This stands for Jump Zero. Jump if the zero flag is set. In this program, the CMP command sets the flags. Arithmetic commands also set the status flags.

MOV  [C0],AL

This will copy AL to address [C0]. The visual display unit works with addresses [C0] to [FF]. This gives a display with 4 rows and 16 columns. Address [C0] is the top left corner of the screen. This command is needed for Task 1.

MOV  [BL],AL

This copies AL to the address that BL points to. BL can be made to point to the VDU screen at [C0] by using MOV BL,C0. BL can be made to point to each screen position in turn by using INC BL. This is needed for task 2.

05KeybIn.Asm

Copy this and paste it into the simulator source code editor.

; --------------------------------------------------------------

;  Input key presses from the keyboard until Enter is pressed.

; --------------------------------------------------------------

CLO		; Close unwanted windows.

Rep:

IN	00	; Wait for key press - Store it in AL.

CMP	AL,0D	; Was it the Enter key? (ASCII 0D)

JNZ	Rep	; No - jump back.  Yes - end.

END

; --------------------------------------------------------------

TASK

11)	Easy!  Display each character at the top left position of the VDU by copying them all to address [C0].

12)	Harder  Use BL to point to address [C0] and increment BL after each key press in order to see the text as you type it.

13)	Harder!  Store all the text you type in RAM when you type it in.  When you press Enter, display the stored text on the VDU display.

14)	Difficult  Type in text and store it.  When Enter is pressed, display it on the VDU screen in reverse order.  Using the stack makes this task easier.

Six - Time Delay Procedure

Program Uses

MOV, CALL, RET, DEC, JNZ, JMP, ORG, PUSH, POP, PUSHF, POPF, END

Here is the example program. 06proc.asm

You can copy the example program from the help page and paste it into the source code editor.

MOV  AL,8

A value is placed into the AL register before calling the time delay procedure. This value determines the length of the delay.

CALL  30

Call the procedure at address [30]. This alters the instruction pointer IP to [30] and the program continues to run from that address. When the CPU reaches the RET command it returns to the address that it came from. This return address is saved on the stack.

Stack

This is a region in memory where values are saved and restored. The stack uses the Last In First Out rule. LIFO. The CALL command saves the return address on the stack. The RET command gets the saved value from the stack and jumps to that address by setting IP.

ORG  30

Origin at address [30]. ORG specifies at what RAM address machine code should be generated. The time delay procedure is stored at address [30].

PUSH  AL

Save the value of AL onto the stack. The CPU stack pointer SP points to the next free stack location. The push command saves a value at this position. SP is then moved back one place to the next free position. In this simulator, the stack grows towards address Zero. A stack overflow occurs if the stack tries to fill more than the available memory.

PUSHF

Save the CPU flags in the status register SR onto the stack. This ensures that the flags can be put back as they were when the procedure completes. The stack pointer is moved back one place. See the Push command.

DEC  AL

Subtract one from AL. This command sets the Z flag if the answer was Zero or the S flag if the answer was negative.

JNZ  REP

Jump Not Zero to the address that Rep marks. Jump if the Z flag is not set.

POPF

Restore the CPU flags from the stack. Increase the stack pointer by one.

POP  AL

Restore the AL register from the stack. This is done by first moving the stack pointer SP forward one place and copying the value at that stack position into the AL register. A stack underflow occurs when an attempt is made to pop more items off the stack than were present.

RET

Return from the procedure to the address that was saved on the stack by the CALL command. Procedures can re-use themselves. This is called recursion. It is a powerful technique and dangerous if you don't understand what is happening! Accidental or uncontrolled recursion causes the stack to grow until it overwrites the program or overflows.

06Proc.Asm

Copy this and paste it into the simulator source code editor.

; ---------------------------------------------------------------

;  A general purpose time delay procedure.

;  The delay is controlled by the value in AL.

;  When the procedure terminates, the CPU registers are

;  restored to the same values that were present before

;  the procedure was called. Push, Pop, Pushf and Popf

;  are used to achieve this.  In this example one procedure

;  is re-used three times.  This re-use is one of the main

;  advantages of using procedures.

;------ The Main Program ----------------------------------------

Start:

MOV	AL,8	; A short delay.

CALL	30	; Call the procedure at address [30]

MOV	AL,10	; A middle sized delay.

CALL	30	; Call the procedure at address [30]

MOV	AL,20	; A Longer delay.

CALL	30	; Call the procedure at address [30]

JMP	Start	; Jump back to the start.

; ----- Time Delay Procedure Stored At Address [30] -------------

ORG	30	; Generate machine code from address [30]

PUSH	AL	; Save AL on the stack.

PUSHF		; Save the CPU flags on the stack.

Rep:

DEC	AL	; Subtract one from AL.

JNZ	REP	; Jump back to Rep if AL was not Zero.

POPF		; Restore the CPU flags from the stack.

POP	AL	; Restore AL from the stack.

RET		; Return from the procedure.

; ---------------------------------------------------------------

END

; ---------------------------------------------------------------

TASK

15)	Re-do the traffic lights program and use this procedure to set up realistic time delays.  02tlight.asm

16)	Re-do the text input and display program with procedures.  Use one procedure to input the text and one to display it.

; ---------------------------------------------------------------

Seven - Text Input and Output

Program Uses

MOV, CALL, RET, HALT, IN, INC, CMP, JZ, JMP, ORG, PUSH, POP, PUSHF, POPF, END

Here is the example program. 07textIO.asm

You can copy the example program from the help page and paste it into the source code editor.

Passing Parameters

MOV  BL,70

The BL register contains 70.  This value is needed by the text input procedure.  It is the address where the text will be stored in RAM.  This is an example of passing a parameter using a register.  All you are doing is getting a number from one part of a program to another.

INC  BL

This command adds one to BL.  The effect is to make BL point to the next memory location ready for the next text character to be stored.

CALL  10

Call the procedure at address [10].  This is achieved in practice by setting the CPU instruction pointer IP to [10].

RET

At the end of the procedure, the RET command resets the CPU instruction pointer IP back to the instruction after the CALL instruction to the procedure.  This address was stored on the stack by the CALL instruction.

HALT

Don't confuse HALT and END.  The END command causes the assembler to stop scanning for more instructions in the program.  The HALT command generates machine code 00 which causes the CPU to halt.  There can be several HALT commands in a program but only one END command.

ORG  10

Origin [10].  The assembler program starts generating machine code from address [10].

PUSH  AL   and   POP  AL

Save the value of AL onto the stack.  This is an area of RAM starting at address BF.  The stack grows towards zero.  The RAM displays show the stack pointer as a blue highlight with yellow text.  Push and Pop are used so that procedures and interrupts can tidy up after themselves.  The procedure or interrupt can alter CPU registers but it restores them to their old values before returning.

PUSHF and POPF

PUSHF saves the CPU flags onto the stack.  POPF restores the CPU flags to their original value.  This enables procedures and interrupts to do useful work without unexpected side affects on the rest of the program.

IN  00

Input from port zero.  This port is connected to the keyboard.  The key press is stored into the AL register.

CMP  AL,0D

Compare the AL register with the hexadecimal number 0D.  0D is the ASCII code of the Enter key.  This line is asking "Was the enter key pressed?"  CMP works by subtracting 0D from AL.  If they were equal then the subtraction gives an answer of zero.  This causes the CPU zero or 'Z' flag to be set.

JZ  Stop

Jump to the Stop label if the CPU 'Z' flag was set.  This is a conditional jump.

MOV  [BL],AL

Move the key press stored in AL into the RAM location that [BL] points to.  INC BL is then used to make BL point to the next RAM location.

JMP  Rep

Jump back to the Rep label.  This is an unconditional jump.  It always jumps and the CPU flags are ignored.

RET

Return from the procedure to the address stored on the stack.  This is done by setting the instruction pointer IP in the CPU.

07TextIo.Asm

Copy this and paste it into the simulator source code editor.

; --------------------------------------------------------------

; A program to read in a string of text and store it in RAM.

; The end of text will be labelled with ASCII code zero/null.

; --------------------------------------------------------------

; THE MAIN PROGRAM

MOV	BL,70	; [70] is the address where the text will

; be stored. The procedure uses this.

CALL	10	; The procedure at [10] reads in text and

; places it starting from the address

; in BL.

; BL should still contain [70] here.

CALL	40	; This procedure does nothing until you

; write it.  It should display the text.

HALT		; DON'T USE END HERE BY MISTAKE.

; --------------------------------------------------------------

; A PROCEDURE TO READ IN THE TEXT

ORG	10	; Code starts from address [10]

PUSH	AL	; Save AL onto the stack

PUSH	BL	; Save BL onto the stack

PUSHF		; Save the CPU flags onto the stack

Rep:

IN	00	; Input from port 00 (keyboard)

CMP	AL,0D	; Was key press the Enter key?

JZ	Stop	; If yes then jump to Stop

MOV	[BL],AL	; Copy keypress to RAM at position [BL]

INC	BL	; BL points to the next location.

JMP	Rep	; Jump back to get the next character

Stop:

MOV	AL,0	; This is the NULL end marker

MOV	[BL],AL	; Copy NULL character to this position.

POPF		; Restore flags from the stack

POP	BL	; Restore BL from the stack

POP	AL	; Restore AL from the stack

RET		; Return from the procedure.

; --------------------------------------------------------------

; A PROCEDURE TO DISPLAY TEXT ON THE SIMULATED SCREEN

ORG	40	; Code starts from address [10]

; **** YOU MUST FILL THIS GAP ****

RET		; At present this procedure does

; nothing other than return.

; --------------------------------------------------------------

END		; It is correct to use END at the end.

; --------------------------------------------------------------

TASK

17)	Write a program using three procedures.  The first should read text from the keyboard and store it in RAM.  The second should convert any upper case characters in the stored text to lower case.  The third should display the text on the VDU screen.

; --------------------------------------------------------------

Eight - Data Table

Program Uses

JMP, DB, MOV, OUT, INC, CMP, JZ, JMP, END

Here is the example program. 08table.asm

You can copy the example program from the help page and paste it into the source code editor.

DB  84

DB stands for Define Byte/s.  In this case 84hex is stored into RAM at address [02].  Addresses [00] and [01] are occupied by the  JMP  Start  machine codes.

84 hex is 1000 0100 in binary.  This is the pattern or noughts and ones needed to turn on the left red light and the right green light.

MOV  BL,02

Move 02 into the BL register.  [O2] is the RAM address of the start of the data table.  BL is used as a pointer to the data table.

MOV  AL,[BL]

[BL] points to the data table.  This line copies a value from the data table into the AL register.

OUT  01

Send the contents of the AL register to port 01.  Port 01 is connected to the traffic lights.

CMP  AL,58

58 is the last entry in the data table.  If AL contains 58, it is necessary to reset BL to point back to the start of the table ready to repeat the sequence.  If AL is equal to 58, the 'Z' flag in the CPU will be set.

JZ  Start

Jump back to start if the 'Z' flag in the CPU is set.

INC  BL

Add one to BL to make it point to the next entry in the data table.

08Table.Asm

Copy this and paste it into the simulator source code editor.

; ----- EXAMPLE 8 ------- DATA TABLES --------------------------

JMP	Start	; Skip past the data table.

DB	84	; Data table begins.

DB	C8	; These values control the traffic lights

DB	31	; This sequence is simplified.

DB	58	; Last entry is also used as end marker

Start:

MOV	BL,02	; 02 is start address of data table

Rep:

MOV	AL,[BL]	; Copy data from table to AL

OUT	01	; Output from AL register to port 01

CMP	AL,58	; Last item in data table ???

JZ	Start	; If yes then jump to Start

INC	BL	; In no then point BL to the next entry

JMP	Rep	; Jump back to do next table entry

END

; --------------------------------------------------------------

TASK

18)	Improve the traffic lights data table so there is an overlap with both sets of lights on red.

19)	Use a data table to navigate the snake through the maze.  This is on port 04.  Send FF to the snake to reset it.  Up, down left and right are controlled by the left four bits.  The right four bits control the distance moved.

20)	Write a program to spin the stepper motor. Activate bits 1, 2, 4 and 8 in sequence to energise the electromagnets in turn.  The motor can be half stepped by turning on pairs of magnets followed by a single magnet followed by a pair and so on.

21)	Use a data table to make the motor perform a complex sequence of forward and reverse moves.  This is the type of control needed in robotic systems, printers and plotters.  For this exercise, it does not matter exactly what the motor does.

; --------------------------------------------------------------

Nine - Parameters

Passing parameters using Registers,  RAM Locations and The Stack.

Program Uses

DB, MOV, CALL, PUSH, POP, JMP, ORG, ADD, RET, PUSHF, POPF, END

Here is the example program. 09param.asm

You can copy the example program from the help page and paste it into the source code editor.

Passing Parameters

Parameters can be passed in three ways.

CPU registers can be used.

RAM locations can be used.

The stack can be used.

The example program uses all three methods to add two numbers together.  The example tasks involve all three methods.

09Param.Asm

Copy this and paste it into the simulator source code editor.

; ----- EXAMPLE 9 ------- Passing Parameters -------------------

; ----- Use Registers to pass parameters into a procedure ------

JMP 	Start	; Skip over bytes used for data storage

DB	0	; Reserve a byte of RAM at address [02]

DB	0	; Reserve a byte of RAM at address [03]

Start:

MOV	AL,5

MOV	BL,4

CALL	30	; A procedure to add AL to BL

; Result returned in AL.

; ----- Use RAM locations to pass parameters into a procedure --

MOV	AL,3

MOV	[02],AL	; Store 3 into address [02]

MOV	BL,1

MOV	[03],BL	; Store 1 into address [03]

CALL	40

; ----- Use the Stack to pass parameters into a procedure ------

MOV	AL,7

PUSH	AL

MOV	BL,2

PUSH	BL

CALL	60

POP	BL

POP	AL	; This one contains the answer

JMP	Start	; Go back and do it again.

; ----- A procedure to add two numbers -------------------------

;	Parameters passed into procedure using AL and BL

;	Result returned in AL

;	This method is simple but is no good if there are a

;	lot of parameters to be passed.

ORG	30	; Code starts at address [30]

ADD	AL,BL	; Do the addition.  Result goes into AL

RET		; Return from the procedure

; ----- A procedure to add two numbers -------------------------

;	Parameters passed into procedure using RAM locations.

;	Result returned in RAM location

;	This method is more complex and there is no limit on

;	the number of parameters passed unless RAM runs out.

ORG	40	; Code starts at address [40]

PUSH	CL	; Save registers and flags on the stack

PUSH	DL

PUSHF

MOV	CL,[02]	; Fetch a parameter from RAM

MOV	DL,[03]	; Fetch a parameter from RAM

ADD	CL,DL	; Do the addition

MOV	[02],CL	; Store the result in RAM

POPF		; Restore original register

POP	DL	; and flag values

POP	CL

RET

; ----- A procedure to add two numbers -------------------------

; 	The numbers to be added are on the stack.

;	POP parameters off the stack

;	Do the addition

;	Push answer back onto the stack

;	The majority of procedure calls in real life make use

;	of the stack for parameter passing.  It is very common

;	for the address of a complex data structure in RAM to

;	be passed to a procedure using the stack.

ORG	60	; Code starts at address [60]

POP	DL	; Return address

POP	BL	; A parameter

POP	AL	; A parameter

ADD	AL,BL

PUSH	AL	; Answer ; The number of pushes must

PUSH	AL	; Answer ; match the number of pops.

PUSH	DL	; Put the stack back as it was before

RET

; --------------------------------------------------------------

END

Task

22)	Write a procedure that doubles a number.  Pass the single parameter into the procedure using a register.  Use the same register to return the result.

23)	Write a procedure to invert all the bits in a byte.  All the zeros should become ones.  All the ones should become zeros.  Pass the value to be processed into the procedure using a RAM location.  Return the result in the same RAM location.

24)	Write a procedure that works out Factorial N.  This example shows one method for working out factorial N.  Factorial 5 is 5 * 4 * 3 * 2 * 1 = 120.  Your procedure should work properly for factorial 1, 2, 3, 4 or 5.  Factorial 6 would cause an overflow.  Use the stack to pass parameters and return the result.  Calculate the result.  Using a look up table is cheating!

25)	Write a procedure that works out Factorial N.  Use the stack for parameter passing.  Write a recursive procedure.  Use this definition of Factorial.  

Factorial ( 0 )  is defined as 1.
Factorial ( N ) is defined as N * Factorial (N - 1).

To work out Factorial (N), the procedure first tests to see if N is zero and if not then re-uses itself to work out N * Factorial (N - 1).  This problem is hard to understand in any programming language.  In assembly code it is harder still.

Ten – Software Interrupts

Using software interrupts and the differences between interrupts and procedures.

Program Uses

JMP, DB, ORG, INT, IRET, MOV, NOT, OUT, END

Here is the example program. 10swint.asm

You can copy the example program from the help page and paste it into the source code editor.

Interrupts and Procedures

Interrupts are short code fragments that provide useful services that can be used by other programs.  Typical routines handle key presses, mouse movements and button presses, screen writing, disk reading and writing and so on.

An interrupt is like a procedure but it is called in a different way.  Procedures are called by jumping to the start address of the procedure.  This address is known only to the program that owns the procedure.  Interrupts are called by looking up the address of the interrupt code in a table of interrupt vectors.  The contents of this table is published and widely known.  MS DOS makes heavy use of interrupts for all its disk, screen, mouse, network, keyboard and other services.

By writing your own code and making the interrupt vector point to the code you wrote, the behaviour of interrupts can be completely altered.  Your interrupt code might add some useful behaviour and then jump back to the original code to complete the work.  This is called TRAPPING the interrupt.

Software interrupts are triggered, on demand, by programs.

Hardware interrupts are triggered by electronic signals to the CPU from hardware items.

Interrupt Vector Table

In the IBM compatible computer, addresses 0 to 1024 decimal are used for storing interrupt vectors.  The entries in this table of vectors point to all the code fragments that control MS DOS screen, disk, mouse, keyboard and other services.  The simulator vectors sit between addresses 0 and 15 decimal.  It is convenient to start a simulator program with a jump command that occupies two bytes.  This means that the first free address for an interrupt vector is [02].

Have another look at the example program.  10swint.asm

Calling an Interrupt

This is quite complex.  The command  INT   02  causes the CPU to retrieve the contents of RAM location 02.  After saving the return address onto the stack, the instruction pointer IP is set to this address.

The interrupt code is then executed.  When complete the IRET command causes the return from the interrupt.  The CPU instruction pointer IP is set to the address that was saved onto the stack earlier.

Trapping an Interrupt

If you wan to trap interrupt 02, change the address stored at address 02 to point to code that you have written.  Your code will then handle the interrupt.  When complete, your code can use IRET to return from the interrupt or it can jump to the address that was originally in address 02.  This causes the original interrupt code to be executed as well.  In this way, you can replace or modify the behaviour of an interrupt.

10SwInt.Asm

Copy this and paste it into the simulator source code editor.

; --------------------------------------------------------------

; An example of software interrupts.

; --------------------------------------------------------------

JMP	Start	; Jump past table of interrupt vectors

DB	51	; Vector at 02 pointing to address 51

DB	71	; Vector at 03 pointing to address 71

Start:

INT	02	; Do interrupt 02

INT	03	; Do interrupt 03

JMP	Start

; --------------------------------------------------------------

ORG	50

DB	E0	; Data byte - could be a whole table here

; Interrupt code starts here

MOV	AL,[50]	; Copy bits from RAM into AL

NOT	AL	; Invert the bits in AL

MOV	[50],AL	; Copy inverted bits back to RAM

OUT	01	; Send data to traffic lights

IRET

; --------------------------------------------------------------

ORG	70

DB	0	; Data byte  - could be a table here

; Interrupt code starts here

MOV	AL,[70]	; Copy bits from RAM into AL

NOT	AL	; Invert the bits in AL

AND	AL,FE	; Force right most bit to zero

MOV	[70],AL	; Copy inverted bits back to RAM

OUT	02	; Send data to seven segment display

IRET

; --------------------------------------------------------------

END

; --------------------------------------------------------------

TASK

26)	Write a new interrupt 02 that fetches a key press from the keyboard and stores it into RAM.  The IBM PC allocates 16 bytes for key press storage.  The 16 locations are used in a circular fashion.

27)	Create a new interrupt that puts characters onto the next free screen location.  See if you can get correct behaviour in response to the Enter key being pressed (fairly easy) and if the Back Space key is pressed (harder).

Eleven – Hardware Interrupts

Program Uses

CMP, INC, IRET, JMP, JZ, MOV, ORG, OUT, POP, POPF, PUSH, PUSHF, ROR, STI

Here is the example program. 11hwint.asm

You can copy the example program from the help page and paste it into the source code editor.

Hardware Interrupts

Hardware Interrupts are short code fragments that provide useful services that can be triggered by items of hardware.  When a printer runs out of paper, it sends a signal to the CPU.  The CPU interrupts normal processing and processes the interrupt.  In this case code would run to display "Paper Out" a message on the screen.  When this processing is complete, normal processing resumes.

This simulator has a timer that triggers  INT  02  at regular time intervals that you can pre-set in the Configuration Tab.  You must put an interrupt vector at address 02 that points to your interrupt code.  Look at the example.

Hardware interrupts are ignored unless the 'I' flag in the status register is set.  To set the 'I' flag, use the set 'I' command, STI.  To clear the 'I' flag, use the clear 'I' command CLI.

Hardware interrupts can be trapped in the same way that software interrupts can.

Hardware interrupts are triggered, as needed by disk drives, printers, key presses, mouse movements and other hardware events.

This scheme makes processing more efficient.  Without interrupts, the CPU would have to poll the hardware devices at regular time intervals to see if any processing was needed.  This would happen whether or not processing was necessary.  Interrupts can be assigned priorities such that a disk drive might take priority over a printer.  It is up to the programmer to optimise all this for efficient processing.

Calling an Interrupt

This is quite complex.  The command  INT   02  whether triggered by hardware or software, causes the CPU to retrieve the contents of RAM location 02.  After saving the return address onto the stack, the instruction pointer IP is set to the address that came from RAM.

The interrupt code is then executed.  When complete the IRET command causes the return from the interrupt.  The CPU instruction pointer IP is set to the address that was saved onto the stack earlier.

Hardware interrupts differ slightly from software interrupts.  A software interrupt is called with a command like  INT   02  and the return address is the next instruction after this.  IP + 2 is pushed onto the stack.  Hardware interrupts are not triggered by an instruction in a program so the return address does not have to be set past the calling instruction.  IP is pushed onto the stack.

Trapping an Interrupt

This is the same as trapping software interrupts.

Look at 10) SW Interrupts

11HwInt.Asm

Copy this and paste it into the simulator source code editor.

; --------------------------------------------------------------

; An example of using hardware interrupts.

; This program spins the stepper motor continuously and

; steps the traffic lights on each hardware interrupt.

; Uncheck the "Show only one peripheral at a time" box

; to enable both displays to appear simultaneously.

; --------------------------------------------------------------

JMP	Start	; Jump past table of interrupt vectors

DB	50	; Vector at 02 pointing to address 50

Start:

STI		; Set I flag. Enable hardware interrupts

MOV	AL,11	;

Rep:

OUT	05	; Stepper motor

ROR	AL	; Rotate bits in AL right

JMP	Rep

JMP	Start

; --------------------------------------------------------------

ORG	50

PUSH	al	; Save AL onto the stack.

PUSH	bl	; Save BL onto the stack.

PUSHF		; Save flags onto the stack.

JMP	PastData

DB	84	; Red		Green

DB	c8	; Red+Amber	Amber

DB	30	; Green		Red

DB	58	; Amber		Red+Amber

DB	57	; Used to track progress through table

PastData:

MOV	BL,[5B]	; BL now points to the data table

MOV	AL,[BL]	; Data from table goes into AL

OUT	01	; Send AL data to traffic lights

CMP	AL,58	; Last entry in the table

JZ	Reset	; If last entry then reset pointer

INC	BL	; BL points to next table entry

MOV	[5B],BL	; Save pointer in RAM

JMP	Stop

Reset:

MOV	BL,57	; Pointer to data table start address

MOV	[5B],BL	; Save pointer into RAM location 54

Stop:

POPF		; Restore flags to their previous value

POP	bl	; Restore BL to its previous value

POP	al	; Restore AL to its previous value

IRET

; --------------------------------------------------------------

END

; --------------------------------------------------------------

TASK

28)	Write a program that controls the heater and thermostat whilst at the same time counting from 0 to 9 repeatedly, displaying the result on one of the seven segment displays.  If you want a harder challenge, count from 0 to 99 repeatedly using both displays.  Use the simulated hardware interrupt to control the heater and thermostat.

29)	A fiendish problem.  Solve the Tower of Hanoi problem whilst steering the snake through the maze.  Use the text characters A, B, C Etc. to represent the disks.  Use three of the four rows on the simulated screen to represent the pillars.

99Keyb.Asm - Keyboard Input

When this window pops up, press any key on the keyboard.  Control, Alt and and Function keys are ignored.  Escape cancels the program run.  All other keys have their ASCII codes routed to the AL register in the CPU.

Copy this and paste it into the simulator source code editor.

; --------------------------------------------------------------

; Press  ALT+R  to run this program.

; Keyboard input and VDU Output.  Run the program.  When

; prompted, type some text, slowly, key press by key press.

; The text you entered will be displayed on the VDU screen.

; Pressing Enter stops the program run.

; --------------------------------------------------------------

MOV	BL,C0	; Address of top left corner of VDU

Start:

IN	00	; Keyboard Input goes into AL register.

CMP	AL,0D	; Was it the Enter Key ?

JZ	Stop	; If so then jump to Stop.

MOV	[BL],AL	; Copy text character in AL to the

; address that BL points to. ( VDU )

INC	BL	; BL now points to next screen location.

JMP	Start	; Jump back to the start.

Stop:

END		; Program Ends.

; --------------------------------------------------------------

99Tlight.Asm - Traffic Lights

Copy this and paste it into the simulator source code editor.

; ----- Control the traffic lights -----------------------------

; Press  ALT+R  to run this program.

; This simple example does not step the lights realistically

; It lacks a time delay facility.

; It would be better to use a data table.

; It works otherwise!

; Solving this properly is one of the learning tasks.

; --------------------------------------------------------------

CLO		; Close unwanted windows

rep:

MOV	al,84	; Red		Green

OUT	01	; Output to port 01 (traffic lights)

MOV	al,c8	; Red+Amber	Amber

OUT	01

MOV	al,30	; Green		Red

OUT	01

MOV	al,58	; Amber		Red+Amber

OUT	01

JMP	rep

END

; --------------------------------------------------------------

99SevSeg.Asm - Seven Segment Display

By setting appropriate bits, segments can be made to light up.  The least significant bit (the right hand one) is used to control which of the displays is active.  Set this bit to zero to send the data to the left hand display.  Set it to one to send data to the right hand display.  The program below shows how this can be done.

Copy this and paste it into the simulator source code editor.

; --------------------------------------------------------------

; Press  ALT+R  to run this program.

; A program to exercise the seven segment displays.

; It is up to you to modify this program to get the displays

; to step through recognisable sequences.  Try counting or

; programming the segments to spell H E L L O.

; Use a data table.  It is poor practice to mix code and data

; as happens in this example.

; --------------------------------------------------------------

start:

mov	al,FF	; Turn on all the right segments

out	02	; Send data to port 02.

mov	al,01	; Turn off all the right segments

out	02

mov	al,FE	; Turn on all the left segments

out	02

mov	al,0	; Turn off all the left segments

out	02

jmp	start	; Jump back to the start

end

; --------------------------------------------------------------

99HOn.Asm - Heater and Thermostat

The left most bit is used to turn the heater on and off.  The right most bit is used to detect whether the temperature is above about 21C.  Use OUT   03 to send data to the heater.  Use IN   03 to get data from the heater into the AL register.

Copy this and paste it into the simulator source code editor.

; --------------------------------------------------------------

; Press  ALT+R  to run this program.

; A program to turn on the heater

; --------------------------------------------------------------

CLO		; Close unwanted windows

MOV	AL,80	; Move 10000000 into AL

OUT	03	; Send AL to port 03

END

; --------------------------------------------------------------

99HOff.Asm - Heater and Thermostat

The left most bit is used to turn the heater on and off.  The right most bit is used to detect whether the temperature is above about 21C.  Use OUT   03 to send data to the heater.  Use IN   03 to get data from the heater into the AL register.

Copy this and paste it into the simulator source code editor.

; --------------------------------------------------------------

; Press  ALT+R  to run this program.

; A program to turn off the heater

; --------------------------------------------------------------

CLO		; Close unwanted windows

MOV	AL,0	; Move 00000000 into AL

OUT	03	; Send AL to port 03

END

; --------------------------------------------------------------

99Snake.Asm - Snake and Maze

To reset the snake, send FF to port 04.  The directions Up, Down, Left and Right are controlled by the left four bits.  The distance moved is controlled by the right four bits.

Copy this and paste it into the simulator source code editor.

; --------------------------------------------------------------

; Press  ALT+R  to run this program.

; A program to control the snake.

; A well written version of this program ought to use a data

; table to hold the numbers that control the snake.

; --------------------------------------------------------------

CLO

Start:

MOV	AL,FF	; Reset the maze

OUT	04	; Send control data to port 04

MOV	AL,27	; 2 means left.  7 distance units

OUT	04	; Send control data to port 04

MOV	AL,4F	; 4 means down.  Down 15 units.

OUT	04	; Send control data to port 04

OUT	04	; Do it twice to move further

MOV	AL,1E	; 1 means right.  E means 14.

OUT	04	; Send control data to port 04

MOV	AL,8F	; 8 means up.  15 distance units.

OUT	04	; Send control data to port 04

OUT	04	; Send control data to port 04

MOV	AL,27	; 2 means left.  7 distance units

OUT	04	; Send control data to port 04

JMP	Start	; Repeat the whole sequence

END

; --------------------------------------------------------------

99Step.Asm - Stepper Motor

About Stepper Motors

Normal motors have a power supply and they spin in a fairly uncontrolled way.

Stepper motors contain electromagnets that rotate the motor armature in small controlled steps.  The simulated motor can rotate in 5 or 10 degree steps.

A common type of stepper motor has four electromagnets.  To step the motor, each magnet is switched on in turn in the correct sequence.  By reversing the sequence, the motor goes backwards.

The motor can be made to go in half steps by turning on pairs of magnets and single magnets in the correct sequence.  When two magnets next to each other are turned on, the armature moves to a position half way between the two magnets.

If the magnets are turned on in the wrong sequence, the motor behaviour is unpredictable.  The simulated motor will simply not move.  It will beep in complaint.

Real life stepper motors can not easily be made to accelerate and decelerate in a controlled way.  This requires some difficult programming.  If the electrical signal to the magnets gets out of step with the rotating armature, the motor can stall.

Stepper motors are commonly used in printers, plotters and robots where accurate incremental movements are needed.

Copy this and paste it into the simulator source code editor.

; --------------------------------------------------------------

; Press  ALT+R  to run this program.

; Stepper motor.

; This shows how it can be made to rotate.

; The numbers controlling the motor should be put into a

; data table.  By altering the numbers in the table, the

; motor can be made to perform complex moves such as might

; be found in a printer, plotter or robot.

; --------------------------------------------------------------

start:

mov	al,1

out	05

mov	al,3

out	05

jmp	start

end

; --------------------------------------------------------------

99Keyb.Asm - Memory Mapped VDU

When this window pops up, press any key on the keyboard.  Control, Alt and and Function keys are ignored.  Escape cancels the program run.  All other keys have their ASCII codes routed to the AL register in the CPU.

Copy this and paste it into the simulator source code editor.

; --------------------------------------------------------------

; Press  ALT+R  to run this program.

; Keyboard input and VDU Output.  Run the program.  When

; prompted, type some text, slowly, key press by key press.

; The text you entered will be displayed on the VDU screen.

; Pressing Enter stops the program run.

; --------------------------------------------------------------

MOV	BL,C0	; Address of top left corner of VDU

Start:

IN	00	; Keyboard Input goes into AL register.

CMP	AL,0D	; Was it the Enter Key ?

JZ	Stop	; If so then jump to Stop.

MOV	[BL],AL	; Copy text character in AL to the

; address that BL points to. ( VDU )

INC	BL	; BL now points to next screen location.

JMP	Start	; Jump back to the start.

Stop:

END		; Program Ends.

; --------------------------------------------------------------

ASCII Table

American Standard Code for Information Interchange

The ASCII code has 128 standard characters and a further 128 characters that vary from machine to machine and country to country. ASCII be being replaced by a 16 bit code with 65536 characters that can represent every text character in the world including those used historically.

The first 128 ASCII characters are shown here.

DEC     0   1   2   3   4   5   6   7   8   9  10  11  12  13  14  15

HEX 0   1   2   3   4   5   6   7   8   9   A   B   C   D   E   F

000 00 Nul                         Bel Bak Tab Lf          Cr

016 10                                         Eof Esc

032 20 Spa  !   "   #   $   %   &   '   (   )   *   +   ,   -   .   /

048 30  0   1   2   3   4   5   6   7   8   9   :   ;   <   =   >   ?

064 40  @   A   B   C   D   E   F   G   H   I   J   K   L   M   N   O

080 50  P   Q   R   S   T   U   V   W   X   Y   Z   [   \   ]   ^   _

096 60  `   a   b   c   d   e   f   g   h   i   j   k   l   m   n   o

112 70  p   q   r   s   t   u   v   w   x   y   z   {   |   }   ~  Nul

The codes from 128 to 255 are not shown here.

Codes with special meanings to DOS, Printers or ANSI screens.

Decimal

0	Nul	NULL character.  ( End of text string marker. )

7	Bel	Bell or beep  character.

8	Bak	Delete or back space character.

9	Tab	TAB character.

10	Lf	Line Feed (start a new line).

13	Cr	is the Carriage Return code.

26	Eof	DOS End Of File code.

27	Esc	Escape code.  It has special effects on printers and ANSI screens.

ANSI = American National Standards Institute.

32	Spa	Space Character.

255	Nul	another NULL character.

Glossary

386

CPU chips in IBM compatible computers are typically numbered 086, 186, 286, 386, 486, 586 or Pentium. 086 chips are now regarded as old fashioned and slow. The Pentium pro was the latest chip when this was written. To run Windows, a 386 chip is the minimum recommended.

8 Bit CPU

The CPU has registers and connections to the outside world that are 8 bits wide. 16 bit and 32 bit CPUs are also common, more powerful and more expensive. 64 bit CPUs exist but are not common (1995)

80x86

The family of Intel chips numbered 8086, 80186, 80286, 80386, 80486 and Pentium.

Accelerator Key

Improves your productivity. For example Alt+F4 closes the current window and is quicker to press than the equivalent mouse or menu actions.

Analogue

Electronic systems that deal with continuously varying signals. Radio, TV and HiFi systems are all analogue. CD Players are digital but the digital signals must be converted to analogue before being sent to the HiFi system.

ANSI

American National Standards Institute

Architecture

CPU designs are more complex than typical building designs. Computer architecture is equivalent to building architecture. To make best use of a computer, it is useful to know something about the computer design or architecture.

ASCII

The American Standard Code for Information Interchange. This is an eight bit code. There are 128 characters which are standard. There are a further 128 characters that vary depending on the country and the graphics symbols required by and printers. American ASCII is being replaced by International Unicode.

ASCII

Code zero is used to mark the end of text strings.

ASM

The usual file extension for assembly code programs

Assembler

Assembly code

Human readable commands like MOV AL,33 correspond closely to CPU machine codes. The assembler program translates the human readable codes into machine codes readable by the CPU

Author

C Neil Bauers can be E Mailed on internet at nbauers@samphire.demon.co.uk

Backup copy

Copies of files kept in case of disaster. These should be kept in a secure place away from the computer system they belong to. Important files should be backed up in more than one place. Sod's law applies to back up files. The file you is the one you have failed to back up.

Base Address

The start address of an object stored in memory. For example : The IBM PC VGA screen base address is B800:0000 followed by 4000 more bytes.

Binary

Base two numbers used by digital systems. Count with two symbols [ 0 1 ] Binary numbers are composed of noughts and ones. Electronically this is achieved by circuits that are switched off or on.

Bit Masks

Patterns of noughts and ones used with AND, OR and XOR to extract or inserts bits into bytes.

Bits

Binary digits. Single digits that are nought or one.

Byte

Eight Bits.

Carriage Return

ASCII code 13 used to move the printer carriage or head to the left of the page. The screen cursor performs in the equivalent way. See also - Line Feed

Case Sensitive

Upper and lower case are taken to be different. This simulator is not case sensitive.

Chip

Shorthand for microchip or integrated circuit. The CPU is often referred to as the CPU Chip.

Click

Usually the left mouse button being pressed when the mouse is pointing at a screen object.

Clock

The CPU clock steps the computer and CPU at regular time intervals keeping all parts of the computer in step. A typical clock speed might be 160 Megahertz.

Comments

These begin with ';' and are used to explain what the program is doing. Good comments explain why things are being done. Bad comments simply repeat what is obvious by looking at the code.

Conditional Jumps

These jumps either take place or not depending on the flags in the status register. See JS, JNS, JO, JNO, JZ, JNZ, JMP.

Control Key

This is used to give keys special meanings. For example the combination of the control key with the F4 function key will close a window in some software packages.

Control Systems

Industrial and domestic equipment is frequently controlled by a small microcomputer called a microcontroller. The control system is programmed once for life so a TV

remote controller can not be re-programmed as a washing machine controller.

CPU

Central Processing Unit. The part of the computer that does the computations. Usually this is a single microchip.

Cursor

A flashing symbol that indicates position within text. Alternatively the mouse cursor indicates the mouse position. Special purpose cursors are used in some software.

Data tables

These store numbers, text or pointers to other data objects. It is easier to look after data in a table than data scattered throughout a program. It is good style to use data

tables.

Decimal Base 10 numbers.

Count with ten symbols. [ 0 1 2 3 4 5 6 7 8 9 ]

Digital Electronic

Systems that use binary.  Computers use binary numbers and are digital. HiFi systems do not use binary and are not digital. (A HiFi remote control system is digital) See analogue.

Direct

Same as Immediate.

Directory or Folder

File systems are organised into directories in much the same way that filing cabinets are organised into draws and folders. Your files should be stored in a directory that you have created. This keeps your files from getting muddled up with all the other files on the computer.

Divide by zero

This will cause an error. Any number divided by zero is infinitely big. This can not be calculated.

End Of File

ASCII code 26 used to indicate the end of MS DOS text files.

Escape ASCII code 27

This character is often interpreted in a special way by programs, VDUs and printers.

Executable Code

Non human readable program code executed by the CPU.

Explorer

See File Manager

Extension

The MS DOS file extension is zero or more characters after the dot in the file name. Word processor files often have .DOC on the end. Assembly code files end in .ASM

F1 Key

Commonly this accesses the on line help.

File

Data stored on disk or tape. When the data is loaded from the file into RAM, it could consist of a program or data used by the program.

File Manager or Explorer

A windows program that enables you to manage your files. Copying, renaming and deleting files and directories are typical file management tasks.

Flags

The Sign, Zero and Overflow flags in the status register indicate the outcome of the previous calculation. See S Flag, O flag and Z flag.

Floppy disk

Used to store files. 3.5 inch disks have a hard rectangular plastic casing to protect the thin floppy disk inside. Older disks are actually floppy. The case is bendy cardboard.

Folder

See Directory

Function keys

F1, F2 ... F10. These keys have special purposes depending on the software in use. F1 usually activates help. F10 usually activates the menu.

General Purpose Registers

AL, BL, CL and DL are used to store data and perform calculations.

Graphics

Images, pictures and geometrical shapes are examples of graphics. Windows displays everything as graphics. This gives good looking displays but a lot of processing is needed to achieve it.

Hard disk

A disk that can not normally be removed from the computer. Most computer files are stored on the hard disk. There should also be backup copies stored elsewhere in case the hard disk fails.

Hexadecimal

Base 16 numbers. Count with 16 symbols. [ 0 1 2 3 4 5 6 7 8 9 A B C D E F ] Hexadecimal and Binary are easily converted which is why hexadecimal is used.

Hot Keys

Ctrl+S and Ctrl+O are examples of hot keys. These give quick access to menu options. Ctrl+S gives the File Save command. Ctrl+O gives the File Open command.

Immediate

The instruction MOV AL,25 is an example of an immediate instruction. See also : Register, Indirect, Register indirect and MOV.

Indirect

Indirection

This is where data in RAM is referred to with a pointer. For example MOV AL,[20] moves the data from RAM location 20 into the AL register. [20] is a pointer to the RAM location. The technique is called indirection. See MOV, Immediate, Register, Register indirect

Instruction Pointer

IP points to the instruction being executed. When the instruction is complete, the IP is moved onto the next instruction.  In the RAM displays, the instruction pointer is highlighted red with yellow text.

Instruction Set

The set of instructions that are recognised by a CPU. Typical instructions are Move, Add and Subtract.

interrupt code

interrupt handler

Interrupt routine

A program fragment designed to be activated at any time that an interrupt occurs. The fragment is stored at an address pointed to by an interrupt vector. Interrupts can be triggered by hardware. For example a key press or the printer running out of paper cause a hardware interrupt. The CPU switches to the code that handles the interrupt. When finished, the CPU continues with its earlier task.

Interrupt Vector

A pointer stored in a table. The pointer points at the interrupt handler. See INT.

IO

Short for Input Output. See IN and OUT

Least significant bit

LSB. The right most bit in a byte which is worth 0 or 1.

LIFO

See Stack.

Line Feed

ASCII code 10 used to start a new line on the printed page or screen. See also - Carriage Return.

List File

This is generated by the simulator assembler. It contains the program written by the programmer. It also contains the machine codes generated by the assembler.

Low level

Low level programming is done using the CPU machine code or mnemonics the are close to the machine codes.

LSB

Least Significant Bit.  Right hand bit in a byte.  Has a value of 1.

Machine codes

Machine codes are executed by the CPU See Assembly codes.

Human readable commands	MOV  AL,55

Hexadecimal equivalent	D0 00 55

Binary machine code	110100000000000001010101

Megabytes

One million bytes.

Megahertz

MHz. Million clock cycles per second. A 33 MHz clock means that the CPU performs 33 million steps per second. These sorts of speeds are needed to fill screens with high resolution graphics quickly.

Memory Mapped

Memory mapped hardware is controlled by writing data into memory locations occupied by that hardware device. This simulator has a memory mapped screen so each screen position corresponds to a memory location.

Microchip

Complex electronic circuits miniaturised onto a single wafer or chip of silicon

Microcontroller

Usually a single chip microcircuit containing a CPU and RAM. Microcontrollers are used in TV remote controllers, washing machines, digital clocks, microwave ovens, industrial plant controllers, car engine management systems and computer games.

Microprocessor

A single chip CPU.

Mnemonic

A memorable and human readable item like MOV that corresponds to a non memorable item like 11010000 that means the same thing.

Most significant bit

MSB. The left most bit in a byte which is worth 128.

MSB

Most Significant Bit.  The left hand bit in a byte.  It has a value of 128 decimal or 80Hex if the byte is unsigned (positive numbers only).  It has a value of -128 if the byte is signed (positive and negative numbers).  The MSB has a value that depends on how wide in bits the data storage location is.

Multiplexing

Combining two or more data flows onto a single carrying medium. For example a thousand telephone calls can be carried on a single cable. De-multiplexing separates the channels and routes them to their correct destinations.

NULL

ASCII code zero used to mark the end of text strings.

O Flag

The O or overflow Flag in the status register indicates if the previous calculation overflowed its register.

Off Line

Computer resources that will be connected and made available at some later time. Off line resources do not need interaction with the user.

On Line

Computer resources that are connected and can be accessed with a negligible or short time delay. On line resources usually need interaction with the user.

OP Code

A binary code that the CPU can interpret as a command. These correspond to commands like MOV and ADD.

Overflow Flag

This is set if the result of the previous calculation was too big to fit the register.

Parameters

Data passed into procedures of functions. Parameters can be passed using registers, RAM locations or the Stack.

Peripherals

Hardware plugged into the computer. Anything from a keyboard or mouse to a power station or chemical works.

Pointers

In the command MOV AL,[25] the 25 is a pointer to the RAM location with address 25. See indirection.

Ports

Input Output Ports. Peripherals are connected to ports. The IN and OUT machine instructions are used to communicate with the peripherals.

Procedures

Code fragments that can be used many times during the execution of a program. See CALL and RET in the instruction set.

Programs

Instructions executed by a computer to perform tasks.

Ram

Random access memory. Electronic memory that stores bytes. Normal RAM forgets what it was storing when switched off.

Recursion

A powerful technique where a procedure or function re-uses itself to achieve a task.

Register

A location in the CPU where data is stored. This simulator has four general purpose registers called AL, BL, CL and DL. It has special purpose registers called IP, SR and SP.

Register

In the instruction CMP AL,BL registers are being compared. See also : Immediate, Indirect, Register indirect.

Register indirect.

In the instruction MOV AL,[BL] the BL register contains a pointer to a RAM location. The data in this RAM location is moved into AL. This is a register indirect move. See also : Immediate, Indirect and Register.

Repetition

This is achieved by using jump commands to make the program jump back and repeat instructions.

Reset CPU

Clear the general purpose registers to zero. Set IP to zero. Set the flags to Zero. Set the stack pointer to BF. The stack grows downwards from address FB.

Return address

The address stored on the stack that the program returns to when a procedure or interrupt is complete.

Run

Run a program. Programs are collections of stored instructions that are usually inactive. To run a program, it must be copied from disk into RAM and the CPU is given the address of the first instruction in the program so it can run it

Unicode

A 65536 character code including every text character for all languages in the world including most dead (disused) languages. This code is replacing ASCII

S Flag

The S or sign flag in the status register indicates if the previous calculation gave a negative result.

Save a file

Copy processed data from RAM onto disk.

Seven segment

displays are used in digital clocks, watches, calculators and so on. Numbers are built up by illuminating combinations of the seven segments.

Sign bit

The left most bit in a binary number that is used to indicate if the number is positive or negative.

Sign Flag

This is set if a calculation gives a negative result.

Signed Numbers

Numbers where the left most bit is the sign bit.

Simulator

Computer software that models reality in some way. Virtual reality aims to make the simulation so realistic that it seems real. Most simulations are designed to be useful rather than realistic.

Source Code

The human readable program code typed into the computer. See executable code.

SP

The stack pointer register.  In the RAM displays, the stack pointer is highlighted blue with yellow text.

SR

The status register.

Stack

An area of memory used for temporary storage according to the LIFO rule. Last in First out. The stack is used to save register contents for later restoration, pass parameters into procedures and return results, reverse the order in which data is stored, save addresses so procedures can return to the right place and there are other uses include doing postfix arithmetic.

Stack Pointer

Points to the next free location on the stack.  In the RAM displays, the stack pointer is highlighted blue with yellow text.

Status Flags

Status Register

The status Register contains status flags that indicate the outcome of the previous calculation. The flags are Sign, Zero and Overflow.

Stepper motor

A special motor that rotates in small controlled angular movements. It is used commonly in printers, plotters and medical instruments.

Task Switching

Use Alt Tab to task switch manually. Operating systems also task switch automatically. For example when word processing, the clock display continues to work because from time to time the operating system switches tasks to keep both going.

Thermostat

A temperature controlled switch. On when too cold. Off when too hot.

Token List

When programs are translated into machine code, one of the first steps is to convert the source code of the program into tokens. These are not usually human readable. The tokens are designed to occupy minimal memory.

Twos complement

Binary numbers where the left most bit determines whether the number is positive or negative.

Unsigned numbers

Numbers without a sign bit. These are always positive.

USERINFO.REG

Registration information is contained in this file.

VDU

Visual display unit. Computer output is commonly displayed on the VDU. There are several VDU display technologies.

Write

A simple Windows word processor. Data is saved to disk in a format unique to the Write program.

Z Flag

The Z or zero flag is set it the previous calculation result was zero.

Hexadecimal and Binary Numbers

Converting Between Binary and Hex

The CPU works using binary. Electronically this is done with switches that are either on or off. This is represented by noughts and ones. A single BIT or binary digit requires one wire or switch within the CPU. Usually data is handled in BYTES or multiples of bytes. A Byte is a group of eight bits. A byte looks like this

01001011

This is inconvenient to read, say and write down so programmers use hexadecimal to represent bytes. Converting between binary and hexadecimal is not difficult. First split the byte into two nybbles (half a byte) as follows

0100 1011

Then use the following table

BINARY	HEXADECIMAL	DECIMAL

0000	0		 0

0001	1		 1

0010	2		 2

0011	3		 3

0100	4		 4

0101	5		 5

0110	6		 6

0111	7		 7

1000	8		 8

1001	9		 9

1010	A		10

1011	B		11

1100	C		12

1101	D		13

1110	E		14

1111	F		15

EXAMPLE

Split the byte into two halves

01001011  becomes  0100  1011

Using the table above

0100  is  4

1011  is  B

The answer is

0100  1011  is  4B  in Hexadecimal.

To convert the other way take a hexadecimal such as E7.

Look up E in the table. It is 1110.

Look up 7 in the table. It is 0111.

E7 is 11100111.

The Full Instruction Set

Arithmetic Logic

Jump Instructions

Move Instructions

Compare Instructions

Stack Instructions

Procedures And Interrupts

Inputs and Outputs

Other Instructions

General Information

CPU Registers

There are four general purpose registers called AL, BL, CL and DL.

IP is the instruction pointer.

SP is the stack pointer.

SR is the status register.  This contains the I, S, O and Z flags.

Flags

Flags give information about the outcome of computations performed by the CPU.  Single bits in the status register are used as flags.  This simulator has flags to indicate the following.

S	The sign flag is set if a calculation gives a negative result.

O	The overflow flag is set if a result is too big to fit in 8 bits.

Z	The zero flag is set if a calculation gives a zero result.

I	is the hardware interrupts enabled flag.

Most real life CPUs have more than four flags.

Registers and Machine Codes

The registers and their equivalent machine code numbers are shown below.

Register names	AL	BL	CL	DL

Machine codes	00	01	02	03

Example : To add one to the CL register use the instruction

Assembly Code	INC	CL

Machine Code Hex	A4	02

Binary	10100100	00000010

A4 is the machine instruction for the INC command.

02 refers to the CL register.

The assembler is not case sensitive.  mov is the same as MOV and Mov.

Hexadecimal numbers may not have more than two hex' digits.

Hexadecimal numbers

15,  3C and FF  are examples of hexadecimal numbers.  When using the assembler,  all numbers should be entered in hexadecimal.  The CPU window display the registers in binary, hexadecimal and decimal.  Look at the Hexadecimal and Binary page for more detail.

Negative numbers

FE is a negative number.  Look at the Negative Numbers table for details of twos complement numbers.

In a byte, the left most bit is used as a sign bit.  This has a value of minus 128 decimal.

Bytes can hold signed numbers in the range -128 to +127.

Bytes can hold unsigned numbers in the range 0 to 255.

Indirection

When referring to data in RAM, square brackets are used.  For example [15] refers to the data at address 15hex in RAM.

The same applies to registers.  [BL] refers to the data in RAM at the address held in BL.  This is important and frequently causes confusion.

These are indirect references.  Instead of using the number or the value in the register directly, these values refer to RAM locations.  These are also called pointers.

Comparing with 80x86 Chips

At the mnemonic level, the simulator instructions look very like 80x86 assembly code mnemonics.  Sufficient instructions are implemented to permit realistic programming but the full instruction set has not been implemented.  All the simulated instructions apply to the low eight bits of the 80x86 CPU.  The rest of the CPU has not been simulated.

In the registered version, CALL, RET, INT, IRET and simulated hardware interrupts are available so procedures and interrupts can be written.

Most of the instructions behave as an 80x86 programmer would expect.  The MUL and DIV (multiplication and division) commands are simpler than the 80x86 equivalents.  The disadvantage of the simulator approach is that overflow is much more probable.  The simulator versions of ADD and SUB are realistic.

The 8086 DIV instruction calculates both DIV and MOD in the same instruction.  The simulator has MOD as a separate instruction.

The machine codes are quite unlike the 80x86 machine codes.  They are simpler, less compact but designed to make the machine code as simple as possible.

With 80x86 machine code, a mnemonic like  MOV  AL,15   is encoded in two bytes.   MOV AL,   is encoded into one byte and the   15   goes into another.  This means that a lot of different machine OP CODES are needed for all the different combinations of MOV commands and registers.

This simulator needs three bytes.   MOV   is encoded as a byte sized OP CODE.  AL  is encoded as a byte containing   00.  The  15   goes into a byte as before.  This is not very efficient but is very simple.

Arithmetic Instructions - Flags are set.

The Commands

ADD  Add.

SUB  Subtract.

MUL  Multiply.

DIV  Divide.

MOD  Remainder left over after a division.

AND  1 AND 1 gives 1.  Any other input gives 0.

OR   0 OR  0 gives 0.  Any other input gives 1.

XOR  Equal inputs give 0.  Non equal inputs give 1.

INC  Add one.

DEC  Subtract one.

NOT  Invert bits.  1 becomes 0.  0 becomes 1.

ROL  Rotate bits left.  Bit at left end moved to right end.

ROR  Rotate bits right. Bit at right end moved to left end.

SHL  Shift bits left.  Left  bit is lost.

SHR  Shift bits right. Right bit is lost.

Direct Examples

Assembler	Machine Code

ADD   AL,BL	A0 00 01

SUB   CL,DL	A1 02 03

MUL   AL,CL	A2 00 02

DIV   BL,DL	A3 01 03

MOD   DL,BL	A6 03 01

AND   CL,AL	AA 02 00

OR    CL,DL	AB 02 03

XOR   BL,AL	AC 01 00

INC   AL	A4 00

DEC   BL	A5 01

NOT   CL	AD 02

ROL   DL	9A 03

ROR   AL	9B 00

SHL   BL	9C 01

SHR   CL	9D 02

Immediate Examples

Assembler	Machine Code

ADD   AL,15	B0 00 15

SUB   BL,05	B1 01 05

MUL   AL,10	B2 00 10

DIV   BL,04	B3 01 04

MOD   DL,20	B6 03 20

AND   CL,55	BA 02 55

OR    CL,AA	BB 02 AA

XOR   BL,FF	BC 01 FF

Examples

ADD   CL,AL

Add CL to AL and put the answer into CL.

ADD   AL,22

Add 22 to AL and put the answer into AL.

The answer always goes into the first register in the command.

DEC   BL

Subtract one from BL and put the answer into BL.

The other commands all work in the same way.

Flags

If a calculation gives a zero answer, set the Z zero flag.

If a calculation gives a negative answer, set the S sign flag.

If a calculation overflows, set the O overflow flag.

An overflow happens if the result of a calculation has more bits than will fit into the available register. With 8 bit registers, the largest numbers that fit are -128 to + 127.

Jump Instructions

These instructions do NOT set the Z, S or O flags.

Instructions	JMP	JZ	JNZ	JS	JNS	JO	JNO

Machine codes	C0	C1	C2	C3	C4	C5	C6

JMP	Jump	Unconditional jump.

JZ	Jump Zero	Jump if the zero flag (Z) is set.

JNZ	Jump Not Zero	Jump if the zero flag (Z) is NOT set.

JS	Jump Sign	Jump if the sign flag (S) is set.

JNS	Jump Not Sign	Jump if the sign flag (S) is NOT set.

JO 	Jump Overflow	Jump if the overflow flag (O) is set.

JNO	Jump Not Overflow	Jump if the overflow flag is NOT set.

The full 8086 instruction set has many other jumps.

There are more flags in the 8086 as well!

Examples

JMP	HERE	; This is the assembly code

C0	??	; This is the equivalent machine code.

; ?? is calculated by the assembler.

HERE:		; This is the label used by the assembler

; when calculating jump addresses.

The JMP command will alter the instruction pointer register, by an amount calculated by the assembler.  It is possible to jump forwards or backwards. The longest jumps are +127  and -128.  This is because these are the biggest numbers that can be stored in eight bit memory locations.

The other jumps work in the same way except the jump is conditional on the correct flag bits being set or clear.

Legal Labels

here:	; A nice correct label

here::	; Not allowed  Only one colon is permitted.

1234:	; Not allowed.  Labels must begin with a letter or '_'.

Move Instructions

Register to Register Moves

Here data is moved from one register to another.

NOTE	This simulator does NOT support this type of move.

To do a move such as

MOV	AL,BL

it is necessary to use two steps as follows ...

PUSH	AL

POP	BL

Immediate Moves

Here a number is moved into a register.  For example

Instruction	MOV	BL,45

Machine codes	D0	01 45

D0 is the machine instruction for an immediate move.

01 refers to the BL register.

45 is the number to be moved into the BL register.

Indirect Moves

Data is moved between registers and ram locations.

The data comes from or goes to random access memory (RAM).  The RAM address is used in the instruction.  For example ...

Instruction	MOV	BL,[15]

Machine codes	D1	01  15

D1 is the machine code for an indirect move.

01 refers to the BL register.

15 is the RAM address.  Note the square brackets.

The data is moved from RAM address [15] into the BL register.  The square brackets are used to show that RAM address 15 is meant and not the ordinary number 15.

To move data from a register into ram,  follow the example below ...

Instruction	MOV	[13],BL

Machine codes	D2	13   01

D2 is the machine code for an indirect move.

01 refers to the BL register.

13 is the RAM address.  Note the square brackets.

The data is moved from the BL register into RAM address [13].  The square brackets are used to show that RAM address 13 is meant and not the ordinary number 13.

Register Indirect Moves

One register contains the data to be moved.  The other register contains the random access memory (RAM) address for the source or destination of the data.

For example ...

Instruction	MOV	DL,[AL]

Machine codes	D3	03  00

D3 is the machine instruction for a register indirect move.

03 refers to the DL register.

00 refers to the AL register.

The AL register contains the RAM address the data.  The data is moved from RAM into the DL register.

Data can be moved in the other direction as follows ...

Instruction	MOV	[DL],AL

Machine codes	D4	 03  00

D4 is the machine instruction for a register indirect move.

03 refers to the DL register.

00 refers to the AL register.

The DL register contains the address in RAM of the data.  The data is moved from the DL register into RAM.

Compare Instructions

When the simulator does a comparison using CMP, it does a subtraction of the two values it is comparing.  If the values are equal, the subtraction gives a zero result and the Z flag is set.  If the number being subtracted is smaller than the other, no flags are set.  If the number being subtracted was greater than the other than a negative answer results so the sign flag is set.  Use JZ and JS or JNZ and JNS to test the result of a CMP command.

Register Comparisons

Here two registers are compared.

For example

Instruction	CMP	AL,BL		; Work out AL - BL

Machine codes 	DA	00 01

DA is the machine instruction for register comparison.

00 refers to the AL register.

01 refers to the BL register.

If the registers hold equal values,  then the (Z) flag is set.  If the first register (AL) holds a value greater than that in the second, the (S) flag is set.  No flags are set otherwise.

Immediate Comparisons

Here a register is compared with a number.

For example ...

Instruction	CMP	AL,0D		; Work out AL - 0D

Machine codes 	DB	00 0D

DB is the machine instruction for immediate comparison.

00 refers to the AL register.

0D is the number being compared.

If the register holds a value equal to the number,  the (Z) flag is set.  If the register (AL) holds a value greater than the number, the (S) flag is set.  No flags are set otherwise.

Note : This is how you would test to see if the Enter key had been pressed.

Indirect Comparisons

A register is compared with the data held in a random access memory (RAM) location.

For example ...

Instruction	CMP	CL,[20]	; Work out CL - [20]

Machine codes	DC	02  20

DC is the machine instruction for indirect comparison.

02 refers to the CL register.

20 is the RAM address where the number being compared is held.

If the register holds a value equal to the number,  the (Z) flag is set.  If the register (CL) holds a value greater than the number, then the (S) flag is set.  No flags are set otherwise.

Stack Instructions

These instructions do NOT set the Z, S or O flags.

After pushing items onto the stack, always pop them off the stack in reverse order.  This is because the stack works by Last In First Out  LIFO.

The stack is used to ...

save register contents for later restoration.

pass parameters into procedures and return results.

reverse the order in which data is stored.

save addresses so procedures can return to the right place.

other uses include doing postfix arithmetic.

The register contents are saved and then restored later.  The stack obeys the last in first out rule. ( LIFO )

PUSH is used to put data onto the stack.

POP is used to get data off the stack.

For example ...

Instruction	PUSH	BL

Machine codes	E0	01

E0 is the machine instruction for PUSH.

01 refers to the BL register.

and ...

Instruction	POP	BL

Machine codes	E1	01

E1 is the machine instruction for POP.

01 refers to the BL register.

The flags register can also be saved and restored using the stack.

For example ...

Instruction	PUSHF

Machine codes	EA

EA is the machine instruction for PUSHF.

and ...

Instruction	POPF

Machine codes	EB

EB is the machine instruction for POPF.

Procedures and Interrupts

Available in the registered version.  Please register.

These do NOT set the Z, S or O flags.

It is a good idea to save registers and flags used by any procedure or interrupt and restore them after the procedure or interrupt has finished its work.  Use push and pushf to save.  Use pop and popf to restore values.

To call a procedure,  use this example ...

Instruction	CALL	30

Machine codes	CA	30

CA is the machine instruction for CALL.

30 is the address of the start of the procedure being called.

To return from a procedure ...

Instruction	RET

Machine codes	CB

At the end of the procedure there is a RET command.  This returns the program to the instruction after the CALL command.  The machine code of the RET command is CB.

The return address that the program returns to after executing the procedure is saved on the stack.

To call an interrupt routine,  use this example ...

Instruction	INT	03

Machine codes	CC	03

CC is the machine instruction for INT.  03 is the address of the interrupt vector used by the INT command.  At RAM location 03 is the interrupt vector.  This is the address of the start of the interrupt code.  The program jumps to the address of the interrupt code and the interrupt is executed.

To return from an interrupt ...

Instruction	IRET

Machine codes	CD

At the end of the interrupt code there is an IRET command.  This returns the program to the instruction after the INT command.  The machine code for IRET is CD.

The return address that the program returns to after executing the interrupt is saved on the stack.

Input and Output Instructions

These do NOT set the Z, S or O flags.

The ports are linked to the simulated outside world.

IN	00	reads from the keyboard.

OUT	01	sends data to the traffic lights.

For example ...

Instruction	IN	00

Machine codes	F0	00

F0 is the machine instruction for IN.  Inputting from a port.  00 is the port number.  The simulator has ports 0hex to 0Fhex.

Instruction	OUT	01

Machine codes	F0	01

F1 is the machine instruction for OUT.  Outputting to a port.  01 is the port number.

Other Instructions

HALT  NOP  CLO  ORG  DB  CLI  STI

These do NOT set the Z, S or O flags.

NOTE:  CLI clears the I flag.  Set sets the I flag.

HALT	Stop the program

Instruction	HALT

Machine codes   	00

00 is the machine instruction for HALT.  The program will cease to run if it encounters a HALT instruction.  Continuous running is cancelled by this command.  You can have several halt commands in one program.  There should be only one END.

NOP	Do nothing for one clock cycle

Instruction	NOP

Machine codes   	FF

FF is the machine instruction for NOP.  The program will do nothing for one clock cycle.  The program then continues as normal.  NOP is used to introduce time delays to allow slow electronics to keep up with the CPU.  These are called WAIT STATES.

CLO	Close all the peripheral windows

Instruction	CLO

Machine Codes	FE

FE is the machine code for CLO.  It applies to this simulator only, and is used to close peripheral windows.  This makes it easier to write demonstration programs without the screen getting too cluttered.

ORG	Code Origin

TO generate code from a starting address other than zero use ORG.  This is useful to place procedures, interrupts or data tables at particular addresses in memory.

ORG	30

No machine code is generated but the address at which subsequent code is generated is set to 30.

DB	Define a byte or string

A convenient way to load ASCII strings into RAM.

DB	"HELLO"

The ASCII codes of  H E L L O  are loaded into RAM.

DB	FF

A way to load hexadecimal values into ram.  It can be used to mark the end of strings with ASCII zero.  Interrupt vectors can be loaded with DB.  Data tables of hexadecimal values can be loaded.  CPU op codes can be loaded directly.  They work in the same way even though the assembler did not generate them.

CLI	Clear the I flag

If the I flag is cleared, hardware interrupts are disabled.  This is the default state for the simulator.  Resetting the CPU will also clear the I flag.  The timer that generates hardware interrupts will do nothing.

STI	Set the I flag

If the I flag is set, the simulator will generate  INT   02  at regular time intervals.  It is necessary to have an interrupt vector stored at address 02 that points to interrupt handler code stored elsewhere.  The interval between timer interrupts can be set using the slider in the Configuration Tab.  If interrupts occur faster than the processor can handle them, a simulated system crash will follow.  Adjust the CPU clock speed and the timer interval to prevent this – or cause it if you want to see what happens.

Instruction Set Summary

Move Instructions.  Flags NOT set.

Assembler       Machine Code  Explanation

MOV    AL,15    D0 00 15      AL   = 15     Copy 15 into AL

MOV    BL,[15]  D1 01 15      BL   = [15]   Copy RAM[15] into BL

MOV    [15],CL  D2 15 03      [15] = CL     Copy CL into RAM[15]

MOV    DL,[AL]  D3 03 00      DL   = [AL]   Copy RAM[AL] into DL

MOV    [CL],AL  D4 03 00      [CL] = AL     Copy Al into RAM[CL]

Direct Arithmetic and Logic Instructions. Flags are set.

Assembler	Machine Code  Explanation

ADD    AL,BL    A0 00 01      AL   =  AL + BL

SUB    BL,CL    A1 01 02      BL   =  BL - CL

MUL    CL,DL    A2 02 03      CL   =  CL * DL

DIV    DL,AL    A3 03 00      DL   =  DL / AL

MOD    AL,BL    A6 00 01      AL   =  AL mod BL

INC    DL       A4 03         DL   =  DL + 1

DEC    AL       A5 00         AL   =  AL - 1

AND    AL,BL    AA 00 01      AL   =  AL AND BL

OR     CL,BL    AB 02 01      CL   =  CL OR  BL

XOR    AL,BL    AC 00 01      AL   =  AL XOR BL

NOT    BL       AD 01         BL   =   NOT BL

ROL    AL       9A 00   Rotate bits left.   LSB  :=  MSB

ROR    BL       9B 01   Rotate bits right.  MSB  :=  LSB.

SHL    CL       9C 02   Shift bits left.    Discard MSB.

SHR    DL       9D 03   Shift bits right.   Discard LSB.

Immediate Arithmetic and Logic Instructions.  Flags are set.

Assembler	Machine Code  Explanation

ADD    AL,12    B0 00 12      AL  =  AL + 12

SUB    BL,15    B1 01 15      BL  =  BL - 15

MUL    CL,03    B2 02 03      CL  =  CL * 03

DIV    DL,02    B3 03 02      DL  =  DL / 02

MOD    AL,10    B6 00 10      AL  =  AL mod 10

AND    AL,0F    BA 00 0F      AL  =  AL AND 0F

OR     CL,F0    BB 02 F0      CL  =  CL OR  F0

XOR    AL,AA    BC 00 AA      AL  =  AL XOR AA

Compare Instructions.  Flags are set.

Assembler	Machine Code  Explanation

CMP    AL,BL    DA 00 01      Set 'Z' if AL = BL

Set 'S' if AL less than BL

CMP    BL,13    DB 01 13      Set 'Z' if BL = 13

Set 'S' if BL less than 13

CMP    CL,[20]  DC 02 20      Set 'Z' if CL = [20]

Set 'S' if CL less than [20]

Branch Instructions.  Flags NOT set.

Depending on the size and direction of the jump, different

machine codes can be generated.  Jump instructions cause

the instruction pointer ( IP ) to be altered.  The largest

possible jumps are +127 bytes and -128 bytes.

Assembler	Machine Code  Explanation

JMP    HERE     C0 12         Increase IP by 12

C0 FE         Decrease IP by 2 (twos complement)

JZ     THERE    C1 09         Increase IP by 9 if Z flag is set

C1 9C         Decrease IP by 100 if Z flag is set

JNZ    A_PLACE  C2 04         Increase IP by 4 if Z flag not set

C2 F0         Decrease IP by 16 if Z flag not set

JS     STOP     C3 09         Increase IP by 9 if S flag is set

C3 E1         Decrease IP by 31 if S flag is set

JNS    START    C4 04         Increase IP by 4 if S flag not set

C4 E0         Decrease IP by 32 if S flag not set

JO     REPEAT   C5 09         Increase IP by 9 if O flag is set

C5 DF         Decrease IP by 33 if O flag is set

JNO    AGAIN    C6 04         Increase IP by 4 if O flag not set

C6 FB         Decrease IP by 5 if O flag not set

Procedures and Interrupts.  Flags NOT set.

CALL, RET, INT and IRET are available only in the registered version.

Assembler	Machine Code  Explanation

CALL   30       CA 30         Save IP on stack and jump to 30.

RET             CB            Get IP from stack and jump to it.

INT    01       CC 01         Run code starting at address in 01

and save return address on the stack.

IRET            CD            Return from interrupt and continue

from address saved on the stack.

Stack Manipulation Instructions.  Flags NOT set.

Assembler	Machine Code  Explanation

PUSH   BL       E0 01         BL Saved onto the stack

POP    CL       E1 02         CL Restored off the stack

PUSHF           EA            Flags in status register saved

POPF            EB            SR restored from the stack

Input Output Instructions.  Flags NOT set.

Assembler	Machine Code  Explanation

IN     0C       F0 0C         Data input  from I/O port 0C to AL

OUT    0E       F1 0E         Data output from AL to I/O port 0E

Miscellaneous Instructions.  CLI and STI set I flag.

Assembler	Machine Code  Explanation

CLO             FE            Close all visible peripheral windows.

HALT            00            Halt the processor clock

NOP             FF            Do nothing for one clock cycle

STI             FC            Set Interrupts Flag - Enabled

CLI             FD            Clear Interrupts Flag - Disabled

ORG    30	None          Code origin. Start generating code

from address 30.

DB     "HELLO"  Define Bytes  Load ASCII codes of HELLO into RAM

DB     0D       Define Byte   Load hexadecimal number into RAM

The List File

The diagram above shows the layout of a list file.  The original program can be seen as well as the machine code generated by the assembler.  The numbers in square brackets are the addresses at which the machine codes were generated.  Program comments appear on the right.

Negative Numbers

Dec Hex	 Dec Hex	 Dec Hex	 Dec Hex

-128  80	-127  81	-126  82	-125  83

-124  84	-123  85	-122  86	-121  87

-120  88	-119  89	-118  8A	-117  8B

-116  8C	-115  8D	-114  8E	-113  8F

-112  90	-111  91	-110  92	-109  93

-108  94	-107  95	-106  96	-105  97

-104  98	-103  99	-102  9A	-101  9B

-100  9C	-099  9D	-098  9E	-097  9F

-096  A0	-095  A1	-094  A2	-093  A3

-092  A4	-091  A5	-090  A6	-089  A7

-088  A8	-087  A9	-086  AA	-085  AB

-084  AC	-083  AD	-082  AE	-081  AF

-080  B0	-079  B1	-078  B2	-077  B3

-076  B4	-075  B5	-074  B6	-073  B7

-072  B8	-071  B9	-070  BA	-069  BB

-068  BC	-067  BD	-066  BE	-065  BF

-064  C0	-063  C1	-062  C2	-061  C3

-060  C4	-059  C5	-058  C6	-057  C7

-056  C8	-055  C9	-054  CA	-053  CB

-052  CC	-051  CD	-050  CE	-049  CF

-048  D0	-047  D1	-046  D2	-045  D3

-044  D4	-043  D5	-042  D6	-041  D7

-040  D8	-039  D9	-038  DA	-037  DB

-036  DC	-035  DD	-034  DE	-033  DF

-032  E0	-031  E1	-030  E2	-029  E3

-028  E4	-027  E5	-026  E6	-025  E7

-024  E8	-023  E9	-022  EA	-021  EB

-020  EC	-019  ED	-018  EE	-017  EF

-016  F0	-015  F1	-014  F2	-013  F3

-012  F4	-011  F5	-010  F6	-009  F7

-008  F8	-007  F9	-006  FA	-005  FB

-004  FC	-003  FD	-002  FE	-001  FF

Positive Numbers

Dec Hex	 Dec Hex	 Dec Hex	 Dec Hex

+000  00	+001  01	+002  02	+003  03

+004  04	+005  05	+006  06	+007  07

+008  08	+009  09	+010  0A	+011  0B

+012  0C	+013  0D	+014  0E	+015  0F

+016  10	+017  11	+018  12	+019  13

+020  14	+021  15	+022  16	+023  17

+024  18	+025  19	+026  1A	+027  1B

+028  1C	+029  1D	+030  1E	+031  1F

+032  20	+033  21	+034  22	+035  23

+036  24	+037  25	+038  26	+039  27

+040  28	+041  29	+042  2A	+043  2B

+044  2C	+045  2D	+046  2E	+047  2F

+048  30	+049  31	+050  32	+051  33

+052  34	+053  35	+054  36	+055  37

+056  38	+057  39	+058  3A	+059  3B

+060  3C	+061  3D	+062  3E	+063  3F

+064  40	+065  41	+066  42	+067  43

+068  44	+069  45	+070  46	+071  47

+072  48	+073  49	+074  4A	+075  4B

+076  4C	+077  4D	+078  4E	+079  4F

+080  50	+081  51	+082  52	+083  53

+084  54	+085  55	+086  56	+087  57

+088  58	+089  59	+090  5A	+091  5B

+092  5C	+093  5D	+094  5E	+095  5F

+096  60	+097  61	+098  62	+099  63

+100  64	+101  65	+102  66	+103  67

+104  68	+105  69	+106  6A	+107  6B

+108  6C	+109  6D	+110  6E	+111  6F

+112  70	+113  71	+114  72	+115  73

+116  74	+117  75	+118  76	+119  77

+120  78	+121  79	+122  7A	+123  7B

+124  7C	+125  7D	+126  7E	+127  7F

Twos complement  The numbers work as follows.

The left most bit in an eight bit byte is the sign bit.

10101010

^

The sign bit has a value of -128decimal or -80 hexadecimal.

The other seven bits are treated as a normal positive number between 0 and 127.  This is true whether the overall number is positive or negative.  For example to store -1 the binary is

1 1111111		-128d  +  127d  =  -1d

^

-128d

To store 127decimal, the binary is

0 1111111		0  +  127d  =  127d

^

The sign bit is zero.

Pop Up Help

CPU    GENERAL PURPOSE REGISTERS

The CPU is where all the arithmetic and logic (decision making) takes place.  The CPU has storage locations called registers.  The CPU has flags which indicate zero, negative or overflowed calculations.  More information is included in the description of the system architecture.

The CPU registers are called AL, BL, CL and DL.

The machine code names are   00, 01, 02 and 03.

Registers are used for storing binary numbers.

Once the numbers are in the registers, it is possible to perform arithmetic or logic. Sending the correct binary patterns to peripherals like the traffic lights, makes it possible to control them.

; semicolon    begins a program comment.

Comments are used to document programs. They are helpful to new programmers joining a team and to existing people returning to a project having forgotten what it is about.

Good comments explain WHY things are being done.  Poor comments simply repeat the code or state the obvious.

RAM ADDRESS    Eg [7F] [22] [AL] [CL]

[7F] the contents of RAM at location 7F

[CL] the contents of the RAM location that CL points to. CL contains a number that is used as the address.

ADD    Add two values together.

ADD sets the CPU flags.

EXAMPLES

ADD	AL,12	; Add 12 to AL. Answer goes into AL

ADD	BL,CL	; Add CL to BL. Answer goes into BL

Machine codes equivalents

B0 00 12	; Add 12 to AL. Answer goes into AL

A0 01 02	; Add CL to BL. Answer goes into BL

AND    AND two binary values.

AND sets the CPU flags.

EXAMPLES

AND	AL,12	; And 12 with AL. Answer goes into AL

AND	BL,CL	; And CL with BL. Answer goes into BL

Machine codes equivalents

BA 00 12	; And 12 with AL. Answer goes into AL

AA 01 02	; And CL with BL. Answer goes into BL

The AND rule is that two ones give a one. All other inputs give nought. Look at this example ...

10101010

AND 00001111

--------

ANSWER 00001010 Note that one and one gives one.

The left four bits are masked to 0.

CALL and RET    Used with procedures

CALL and RET do not set the CPU flags.

EXAMPLE

CALL 50	; Call the procedure at address 50.

Machine codes equivalent

CA 50	; Call the procedure at address 50.

The CPU pushes the instruction pointer value IP + 2 onto stack. Later the CPU will return to this address.

The CPU instruction pointer is set to 50. The CPU executes instructions from this address until it reaches the RET command. It then pops the value of IP off the stack and jumps to this address where execution resumes.

CLI and STI    Used with interrupts

CLI clears the interrupt flag 'I' in the status register.  STI sets the interrupt flag 'I' in the status register.  The machine code for CLI is FD.  The machine code for STI is FC.

If 'I' is set, the CPU will respond to interrupts.  The simulator generates a hardware interrupt at regular time intervals that you can set.

If 'I' is set, there should be an interrupt vector at address [02]. The CPU will jump to the code that this vector points to whenever there is an interrupt.

CLO    Close unwanted windows

CLO does not set the CPU flags.  The machine code for CLO is FE.

CLO is not an x86 command. It closes all unnecessary simulator windows which would otherwise have to be closed manually one by one.

CMP    Compare

Compare CMP sets the CPU flags.

EXAMPLE

CMP	AL,0D	; Compare AL with 0D

Machine codes equivalents

DB	00 0D	; Compare AL with 0D

If the values being compared are ...

EQUAL set the 'Z' flag.

AL less than 0D set the 'S' flag.

AL greater than 0D set no flags.

EXAMPLE

CMP	AL,BL	; Compare AL with BL

Machine codes equivalents

DA	00 01	; Compare AL with BL

If the values being compared are ...

EQUAL set the 'Z' flag.

AL less than BL set the 'S' flag.

AL greater than BL set no flags.

EXAMPLE

CMP	CL,[20]	; Compare CL with 20

Machine codes equivalents

DC	02 20	; Compare CL with 20

If the values being compared are ...

EQUAL set the 'Z' flag.

CL less than contents of address 20 set the 'S' flag.

CL greater than contents of address 20 set no flags.

DB    Define Byte

DB does not set the CPU flags.

DB gives a method for loading values directly into RAM.

DB does not have a machine code.  The numbers or text after DB are loaded into RAM.

DB	22 ; Here is a data table loaded into RAM.

DB	33

DB	44

DB	0

DB	"Hello World!"	; ASCII codes loaded into RAM

DB	0			; End of text marked by NULL

DEC and INC    Add one, Subtract one

DEC and INC set the CPU flags.

EXAMPLES

DEC	AL	; Subtract one from AL.

INC	BL	; Add one to BL.

Machine codes equivalents

A5	00	; Subtract one from AL.

A4	01	; Add one to BL.

DIV and MOD    Divide and Remainder

DIV and MOD set the CPU flags.

DIV differs from the x86 DIV.

MOD is not and x86 command.

The x86 DIV calculates div and mod in one command.  The answers are put into different registers. This is not possible with the 8 bit simulator so div and mod are separated and simplified.

8 DIV 3 is 3 (with remainder 2). 8 mod 3 is 2

EXAMPLES

DIV	AL,5	; Divide AL by 5.  Answer goes into AL.

DIV	AL,BL	; Divide AL by BL. Answer goes into AL.

MOD	AL,5	; MOD AL by 5.  Remainder goes into AL.

MOD	AL,BL	; MOD AL by BL. Remainder goes into AL.

Machine codes equivalents

B3	00 05	; Divide AL by 5.  Answer goes into AL.

A3	00 01	; Divide AL by BL. Answer goes into AL.

B6	00 05	; MOD AL by 5.  Remainder goes into AL.

A6	00 01	; MOD AL by BL. Remainder goes into AL.

END

End does not set the CPU flags.

END stops further program execution.  The machine code for End and Halt is 00.

The simulator achieves this by stopping the CPU clock.  All code after END is ignored by the assembler.  There should be only one END in a program.

HALT

HALT does not set the CPU flags.

HALT stops further program execution.  The machine code for End and Halt is 00.

The simulator achieves this by stopping the CPU clock.  There may be several HALT commands in a program.

IN and OUT  Communicate with peripherals

IN and OUT do not set the CPU flags.

Input and Output using the ports.

EXAMPLES

MOV	AL,55	; Move 01010101 into AL

OUT	01	; Sent it to port 01 (Traffic Lights)

IN	03	; Read from port 03 into AL

IN	00	; 00 is the keyboard.

; 03 is the thermostat.

Machine codes equivalents

D0	00 55	; Move 01010101 into AL

F1	01	; Sent it to port 01 (Traffic Lights)

F0	03	; Read from port 03 into AL

F0	00	; 00 is the keyboard.

INT and IRET    Used with interrupts

INT and IRET do not set the CPU flags.

EXAMPLE

INT	02	; Call the interrupt whose

; start address is stored

; at address 02. The addresses

; 00..0F can be used as

; interrupt vectors.

IRET	; Return from the interrupt.

Machine codes equivalents

CC	02	; Call the interrupt

CD	; Return from the interrupt.

The CPU pushes the instruction pointer value IP + 2 onto stack. Later the CPU will return to this address.

The CPU instruction pointer is set to the address stored in location 02. The CPU executes instructions from this address until it reaches the IRET command.  It then pops the value of IP off the stack and jumps to this address where execution resumes.

JMP    Jump - Set IP to a new value

JMP does not set the CPU flags.

EXAMPLE

JMP	FOO	; Jump to the address labelled by FOO.

Machine codes equivalents

C0	12	; Jump forward 12 places.

C0	FE	; Jump back 2 places.

JMP works by adding a number to the CPU instruction pointer IP. This number can be in the range -128 .. +127.  This determines the longest possible jump.

JMP is unconditional. JMP ignores the CPU flags.

JNO Jump NOT Overflow

JNO does not set the CPU flags.

EXAMPLE

JNO	FOO	; Jump to the address labelled by FOO.

Machine codes equivalents

C6	12	; Jump forward 12 places.

C6	FE	; Jump back 2 places.

JNO works by adding a number to the CPU instruction pointer IP. This number can be in the range -128 .. +127.  This determines the longest possible jump.

JNO will perform the jump if the 'O' flag in the CPU is NOT set.

JNS Jump NOT Sign

JNS does not set the CPU flags.

EXAMPLE

JNS	FOO	; Jump to the address labelled by FOO.

Machine codes equivalents

C4	12	; Jump forward 12 places.

C4	FE	; Jump back 2 places.

JNS works by adding a number to the CPU instruction   pointer IP. This number can be in the range -128 .. +127.  This determines the longest possible jump.

JNS will perform the jump if the 'S' flag in the CPU is NOT set.

JNZ Jump NOT Zero

JNZ does not set the CPU flags.

EXAMPLE

JNZ	FOO	; Jump to the address labelled by FOO.

Machine codes equivalents

C2	12	; Jump forward 12 places.

C2	FE	; Jump back 2 places.

JNZ works by adding a number to the CPU instruction pointer IP. This number can be in the range -128 .. +127.  This determines the longest possible jump.

JNZ will perform the jump if the 'Z' flag in the CPU is NOT set.

JO Jump Overflow

JO does not set the CPU flags.

EXAMPLE

JO	FOO	; Jump to the address labelled by FOO.

Machine codes equivalents

C5	12	; Jump forward 12 places.

C5	FE	; Jump back 2 places.

JO works by adding a number to the CPU instruction   pointer IP. This number can be in the range -128 .. +127.  This determines the longest possible jump.

JO will perform the jump if the 'O' flag in the CPU IS set.

JS Jump Sign

JS does not set the CPU flags.

EXAMPLE

JS	FOO	; Jump to the address labelled by FOO.

Machine codes equivalents

C3	12	; Jump forward 12 places.

C3	FE	; Jump back 2 places.

JS works by adding a number to the CPU instruction pointer IP. This number can be in the range -128 .. +127.  This determines the longest possible jump.

JS will perform the jump if the 'S' flag in the CPU IS set.

JZ Jump Zero

JZ does not set the CPU flags.

EXAMPLE

JZ	FOO	; Jump to the address labelled by FOO.

Machine codes equivalents

C1	12	; Jump forward 12 places.

C1	FE	; Jump back 2 places.

JZ works by adding a number to the CPU instruction pointer IP. This number can be in the range -128 .. +127.  This determines the longest possible jump.

JZ will perform the jump if the 'Z' flag in the CPU is set.

MOV    Copy data from place to place

MOV does not set the CPU flags.

EXAMPLES

MOV	AL,12		; Copy 12 hex into AL

MOV	BL,[2F]	; Copy from memory location [2F] to BL

MOV	[33],CL	; Copy from CL to memory location [33]

MOV	AL,[DL]	; DL points to a memory location.

; Copy from this location to AL.

MOV	[BL],CL	; BL points to a memory location.

; Copy from CL to this location.

Machine codes equivalents

D0	00 12	; Copy 12 hex into AL

D1	01 2F	; Copy from memory location [2F] to BL

D2	33 02	; Copy from CL to memory location [33]

D3	00 03	; DL points to a memory location.

; Copy from this location to AL.

D4	02 03	; BL points to a memory location.

; Copy from CL to this location.

MUL    Multiply

MUL sets the CPU flags.

MUL differs from the x86 MUL.

The x86 MUL places the result into more than one register. This is not possible with the 8 bit simulator so MUL has been simplified. A disadvantage is that an overflow is much more likely to occur.

EXAMPLES

MUL	AL,5	; Multiply AL by 5. Answer goes into AL.

MUL	AL,BL	; Multiply AL by BL. Answer goes into AL.

Machine codes equivalents

B2	00 05	; Multiply AL by 5. Answer goes into AL.

A2	00 01	; Multiply AL by BL. Answer goes into AL.

NOP    Do nothing

NOP does not set the CPU flags.

Do nothing for one CPU clock cycle. This is needed to keep the CPU synchronised with accurately timed electronic circuits. The CPU might need to delay before the electronics are ready.  The machine code for NOP is FF.

NOT    Invert all bits

NOT sets the CPU flags.

EXAMPLES

NOT	CL	; Invert the bits in CL.

Machine codes equivalents

AD	02	; Invert the bits in CL.

If CL contained 01010101 before using NOT,

it will contain 10101010 afterwards.

OR    OR two binary values.

OR sets the CPU flags.

EXAMPLES

OR	AL,12	; Or 12 with AL. Answer goes into AL

OR	BL,CL	; Or CL with BL. Answer goes into BL

Machine codes equivalents

BB	00 12	; Or 12 with AL. Answer goes into AL

AB	01 02	; Or CL with BL. Answer goes into BL

The OR rule is that two noughts give a nought. All other inputs give one. Look at his example ...

10101010

OR 00001111

--------

ANSWER 10101111 Note that 0 OR 0 gives 0.

The right four bits are masked to 1.

ORG    Set code origin

ORG does not set the CPU flags.

ORG	50	; Generate code from address 50

ORG is not a CPU instruction. It is an instruction to the assembler program to tell it to generate code at a particular address. It is useful for writing procedures and interrupts. It can also be used to specify where in memory, data tables go.  There is no machine code equivalent.

PUSH and POP    Stack commands

PUSH and POP do not set the CPU flags.

Push saves a byte onto the stack. Pop gets it back.The stack is an area of memory that obeys the LIFO rule - Last In First Out. The stack can be used to

hold the return address of a procedure call

hold the return address of an interrupt call

pass parameters into procedures

get results back from procedures

save and restore registers and flags

reverse the order of data.

EXAMPLES

PUSH	AL	; Save AL onto the stack

POP	AL	; Restore AL from the stack

Machine codes equivalents

E0	00	; Save AL onto the stack

E1	00	; Restore AL from the stack

PUSHF and POPF    CPU flags and stack

PUSHF does not set the CPU flags.

POPF  sets the CPU flags.

Pushf saves the CPU flags onto the stack. Popf gets them back again. The stack is an area of memory that obeys the LIFO rule - Last In First Out.  The stack can be used to

hold the return address of a procedure call

hold the return address of an interrupt call

pass parameters into procedures

get results back from procedures

save and restore registers and flags

reverse the order of data.

EXAMPLES

PUSHF	; Save CPU flags onto the stack

POPF	; Restore CPU flags from the stack

Machine codes equivalents

EA	; Save CPU flags onto the stack

EB	; Restore CPU flags from the stack

ROL and ROR    Rotate bits left / right

ROL and ROR set the CPU flags.

EXAMPLES

ROL	AL	; Rotate bits left one place.

ROR	DL	; Rotate bits right one place.

Machine codes equivalents

9A	00	; Rotate bits left one place.

9B	03	; Rotate bits right one place.

The end bit is rotated round to the other end.

Before 10000110

After ROL 00001101

Before 10000110

After ROR 01000011

SHL and SHR    Shift bits left / right

SHL and SHR set the CPU flags.

EXAMPLES

SHL	AL	; Shift bits left one place.

SHR	DL	; Shift bits right one place.

Machine codes equivalents

9C	00	; Shift bits left one place.

9D	03	; Shift bits right one place.

The end bit is discarded.

Before 10000110

After SHL 00001100

Before 10000110

After SHR 01000011

SUB    Subtract two values.

SUB sets the CPU flags.

EXAMPLES

SUB	AL,12	; Subtract 12 from AL. Answer goes into AL

SUB	BL,CL	; Subtract CL from BL. Answer goes into BL

Machine codes equivalents

B1	00 12	; Subtract 12 from AL. Answer goes into AL

A1	01 02	; Subtract CL from BL. Answer goes into BL

XOR    Exclusive OR two values.

XOR sets the CPU flags.

Non equal inputs give a one.

EXAMPLES

XOR	AL,12	; 12 XOR AL. Answer goes into AL

XOR	BL,CL	; CL XOR BL. Answer goes into BL

Machine codes equivalents

BC	00 12	; 12 XOR AL. Answer goes into AL

AC	01 CL	; CL XOR BL. Answer goes into BL

XOR can be used to invert selected bits.

00001111 This is a bit mask.

XOR 01010101

--------

01011010 The left four bits are unaltered.

The right four bits are inverted.

HELP AVAILABLE

Pop up help and examples are available for the following key words.  Highlight the word and press the F1 key.

ADD	AL	AND	BL	CALL	CL	CLI	CLO	CMP	CPU	DB	DEC	DIV	DL	END	HALT	IN	INC	INT	IRET	JMP	JNO	JNS	JNZ	JO	JS	JZ	MOD	MOV	MUL	NOP	NOT	OR	ORG	OUT	POP	POPF	PUSH	PUSHF	RET	ROL	ROR	SHL	SHR	STI	SUB	XOR

Hexadecimal numbers can be confused with commands.  Typical hex numbers are 00, 02, 12, 1D, 7F, AB, F7, FF

The CPU registers are called AL, BL CL and DL.

Truth Tables and Logic

A mathematician called Bool invented a branch of maths for processing true and false values instead of numbers. This is called Boolean Algebra. Simple Boolean algebra is consistent with common sense but if you need to process decisions involving many values that might be true or false according to complex rules, you need this branch of mathematics.

The Commands - Flags are Set

AND	1 AND 1 gives 1.	Any other input gives  0.

OR	0 OR  0 gives 0.	Any other input gives  1.

XOR	Equal inputs give 0.	Non equal inputs give  1.

NOT	Invert input bits.	0 becomes 1. 1 becomes 0.

NAND	(NOT AND) 1 AND 1 gives 0.	Any other input gives  1.

NOR	(NOT  OR) 0 OR  0 gives 1.	Any other input gives  0.

Computers work using LOGIC. Displaying graphics such as the mouse cursor involves the XOR (Exclusive OR) command. Addition makes use of AND and XOR.  These and a few of the other uses of logic are described below.

The one line descriptions of the rules above are clearer if shown in Truth Tables.

AND

Both inputs must be true for the output to be true. AND is used for addition and decision making. In binary, 1 + 1 = 10. The 1 in the answer 10 comes from 1 AND 1. The 0 in the answer 10 comes from 1 XOR 1. These rules combined make addition possible.

------------

A B   Output

------------

0 0     0

0 1     0

1 0     0

1 1     1

OR

Both inputs must be false for the output to be false. OR is used in decision making. Both AND and OR are used for Bit Masking. Bit masking is used to pick individual bits out of a byte or to set particular bits in a byte. OR is used to set bits to one. AND is used to set bits to nought. AND is used to test if bits are one. OR is used to test if bits are nought.

------------

A B   Output

------------

0 0     0

0 1     1

1 0     1

1 1     1

XOR

If the bits in a graphical image are XORed with other bits a new image appears. If the XORing is repeated the image disappears again. This is how the mouse and text cursors get moved around the screen. XOR is combined with AND for use in addition. XOR detects if the inputs are equal or not.

------------

A B   Output

------------

0 0     0

0 1     1

1 0     1

1 1     0

NAND

NAND is really AND followed by NOT. Electronic circuits are commonly built from NAND gates (circuits). Computer programming languages and this simulator do not provide NAND. Use NOT AND instead.

------------

A B   Output

------------

0 0     1

0 1     1

1 0     1

1 1     0

NOR

NOR is really OR followed by NOT. Electronic circuits are commonly built from NOR gates (circuits). Computer programming languages and this simulator do not provide NOR. Use NOT OR instead.

------------

A B   Output

------------

0 0     1

0 1     0

1 0     0

1 1     0

NOT

NOT is used to invert bits or True/False values. All the rules above had two inputs and one output. NOT has a single input and output.

------------

A   Output

------------

0     1

1     0

The Half Adder Truth Table

------------------

A B    SUM   CARRY

XOR   AND

------------------

0 0     0     0

0 1     1     0

1 0     1     0

1 1     0     1

Using the Editor

Controlling the source code editor in the simulator is similar to most word processors and text editors such as the Windows Notepad.

Undo an editing error	When you have an accident and delete or mess up something by mistake, you can press Ctrl+Z to UNDO the last thing you did.  This can be very useful.

Move the text cursor	For small movements, use the Arrow Keys, Home, End, Page Up and Page Down.  You can use the mouse too.

Larger movements	Hold down the Ctrl key and use the Arrow Keys, Home, End, Page Up, and Page Down.    You can use the mouse too.

Delete previous character	Backspace Key

Delete next character	Delete Key

Highlight a block of text	Hold down the Shift key and use the Arrow Keys, Home, End, Page Up and Page Down.  You can drag the mouse with the left button pressed to do this.

Highlight	whole words, lines or documents, hold down Shift and Ctrl and then use the Arrow Keys, Home, End, Page Up and Page Down.  Alternatively drag the mouse with the left button pressed.

Copy a highlighted block	Ctrl+C

Cut a highlighted block	Ctrl+X

Paste a highlighted block	Ctrl+V

Delete a highlighted block	Press the Delete key

Save a file	Alt+F  s	OR	Ctrl+S

Save a file with a new name	Alt+F  a

Open a file	Alt+F  o	OR	Ctrl+O

Quit	Alt+F  x

VDU Display

The memory mapped Visual Display Screen.

The 64 bytes of RAM at addresses from C0 to FF are mapped onto the 64 possible screen positions.  This gives four rows of sixteen characters.  Writing ASCII codes to these RAM locations causes text to appear on screen.

Here is a code fragment that does the job.

Copy this and paste it into the simulator source code editor.

MOV	AL,41	; 41 is the ASCII code of 'A'

MOV	[C0],AL	; C0 is address of first screen position

MOV	AL,42	; 42 is the ASCII code of 'B'

MOV	[C1],AL	; The next screen position

MOV	AL,43	; 43 is the ASCII code of 'C'

MOV	[C2],AL	; The third screen position

END

Lift Simulation

The task here is to control the lift as realistically as possible.  The control programs must be tested thoroughly as it is easy to write programs containing interesting bugs.

To send control data to the lift, use a pair of lines like this.  Use one to turn things on.  Use nought to turn things off.

MOV	AL,XX	; Where XX is a hexadecimal number

OUT	06	; Send data to the lift.

To get data from the lift, use a line like this.  A one means that the sensor is turned on.  A nought indicates the sensor is turned off.

IN	06	; Get control data from the lift.

The line above places control data into the AL register.  Here it can be tested to determine what state the lift is in.

|  | The lift simulation consists of a motor which raises and lowers the lift cage. When the lift is nearly up or down, the cage goes slowly. In mid shaft the lift moves faster. It is not possible to control this aspect of the lift behaviour. The simplest possible lift control system would just switch the motor manually. This could be simulated with keyboard key presses. This would not be realistic for a modern lift although older coal mine lifts used a system much like this. There are buttons that can be used to call the lift up or down. The motor can be programmed to respond to these switches. The call buttons light up when pressed. This light should be programmed to stay on until the lift arrives and then turned off. |
| --- | --- |

Real life lifts allow the call down button to be pressed while the lift is going up and vice versa.  Try to simulate this.

There are lift position sensor switches.  These can be used to stop the lift at the top or bottom of the lift shaft.

The control program must be running fast enough to detect events from the lift sensors in a reasonable time.  Normally, the control program should be run as fast as the simulator allows.  There might be problems controlling the lift with the simulator running on a slower computer.  The same problems can occur in real life too, but only when controlling events on a very short time scale.

The lift can be "damaged" in the following ways.

By turning on both the up and down motor directions at the same time.

By moving the cage too low or too high in the lift shaft.

To repair the damage, press the "Repair Crashed Lift" button.  Until this is done, the lift ignores all control data.