�������������������������������������������������������Ŀ

�������������������������������������������������������Ĵ

���Ĵ                                               ���Ĵ

���Ĵ       ���������   �ͻ    �  �����������       ���Ĵ

���Ĵ           �       � Ȼ   �       �            ���Ĵ

���Ĵ           �       �  Ȼ  �       �            ���Ĵ

���Ĵ           �       �   Ȼ �       �            ���Ĵ

���Ĵ       ���������   �    �ͼ       �            ���Ĵ

���Ĵ                                               ���Ĵ

���Ĵ         ������ͻ   �����ͻ   �                ���Ĵ

���Ĵ               ɼ   �  ��͹   �                ���Ĵ

���Ĵ             �ͼ    � �μ �   ���ͻ            ���Ĵ

���Ĵ             �      ��ʼ  �   �   �            ���Ĵ

���Ĵ             �      �����ͼ   �   �            ���Ĵ

���Ĵ                                               ���Ĵ

�������������������������������������������������������Ĵ

���������������������������������������������������������

b y

T e d   O ' C o n n o r

The program INT 70h is a very simple driver for the

hercules graphics card (aka the HGC).  It is intended to ease

the use of the graphics while still allowing full use of the

card at machine language speed.  It even has its own language

for accessing the different functions.

There are only 8 commands in the INT 70h language:

0  Stop the interupt program

1  Turn graphics on

2  Turn text on

3  Draw a point

4  Draw a line

5  Set the active page

6  Clear the given page

7  Fill the given page

Since there are only 7 instructions, I use the four most

significant bits in a byte to specify a function in a command

byte.  The four least significant bits specify a page of

memory in some functions (the hercules graphics card has two

graphics/text pages).

The language is a very simple one.  You set a byte in

memory to a value as specified above.  If it needs parameters

(i.e. screen coordinates for a point), you use the next few

WORDS of memory to specify them.  You use memory words (2

bytes) because a screen coordinate may be more than 255 which

is as large a value as a byte can handle.  One last important

note:

Always follow your program code by a

byte value of 0 in the command byte

position.  This tells the interrupt when

to stop executing your code.

This is an example of what your code should look like:

�����������������������������������������������������Ŀ

(1)   �   DB   11h              ; turn on the graphics mode �

�                         ;     (page 1)              �

(2)   �   DB   51h              ; set active page           �

(3)   �   DB   42h              ; draw a line using XOR     �

(4)   �   DW   10, 20, 40, 80   ; from (10,20) to (40,80)   �

�������������������������������������������������������

line (1):   10h --->  turn on graphics

+ 1h --->  page 1 (you can use page 0 or 1)

----

11h

line (2):   50h --->  set active page (for plotting points

and lines)

+ 1h --->  page 1 is active

----

51h

line (3):   40h --->  draw a line

+ 2h --->  use the XOR method

----

42h

line (4):   10, 20, 40, 80 --->  words (specified w/ DW

instead of DB)

So, this program would turn on the graphics page,  without

clearing it and plot a line using an exclusive or method.

To call this interrupt, you must do two things:

1)  put the address in the AX & BX registers

2)  use the instruction  INT  70h

The address in AX & BX is the segment where the code is being

kept is put into AX and the offset within that segment is put

into BX.  This can be accomplished in one of two ways:

MOV  AX, SEG <codename>

MOV  BX, OFFSET <codename>

INT  70h

or

MOV  AX, DS

LEA  BX, <codename>

INT  70h

Functions:

number of    purpose of

number  name             parameters   parameters

������ ����������������  ����������   �������������

10h   Turn on graphics   0  *

20h   Turn on text       0  *

30h   Draw a point       2  ^        X & Y coordinates

40h   Draw a line        4  ^        X&Y coordinates

of both end point

50h   Set the active pg  0  *

60h   Clear page         0  *

70h   Fill page          1  *        value to fill with

^ -- this means that the type of plotting used is in the four

least significant bits:

1:   turn the points chosen on

2:   use exclusive or to plot the points

(i.e. if the point is on, turn it

off. if it is off, turn it on.)

3:   turn the points chosen off

* -- this means that the page number for the function should

be in the four least significant bits

0:   page 0  (the page it usually is on when you

start)

1:   page 1  (used by the CGA card)

This is an example of a full MASM program using INT 70h:

CSEG      SEGMENT

ORG  100h

ASSUME CS:CSEG, DS:CSEG, SS:CSEG

START:    MOV  AX, CS          ; this gets the segment value

LEA  BX, Gr_code     ; this gets the offset

INT  70h             ; call the graphics interupt

MOV  AH, 7           ; DOS fn to get a key

INT  20h             ; call the DOS interupt

MOV  AX, CS          ; get the segment value

LEA  BX, Reset       ; get the offset

INT  70h             ; call the graphics interupt

INT  20h             ; end the program

Gr_code   DB   11h             ; turn on graphics (page 1)

DB   61h             ; clear page 1

DB   51h             ; make page 1 the active page

DB   31h             ; put a point (using OR)

DW   5, 5            ;   at (5, 5) on the screen

DB   42h             ; draw a line (using XOR)

DW   10, 20, 40, 80  ;   from (10, 20) to (40, 80)

DB   32h             ; switch a point (using XOR)

DW   6, 5            ;   at (6, 5)

DB   33h             ; turn off a point (mask using

AND)

DW   5, 5            ;   at (5, 5)

DB   0               ; signify end of code

Reset     DB   20h            ; reset to text mode

DB   70h            ; fill page 0 with . . .

DB   32, 7          ; spaces with normal attributes

DB   0              ; signify end of code

CSEG      ENDS

END  START