SMART R8 CONTROL v1.50

USER'S MANUAL

(Unregistered Version)

22 August 1997

Copyright (c) 1992-97 FineWare

MARK J. FINE

FineWare

11252 Cardinal Drive

Remington, Virginia 22734-2032

TABLE OF CONTENTS

WARRANTY, LICENSING AND REGISTRATION....................................1

INTRODUCTION............................................................3

INSTALLATION............................................................4

SETUP...................................................................4

RUNNING.................................................................4

Setup Utility......................................................5

Demo Mode..........................................................5

Main Screen Description............................................6

RADIO CONTROLS..........................................................8

Esc - Power........................................................8

Arrows - Stepped Tuning............................................8

Numbers - Direct Access Tuning.....................................8

Alt-Z - Step Increments............................................8

Alt-V - VFO........................................................9

Alt-= - VFO A=B....................................................9

Alt-A - ANT........................................................9

Alt-R - RF.........................................................9

Alt-G - AGC........................................................9

Alt-B - BW.........................................................10

Alt-M - MODE.......................................................10

Alt-K - NB.........................................................10

Alt-N - NOTCH......................................................10

Alt-S - SYNCHRO....................................................10

Alt-E - Memory Mode Operation......................................10

Alt-Y - Jump To Memory Location....................................10

Alt-I - Insert Memory Location.....................................10

Alt-C - Scanning Mode Operation....................................11

Alt-P - Scanning Parameters........................................11

Alt-T - Tape VOX Control...........................................11

DATABASE QUERIES........................................................12

F5 - What Shortwave Stations Are On Now............................12

Shift-F5 - What BCB/CB  Stations Are On............................12

Alt-F5 - What Utility Stations Are On..............................12

F6 - Find Station Frequencies......................................13

F7 - Station's Full Program Schedule...............................14

F8 - Who Else Uses This Frequency..................................15

Intuitive Database Searches........................................16

F9 - Save Changed Database.........................................16

Shift-F9 - Swap Databases (Registered Users Only)..................16

Alt-F9 - Dump Database To Disk (Registered Users Only).............16

F3 - List Selected DX Shows........................................17

OTHER FUNCTIONS.........................................................18

F1 - Display Help Screen...........................................18

F2 - Display Shortwave Band Plan...................................18

Alt-F - Run HF Propagation Prediction (Registered Users Only)......18

Alt-U - Publish Database on Printer (Registered Users Only)........19

Alt-L - Enter Log Information......................................19

F4 - Display Log Information.......................................20

Alt-D - Demo Mode..................................................20

Alt-Q - Clear Screen...............................................20

F10 - Shell to DOS.................................................20

Alt-X - Exit Program...............................................20

Smart R8 Control  version 1.50                                            i

EXPECTED FUTURE ENHANCEMENTS............................................21

CONCLUSION..............................................................21

ACKNOWLEDGMENTS.........................................................22

Smart R8 Control  version 1.50                                           ii

SMART R8 CONTROL

version 1.50

Copyright (c) 1992-97 FineWare

Thank you for choosing Smart R8 Control v1.50.  The system requirements for

operating this program are the Drake R8 or R8A Communications Receiver and

an MS-DOS based Computer running at least MS-DOS version 3.0, with a mini-

mum of 469k free base RAM and one available serial port.  Operation of

Smart R8 Control v1.50 under the OS/2 operating system (even in DOS Com-

patibility mode) should be avoided.  This users manual shows what the soft-

ware will do, how to set it up, and how to run it.  The distribution file

SMCON150.ZIP should contain the following files:

README.1ST     - Text file with special announcements, etc.

SC.EXE         - The Smart R8 Control v1.50 Program

SCSETUP.EXE    - The Smart R8 Control Setup Utility v1.50

SC.DOC         - This text

SWBCGIDE.DAT   - Shortwave frequency database file

STATIONS.DAT   - Shortwave stations database file

DXSHOWS.TXT    - Selected shortwave programming

SMARTCON.ICO   - Icon for Microsoft Windows

DEMO.BAT       - Starts Smart R8 Control in Demo Mode

SMARTCON.HIS   - Version history of Smart R8 Control

ORDER.FRM      - Ordering Information for Smart R8 Control v1.50

*    REGISTER.EXE   - Registration Utility.

*    SCT.EXE        - Database Utility v1.50

*    SCI.EXE        - Import Utility v1.50 for SWBC Schedules database

*    SCR.EXE        - Printing Utility v1.50

*    SCP.EXE        - Propagation Prediction Utility v1.50

*    SCP.FIL        - Coordinate file for distribution database

*    SCP.TRS        - Coordinate file for SWBC Schedules database

*    SCUTIL.DOC     - Documentation for SCT, SCI, SCR and SCP

*    USE.BAT        - Database selection batch file

Note:  The files REGISTER.EXE, SCT.EXE, SCI.EXE, SCR.EXE, SCP.EXE, SCU-

TIL.DOC and USE.BAT are not contained in the shareware distribution file

SMCON150.ZIP and are reserved for registered copies only.

Although fundamental tuning techniques are covered herein, this documenta-

tion assumes that the reader has some working knowledge of the Drake R8 or

R8A Communications Receiver.  However, it is suggested that the user print

out this text (SC.DOC) for future reference.

WARRANTY, LICENSING AND REGISTRATION.

This and all of the other files contained in the distribution file

SMCON150.ZIP are provided as shareware to be used and disseminated in an

Smart R8 Control  version 1.50                                 Page 1 of 22

unmodified form.  The author requests that no fee be charged by any agent

other than FineWare (Mark J. Fine), for the enjoyment or use of this soft-

ware.  Neither the author nor his associates are responsible for any re-

sults of using this software or hardware modification recommendations in-

cluded in this document on any computer system or radio receiver, whether

used or performed either properly or improperly.  The user hereby agrees

that he is using this software/modifying his hardware at his own risk.

This and all of the other files contained in the file SMCON150.ZIP are pro-

vided as is, with no warranties, express or implied, including but not lim-

ited to the implied warranties of fitness for a particular purpose, and all

such warranties are expressly and specifically disclaimed.  In no event

should the author or any of his associates be responsible for any indirect,

special, incidental or consequential damages or lost profits to you or any

other person or entity regardless of the legal theory, even if the author

or any of his associates have been advised of the possibility of such dam-

age.

No person or entity is authorized to expand or alter either these warran-

ties or this agreement; any such representation will not bind the author or

his associates.

The user is licensed to operate the software free of charge, for the pur-

poses of inspection, for a period of sixty (60) days.  After the 60 day pe-

riod expires, the user is expected to pay the nominal registration charge

of twenty-five United States dollars (US$25).  Checks or money orders may

be made payable to MARK J. FINE and can be sent to the following address:

MARK J. FINE

c/o FineWare

11252 Cardinal Drive

Remington, Virginia 22734-2032

Support of this shareware product ensures its continued development and

dissemination.  As a bonus for registering Smart R8 Control, the user will

receive future updates to the software, free of charge.  As an extra added

bonus, the registering party will receive free companion utilities as fol-

lows:

SCT.EXE - Allows the user to update and compile the shortwave database with

user data and frequency schedule changes.

SCI.EXE - Allows subscribers to FineWare's SWBC Schedules database to im-

port regularly updated shortwave schedules.

USE.BAT - Allows the user to select between the imported SWBC Schedules da-

tabase and the Smart R8 Control distribution database.

SCR.EXE - Allows the user to print the full shortwave database in a neatly

collated two-column newspaper style format.

SCP.EXE - Allows the user to predict HF propagation conditions to a par-

ticular station or area of the world.

Smart R8 Control  version 1.50                                 Page 2 of 22

SCUTIL.DOC - Documentation and special instructions for operating the above

companion utilities.

INTRODUCTION.

Because most shortwave broadcast stations change frequencies during the

day, it's often frustrating have to double check mounds of reference mate-

rial while trying to listen at the same time.  For example, consider the

following:

Scenario 1:  Upon settling in for a relaxing session of twirling the dial,

an unfamiliar station pops out of the ether.  While waiting for the station

to ID, you fumble through pages of reference material trying to get a

heads-up on what the station might be.

Scenario 2:  You have an urge to listen to a particular station such as Ra-

dio Sweden, Radio Netherlands, Voice of Russia, or Radio Australia (if the

conditions are right).  Again, you fumble through your references to find

out what frequency to tune to.  Not only have you found the station's pro-

gramming schedule, but the station won't be on for at least another hour.

For these and other cases, the Smart R8 Control program is designed to both

simplify and enhance the shortwave listening hobby.  Smart R8 Control inte-

grates a database of frequency information with the power of remotely con-

trolling the shortwave receiver.  Smart R8 Control emulates most of the R8

or R8A's front panel controls, adding the capabilities of tuning by user

selected increments (i.e., 5 kHz) and controlling an attached tape re-

corder; while adding the name of the station you're listening to and a dual

24-hour clock (local/UT) to the continuous display.

Aside from displaying the station you're tuned to, Smart R8 Control allows

smart queries to the integrated frequency database.  Using an internally

designed windowing environment, Smart R8 Control allows the user to choose

from a list of stations broadcasting at that time.  Once a station is se-

lected, a separate window is created containing that station's list of cur-

rent frequencies and target areas.  The user can now automatically tune the

receiver by selecting a frequency from the list.  A marking feature is in-

cluded, to allow the user to highlight particularly reliable frequencies in

the list (i.e., the 5975, 7325 and 9915 kHz evening frequencies for the BBC

to North America).  If a new frequency has been heard during that time pe-

riod, it can be added to the list.

The frequency database can also be queried to display a station's full fre-

quency schedule, when selected programs are on that particular day,  as

well as a query to find what other stations use the frequency the receiver

is tuned to.  Other ancillary features include a logging facility and a

'reminder' display of the shortwave frequency bands.

Smart R8 Control  version 1.50                                 Page 3 of 22

INSTALLATION.

Unarchive the entire distribution file SMCON150.ZIP into a single disk di-

rectory.  This is important, because the main program, SC.EXE, expects to

find all of the individual files and programs it creates and uses in a sin-

gle disk directory.  If this is the initial use of Smart R8 Control, please

pay close attention to the following 'SETUP' and 'RUNNING' sections.

SETUP.

First, it is suggested that most terminate but stay resident programs

(TSRs) are removed from the environment and that the following commands are

included in the user's CONFIG.SYS file:

DEVICE=[drive:][path]ANSI.SYS

FILES=25

BUFFERS=20

It is necessary (but not essential) that the computer's real time clock be

accurately set.  If no battery back up clock is used in the computer, the

DOS TIME and DATE commands should be used prior to running Smart R8 Con-

trol.  The importance of this will be detailed later.  It is essential,

though, that the user know the UT-to-local time offset, which of the com-

puter's serial ports the receiver is connected to, and which of the re-

ceiver's two antenna ports is connected to the shortwave antenna.  This in-

formation is only required the first time the program is run.

RUNNING.

Upon first starting Smart R8 Control, the user will be prompted to input

the specific radio (R8 or R8A that is being used.  This is essential, be-

cause the communications for one receiver is much different than the other.

This capability is also new in v1.40.

Next, the user will be prompted to input the number of hours required to

convert to Universal Coordinated Time (UT).  The main window of Smart R8

Control contains a real-time 24-hour UT/local time clock both for the

user's information, and for use in some of the program's other functions.

The user should set the offset with the number of hours required to be

added to his local time to achieve UT.  For example, the offset would be

`5' for winter time on the East Coast of the US, or `-10.5' for summer time

in South Australia.

The third piece of information required by Smart R8 Control is the COM port

number to which the receiver is connected.  The program can only recognize

ports 1 through 4, if they exist.  In some cases, ports 3 and 4 may not use

the standard IRQs and I/O addresses.  In this event, the user may adjust

the IRQ and address.

Smart R8 Control  version 1.50                                 Page 4 of 22

Lastly, the program will prompt for the preferred shortwave antenna port.

An entry of '1' will select the receiver's 50 Ohm (SO-259) connector, and a

'2' will select the receiver's 300 Ohm (spring-clip) connector.  This op-

tion serves as a default for those users who have installed the optional

VHF converter, and preempts the receiver from automatically switching to

port 1 when returning to HF, if this is not desired.

These three pieces of information will then be stored in an initialization

file, in the default directory, named 'SMARTCON.DAT'.  Upon future runs of

the program, this file will be automatically read at start-up.  Since these

options can change, the user can enter these options independently, upon

further runs of the program, at the command line:

SC G5               (Changes the UT offset to '5')

SC R1               (Sets the receiver to COM1:)

SC A2               (Sets the preferred antenna to port '2')

Setup Utility.

The Setup Utility is automatically invoked whenever any of the above pa-

rameters are entered on the command line.  The Setup Utility will also be

invoked if the file 'SMARTCON.DAT' is either missing or is of the incorrect

version.

The user has the option to manually invoke the Setup Utility before Smart

R8 Control is run.  Also within the setup utility, are the capabilites to

modify the default COM: port's IRQ and I/O address; as well as the user's

default printer type.  The user may also change any of the default screen

colors.  Where applicable, each change will be evidenced on the sample

setup screen.  The key commands used in the Setup Utility are as follows:

O         - Change the default radio from R8 to R8A.

G         - Increase the UT offset amount.

R         - Increase the allowable COM: port number.

I         - Modify the default port's IRQ and I/O address.

A         - Toggle the preferred antenna port number.

P         - Toggle the default printer type.  (Registered Users Only).

F1        - Change the default foreground color.

Shift F1  - Change the default background color.

F2        - Change the default alternate foreground color.

F3        - Change the default highlighting/cursor color.

F4        - Change the default window frame foreground color.

Shift F4  - Change the default window frame background color.

F5        - Change the default warning/error forground color.

Shift F5  - Change the default warning/error background color.

F10       - Exit the Setup Utility and save changes in SMARTCON.DAT.

Demo Mode.

The serial interface for the R8 requires a hardware handshake, which used

to hang the program if it was started without a radio attached.  A radio

detection scheme is resident within the software.  If the radio (or any

other serial device) is not detected on the specified serial port, the pro-

gram will automatically go into Demo Mode.  If Smart R8 Control is to be

Smart R8 Control  version 1.50                                 Page 5 of 22

run for database access only, or if the receiver is unavailable for any

reason, the program can be also forced into 'Demo Mode'.  A special batch

file, DEMO.BAT, has been included to start the software in Demo Mode.  Oth-

erwise, the program can be started without the hardware handshake engaged

by using a 'D' switch on the command line:

SC D

Main Screen Description.

After all initial prompts are answered, Smart R8 Control will read the fre-

quency database and go to its main screen.  The main screen's primary com-

ponent is the radio's faceplate window, located in the upper left hand cor-

ner.  The layout of the faceplate is similar to that of the R8, except that

it also displays the current tuning step size, the local and UT times, a

carrier present indicator, and space for up to four co-channel station

names.  A typical faceplate will look similar to the following:

*D-----------[ Smart R8 Control v1.50 ]----POWER--|

|   41m  T  7315.00 kHz  MEM  09 STEP(kHz) LOCAL  |

| Carrier  NB W  Notch  Scan  LC    5.00   23:33t |

| VFO  ANT   RF   AGC   BW  MODE  SYNCHRO   UTC   |

|  B=   2   Off  Slow  6.0   AM      *     03:33w |

| 0300-0400 smtwhfa WHRI Noblesville IN        na |

|                                                 |

|                                                 |

|                                                 |

|-UNREGISTERED COPY- F1 - Help <=================>|

Upper Left Corner:

Space is reserved for a database status indicator.  This space will display

a '*' only if the database has changed.

Space has been reserved for a Demo Mode indicator.  This space will display

a 'D' only if the program is in Demo Mode.

Upper Right Corner:

If a mouse driver is found, this space will display 'POWER' and will serve

as the power switch to the radio.

Line 1:

The current shortwave band is displayed in meters, for the currently tuned

frequency. During some operations, this space is reserved for a band

indicator.  The space will display 'ERROR' for bad entries, or if a

command was unsuccessfully sent to the receiver.  A 'WAIT' indication

will display during database or tuning operations.  Additional key-

strokes may be buffered while either indicator is showing.

Space is reserved for the Tape VOX indicator.  If the Tape VOX mode is

switched on, this space will display a 'T', as shown.  If the Tape VOX

mode is switched off, this space will be blank.  If a mouse driver is

found, this space will display a 't' if the Tape VOX mode is off.

The frequency is displayed in kHz for frequencies under 30 MHz, or in MHz

for tuning with the receiver's optional VHF Converter.  If the re-

ceiver is off, this space will display 'O F F'.

Smart R8 Control  version 1.50                                 Page 6 of 22

Space is reserved for the memory mode indicator 'MEM' if memory mode is ac-

tive.  The number of the memory channel currently selected will be

continuously displayed.  If a mouse driver is found, this space will

display a 'mem' if memory mode is inactive.

Line 2:

Space is reserved for a carrier indicator.  If a signal is present and is

at a level above the squelch threshold, 'Carrier' will display, as

shown.

Space is reserved for the status of the noise blanker.  If the noise

blanker is engaged, the indicator will show 'NB N' or 'NB W' for the

narrow or wide settings, respectively.  If a mouse driver is found,

this space will display 'nb', if the noise blanker is off.

Space is reserved for the status of the notch filter.  If the notch filter

is engaged, the indicator will display 'Notch', as shown.  If a mouse

driver is found, this space will display 'notch' if the notch filter

is off.

Space is reserved for a scanning indicator.  If the receiver is in scanning

mode the indicator will display 'Scan', as shown.  If a mouse driver

is found, this space will display 'scan' if the receiver is not scan-

ning.

The scanning method is shown as letters.  The first letter refers to 'M'

for Memory, 'L' for List, or 'A' for A-B modes.  The second letter re-

fers to 'S' for Seek, 'T' for Time, or 'C' for Carrier Detect types.

If these values are not set, this space will display '??'.

The tuning step size is displayed in kHz.

The local time is displayed in 24-hour format.  Next to the time, the day

of the week is displayed as a single letter.  'h' is used for Thurs-

day, and 'a' is used for Saturday.  All other letters pertain to the

first letter of that day.

Line 4:

Line 4 contains indicators for the VFO, antenna port, RF amplifier, AGC,

bandwidth, and choice of demodulation.  If the A VFO is set to equal

the B VFO, then a `=' will immediately follow the VFO indicator.

Space is reserved for the status of the synchronous detector.  If the syn-

chronous detector is engaged, the indicator will display a '*', as

shown.

Universal Coordinated Time is displayed in 24-hour format.  Next to the

time, the UT day of the week is displayed as a single letter.  The

same convention is used for that of the local time clock.

Lines 5-8:

The bottom section of the faceplate will display the name of the station

currently tuned to.  In the event that a few stations may be occupying

this frequency, up to four stations will be listed.  If the frequency

has been previously marked for a particular station, that station's

name will be highlighted.  The format of the shortwave database, hence

the information displayed, is based on the Shortwave Guide section of

Grove Enterprises' Monitoring Times magazine.  For those unfamiliar

with this format, it is described as follows:

The first field in each line will contain the time period this frequency

will be occupied by the station.  This period will be displayed to re-

flect a portion within the one-hour block.

Smart R8 Control  version 1.50                                 Page 7 of 22

The next field contains the days of the week for which this broadcast would

occur.  This field uses the same convention described for the local

and UT clocks.  For simplicity, if the broadcast is made daily, this

space will be empty, otherwise, the particular day will display in the

designated space, as shown above.

The next two fields will display the name or location of the station and

target area, if known.  Typical target areas are 'do' for domestic

service, 'om' for omnidirectional pattern, 'na' for North America,

'am' for the Americas, 'ca' for Central America, 'sa' for South Amer-

ica, 'eu' for Europe, 'af' for Africa, 'as' for Asia, 'pa' for the Pa-

cific, 'au' for Australia, or 'va' for some combination of target ar-

eas.  Some feeders may display a 'uf' or 'lf' in this area to desig-

nate that the transmission is in upper or lower sideband.

If the station is a medium wave station (AM Broadcast Band), the name and

target area field are expanded to show the station's output power in

kW and the approximate times of operation, if known.  The codes used

for this are 'an' for all night, 'ar' for all night-reduced power at

night, 'nh' for normal hours, 'nr' for normal hours-reduced power at

night, 'dr' for day time only-reduced power during dusk/dawn, and 'da'

for daytime operation only.

Bottom Line:

The bottom left corner will display the user's registration number.

(Registered users only).

The center of the bottom line will provide a reminder that F1 is the pro-

gram's help key.  If a mouse driver is found, clicking in this area

will bring up the help screen.  The help screen also serves as the

main menu for the mouse.  Clicking on any function key designator on

the help screen will invoke that key combination.

If a mouse driver is found, a tuning bar will be displayed in the bottom

right corner, as shown.  Clicking on the left half of the tuning bar

will decrease the receiver's frequency.  Clicking on the right half of

the bar will increase the receiver's frequency.  Similarly, clicking

on either arrow of the bar will nudge the receiver's frequency by 0.01

kHz (0.1 kHz for the VHF Converter).

RADIO CONTROLS.

Esc - Power.

Arrows - Stepped Tuning.

Numbers - Direct Access Tuning.

Alt-Z - Step Increments.

The receiver can be turned on and off with the keyboard 'Esc' key.  If a

mouse is used, this can be emulated by clicking on the 'POWER' icon.  The

receiver can be tuned in a stepping fashion with the left and right cursor

keys.  If a mouse is used, the receiver can be tuned by clicking on the

left or right halves of the tuning bar.  Clicking on the arrows of the tun-

ing bar will tune the receiver in a set increment of 0.01 kHz (0.1 kHz for

the VHF Converter).  If desired, the receiver can be direct-access tuned by

typing in the frequency numerically, and hitting return.  The stepping in-

crement defaults to 5 kHz upon program start-up.  This can be changed by

Smart R8 Control  version 1.50                                 Page 8 of 22

pressing Alt-Z, or by clicking in the stepsize area of the faceplate if a

mouse is used.  This will prompt for the increment to change.  The incre-

ment is entered in kHz and is valid for a range of .01 to 99999.99 kHz.

When using the direct-access tuning, typing 15, 15.000 or 15000 will yield

the same result: 15 MHz.  This only works between the frequency range of

.176 through 55 MHz.  Frequencies below .176 MHz and above 108 MHz must be

entered in MHz (i.e., .153 for 153 kHz or 147 for 147 MHz).  Since the re-

ceiver only tunes between .100 - 30,35 - 55, and 108 - 175 MHz, any other

frequency will yield an error message.  If the optional VHF Converter is

not installed, tuning in the latter two bands will yield an error message.

All of these key positions were specifically chosen because of the layout

of the 101 style keyboard.  On the 101, the 'Esc' key is in the upper left

hand corner of the keyboard, which keeps it nicely out of the way.  Also on

the 101 keyboard, are separate cursor and numeric keypads.  If the 'Num-

Lock' is depressed, it is extremely easy to punch in a direct-access fre-

quency or incrementally tune.

The user may cancel any user input.  This is accomplished by pressing the

'Esc' key while an input prompt is present.  If a mouse driver is present,

the user may click on the 'Esc' area on some of the input prompts.

Other R8/R8A features can be accessed as follows:

Alt-V - VFO.

The VFO can be toggled from A to B and vice versa by pressing Alt-V, or by

clicking in the VFO area if a mouse driver is present.

Alt-= - VFO A=B.

The A and B VFOs can be set to the same frequency and other radio settings,

by pressing Alt-=.  If the user is using the A VFO, it will be copied to

the B VFO, and vice versa.

Alt-A - ANT.

The antenna port can be changed by pressing Alt-A and entering '1' for port

1, '2' for port 2, or 'Conv' for the VHF Converter.  If a mouse driver is

present, clicking in the ANT area, and then on the '1', '2' or 'Conv' in

the prompt will accomplish the same thing.  The user may cancel this change

by either hitting the 'Esc' key, or clicking on 'Esc' in the user prompt.

Alt-R - RF.

The RF Amplifier can be changed by pressing Alt-R and entering 'Att' for

the attenuator, 'Pre' to engage the preamplifier, or 'Off' for no RF en-

hancement.  If a mouse driver is present, clicking in the RF area, and then

on the 'Att', 'Off' or 'Pre' in the prompt will accomplish the same thing.

The user may cancel this change by either hitting the 'Esc' key, or click-

ing on 'Esc' in the user prompt.

Alt-G - AGC.

The AGC setting can be changed by pressing Alt-G and entering 'Off','Slow',

or 'Fast'.  If a mouse driver is present, clicking in the AGC area, and

then on the 'Off', 'Slow' or 'Fast' in the prompt will accomplish the same

Smart R8 Control  version 1.50                                 Page 9 of 22

thing.  The user may cancel this change by either hitting the 'Esc' key, or

clicking on 'Esc' in the user prompt.

Alt-B - BW.

The IF bandwidth can be changed by pressing Alt-B and entering '0.5',

'1.8', '2.3', '4.0' or '6.0' for the respective filter setting in kHz.  If

a mouse driver is present, clicking in the BW area, and then on the '0.5',

'1.8', '2.3', '4.0' or '6.0' in the prompt will accomplish the same thing.

The user may cancel this change by either hitting the 'Esc' key, or click-

ing on 'Esc' in the user prompt.

Alt-M - MODE.

The demodulation mode can be switched by pressing Alt-M and entering 'AM',

'FM', 'CW', 'RTTY', 'LSB' or 'USB'.  If a mouse driver is present, clicking

in the MODE area, and then on the 'AM', 'FM', 'CW', 'RTTY', 'LSB' or 'USB'

in the prompt will accomplish the same thing.  The user may cancel this

change by either hitting the 'Esc' key, or clicking on 'Esc' in the user

prompt.

Alt-K - NB.

The noise blanker can be switched by pressing Alt-K and entering 'Narr',

'Wide' or 'Off'.  If a mouse driver is present, clicking on the NB area,

and then on the 'Narr', 'Wide' or 'Off' in the prompt will accomplish the

same thing.  The user may cancel this change by either hitting the 'Esc'

key, or clicking on 'Esc' in the user prompt.

Alt-N - NOTCH.

The notch filter can be toggled on and off by pressing Alt-N, or by click-

ing in the NOTCH area if a mouse driver is present.

Alt-S - SYNCHRO.

The synchronous detector can be toggled on and off by pressing Alt-S, or by

clicking in the SYNCHRO area if a mouse driver is present.

Note:  For those above controls that require further user entry, the selec-

tion can be abbreviated by using the first character of the response (i.e.,

the user can type 'R' instead of 'RTTY' when responding to the Mode con-

trol, Alt-M).  Unfortunately, there are no provisions to remotely control

receiver's tone, notch, squelch, passband offset, volume or RF control

knobs; nor the internal clock or timer settings.  There are provisions to

remotely control the tuning knob step display, but because of the features

provided by this program, the point is moot.

Alt-E - Memory Mode Operation.

Alt-Y - Jump To Memory Location.

Alt-I - Insert Memory Location.

Memory mode tuning can be toggled on and off by pressing Alt-E, or by

clicking in the MEM area if a mouse driver is present.  When in memory

mode, the 'MEM' indicator will light and the receiver will tune to the mem-

ory number continuously displayed on the faceplate.  Pressing Alt-E again

(or by clicking in the MEM area) will download the memory settings to the

current VFO, which is analogous to the receiver's M-V key.  In memory mode,

using the right and left cursor keys (or clicking on the tuning bar) will

step up and down the memory slots, respectively.  An alternate method for

Smart R8 Control  version 1.50                                Page 10 of 22

accessing a memory number is to press Alt-Y and enter the memory number.

The user may cancel this action by hitting the 'Esc' key.  If a mouse

driver is present, the Alt-Y key combination can be invoked by clicking on

the displayed memory number.  The receiver may be tuned by direct-access

without having to leave memory mode.  Smart R8 Control will handle this

automatically.

A memory number can be loaded (inserted) with the current settings by

pressing Alt-I and entering the memory number.  The user may cancel this

action by hitting the 'Esc' key.  If a mouse driver is present, the Alt-I

combination can be invoked by calling up the help screen and clicking on

the '@I' symbol.  Memory insertion can not be performed while operating

within memory mode.  This last feature is analogous to the V-M key on the

receiver, and only locations 0 through 99 are valid for the R8, 0 to 439

for the R8A.

Alt-C - Scanning Mode Operation.

Alt-P - Scanning Parameters.

Scanning mode can be accessed by pressing Alt-C, or by clicking on the

'SCAN' area if a mouse driver is present.  This function does not work very

well on the shortwave bands if the squelch is not properly set.  It is only

recommended if the RF section is switched to 'OFF' and the squelch is set

to open at approximately 10 to 20dB over S9.  Using the left and right cur-

sor keys during scanning mode (or by clicking on the tuning bar) will allow

the user to change scanning to increment in that direction, respectively.

If the scanning parameters have not been set, indicated by a '??' on the

faceplate, Smart R8 Control will prompt for them.

The parameters may be changed later by pressing Alt-P, or by clicking on

the PARAMETERS area if a mouse driver is present.  Scanning modes are en-

tered as 'M' for Memory (scans all memories), 'L' for List (scans specific

memory banks), or 'A' for A-B (scans between the frequencies in the A and B

VFOs).  This prompt can be answered by clicking on the desired scanning

mode in the prompt, if a mouse driver is present.  If list scanning is se-

lected, the program will prompt for the banks to be scanned (i.e., typing

'123' will scan banks 1, 2 and 3).  The program will then prompt for the

scanning type.  Scanning types are entered as 'S' for Seek (stop scanning

on carrier found), 'T' for Time (stop for a while on carrier found), or 'C'

for Carrier Detect (stop on carrier found, continue when dropped).  This

prompt can also be answered by clicking on the desired scanning type in the

prompt, if a mouse driver is present.  The user may cancel any step in this

procedure by either hitting the 'Esc' key, or clicking on 'Esc' in the user

prompt, if provided.  If memory or list scanning is used, the user must re-

member to enter memory mode and initially tune to a memory location within

the specified memory list.  Failure to do so has been known to confuse the

receiver.  Note that List scanning cannot be controlled for the R8A.

Alt-T - Tape VOX Control.

Smart R8 Control provides for a Tape VOX control.  If a tape recorder is

connected to the line audio and timer connections in the back of the re-

ceiver, pressing Alt-T will activate the timer relay in the receiver.  This

can also be accomplished by clicking on the 't' or 'T' indicator, if a

mouse driver is present.  This is provided as a handy 'footswitch' for a

controlling a tape recorder in order to capture radio programs or updates

Smart R8 Control  version 1.50                                Page 11 of 22

to the program schedules.  The Tape VOX control, once activated, will only

roll the tape if a carrier is present.  This way, if the receiver is being

used on the VHF Service frequencies (i.e., Police, Fire), 'dead air' will

not be recorded, thus compressing tape usage.  The Tape VOX mode will not

operate while the receiver is scanning.

DATABASE QUERIES.

F5 - What Shortwave Stations Are On Now.

Shift-F5 - What BCB/CB  Stations Are On.

Alt-F5 - What Utility Stations Are On.

This feature will query the database to provide a selectable list of sta-

tions that are on at that time.  This list will be shown in a scrollable

window in the bottom half of the full screen.  The stations are listed from

left to right by order of start time, end time, and country name.  Time pe-

riod annunciators have been added to the display in order to provide less

confusion.  Certain stations, such as the BBC, will jump around the window

upon subsequent calls due to the fact that most of the BBC entries are in-

dexed in 15 minute blocks.  This is despite the fact that the BBC broad-

casts around the clock.

If any of the frequencies that the station uses have been previously marked

by the user, the station name will be highlighted.  This will alert the

user that this station has been heard previously, during that time (see 'F6

- Find Station Frequencies', below).  The list of station names can be in-

dexed with a cursor bar.  If the station list contains more stations than

is shown in the visible window, arrows will be displayed inside the left

edge of the window.  Navigation of the cursor bar can be accomplished with

all of the normal cursor keys (arrows, page keys, home, and end).  Once a

station is highlighted with the cursor bar, it may be selected by pressing

the 'Enter' key.  This has the same effect as pressing F6 to find the sta-

tions broadcast frequencies (see 'F6', below).  If a frequency list is al-

ready showing, the user can automatically go back to that window by using

the 'Tab' key.

If a mouse driver is present, clicking the station name will display that

station's frequency list.  If the desired station is not listed, the window

may be paged by clicking in the upper or lower portions of the scroll bar.

It may be scrolled, one line at a time, by clicking on the arrows of the

scroll bar.  The scroll bar is located on the left hand side of the window.

Tabbing between the station and frequency windows can be accomplished by

clicking on the bottom line of the screen or clicking within the desired

window.

The user may query this window by three separate classes of HF (and MW)

broadcasts.  Pressing F5, alone, will display all scheduled and 24 hour

shortwave stations.  Pressing Shift and F5 together, will provide the user

a display of all AM Broadcast Band (BCB) entries and Citizen Band channels

in the database.  Pressing the Alt and F5 keys together will provide the

user a display of all utility stations (e.g., Amateur nets, military, mari-

time, time, press, etc.) in the database.  If a mouse driver is present,

Smart R8 Control  version 1.50                                Page 12 of 22

clicking on the 'F5', 'Sh', or '@' portions of the help screen will invoke

these functions.

F6 - Find Station Frequencies.

A shortcut to finding a particular station's frequencies is to use this di-

rect query.  After pressing the F6 function key, Smart R8 Control will

prompt the user to provide the name of the station to query.  The function

can also be invoked by clicking on the 'F6' symbol in the help screen if a

mouse driver is present.  The user may cancel this action by hitting the

'Esc' key, while the input prompt is displayed.  Once the string is en-

tered, and a matching station is on at that time, a window containing a

frequency list will appear along the right edge of the full screen.  Each

frequency will be associated with the time and days of broadcast (within

the one-hour block) and the intended target area.  Typically, on the East

Coast of North America, frequencies targeted for Asia and the Pacific can

be avoided during certain times of the day.  If any of the frequencies in

the list has been previously marked by the user, the frequency will be

highlighted.  It is anticipated that this will be used to alert the user

that this station has been previously heard on this frequency during that

time period.

The list of frequencies can be indexed with a pointer.  If the frequency

list contains more frequencies than is shown in the visible window, arrows

will be displayed inside the top edge of the window.  Navigation of the

pointer can be accomplished with the up and down cursor, up and down page,

home, and end keys.  Once a frequency is highlighted with the pointer, it

may be selected by pressing the 'Enter' key.  This will subsequently tune

the radio to that frequency.  If the target area is annotated as 'uf' or

'lf', the radio will automatically change to upper or lower sideband, re-

spectively.  If the station list window is present at the bottom of the

full screen, the user can automatically go back to that window by using the

'Tab' key.

If a mouse driver is present, clicking the desired frequency will tune the

receiver.  If the desired frequency is not listed, the window may be paged

by clicking in the upper or lower portions of the scroll bar.  It may be

scrolled, one line at a time, by clicking on the arrows of the scroll bar.

The scroll bar is located on the right hand side of the window.  Tabbing

between the station and frequency windows can be accomplished by clicking

on the bottom line of the screen or clicking within the desired window.

Mouse users may now automatically invoke this function, by clicking on the

left-hand (time block) portion of any displayed station ID line in the

faceplate window.

The frequency can be marked for future reference by pointing to it and by

pressing the 'Ctrl' and 'Enter' keys, simultaneously.  This feature is a

useful way to draw attention to frequencies audible at the listener's loca-

tion.  A marked frequency will be evidenced by a highlighted entry in this

window, as well as the station name in the faceplate window if the receiver

is currently tuned to it.

The user can make minor changes to the database through this window.  A

frequency entry can be modified by pointing to it and by pressing the 'In-

Smart R8 Control  version 1.50                                Page 13 of 22

sert' (or 'Ins') key.  Similarly, the user may wish to add a new frequency

to the list, this may be done by pointing to the approximate location the

entry is to be inserted and pressing either the 'Ctrl' and 'Home' or 'Ctrl'

and 'End' keys.  'Ctrl-Home' will add the entry above the pointer, and

'Ctrl-End' will add the entry below the pointer.

Whether the user is modifying or adding a frequency to the list, the soft-

ware will then prompt the user to enter the new frequency.  If the user

simply presses 'Enter' in response to this prompt, the frequency that the

Drake R8 receiver is currently tuned to will serve as the default input

frequency.  The software will then prompt the user for the new target area

for this frequency, if known.  The user may elect to leave this blank.  Af-

ter completing input of all the modification parameters, the software will

prompt the user if he wishes to go ahead with the modification, as a safety

measure.

The user can also delete a frequency in the list.  This is done by pointing

to the frequency entry and pressing the 'Delete' or ('Del') key.  As be-

fore, the software will then prompt the user if he wishes to go ahead with

the frequency entry deletion, as a safety measure.

Whether the user is marking, modifying, adding or deleting a frequency en-

try in this window, the database will be changed.  This will be evidenced

by '*' in the upper left hand corner of the faceplate window.  The database

change will not become permanent until it is saved (F9) to disk.

F7 - Station's Full Program Schedule.

A particular station's full 24-hour broadcast schedule may be queried by

using the F7 function key.  If a mouse driver is present, this function can

be invoked by clicking on the 'F7' symbol in the help screen.  As with the

F6 function key, Smart R8 Control will prompt the user to provide the name

of the station to query.  The user may cancel this action by hitting the

'Esc' key, while the input prompt is displayed.  Once the station name is

entered, a scrollable window containing the full broadcast schedule will

appear along the bottom half of the full screen.  The format of the infor-

mation displayed will be similar to that found in the Shortwave Guide sec-

tion of Grove Enterprises' Monitoring Times magazine.  Frequencies previ-

ously marked are highlighted in the list to allow for planning purposes.

If the schedule contains more listings than are shown in the visible win-

dow, arrows will be displayed inside the left edge of the window.  Naviga-

tion of the schedule window can be accomplished with the up and down cur-

sor, up and down page, home, and end keys.

If a file copy of the schedule is desired, the query can be made by press-

ing the 'Shift' and F7 function keys, simultaneously (or by clicking on

that selection in the help window if a mouse driver is present).  The

schedule may be printed in the same manner, by either pressing 'Enter' in

response to the file name prompt, or by using the file name 'PRN'.  The

user may cancel either of these actions by hitting the 'Esc' key, while the

input prompt is displayed.  Since printed and file copies of the schedule

cannot highlight marked frequencies, per se, marked frequencies will be

followed by a '*' instead.  While the database is being scanned, a small

bar scale display will show the relative progress of the operation.

Smart R8 Control  version 1.50                                Page 14 of 22

If a mouse driver is present, the window may be paged by clicking in the

upper or lower portions of the scroll bar.  It may be scrolled, one line at

a time, by clicking on the arrows of the scroll bar.  The scroll bar is lo-

cated on the left hand side of the window.  Tabbing from this window to ei-

ther the station or frequency windows can be accomplished by clicking on

the bottom line of the screen.

Mouse users may now automatically invoke this function, by clicking on the

left-hand (time block) portion of any displayed station ID line in a Who

Else (F8) window.

Users may tune to any frequency in the full program schedule window.  This

will allow users to double check other frequencies used by that station at

other times of the day.  If a mouse driver is present, clicking on any of

the frequencies in the window will tune the radio to that frequency.  If

the target area for that frequency is annotated as 'uf' or 'lf', the radio

will automatically change to upper or lower sideband, respectively.  Any

other annotated target area will cause the radio to switch to the AM mode,

unless the target area is blank.  In this case, the demodulation mode of

the radio will not be changed.

F8 - Who Else Uses This Frequency.

The database typically contains entries for broadcasts in English.  If for-

eign language broadcasts are encountered, this will usually be made on a

frequency used for English broadcasts as well.  A listing of all stations

that will use the currently tuned frequency at any time of the day may be

accessed by pressing the F8 function key and entering the frequency in

question.  The prompt will default to the frequency currently tuned.  The

user may cancel this action by either hitting the 'Esc' key, or clicking on

'Esc', while the input prompt is displayed.  This function can also be in-

voked by clicking on the 'F8' symbol in the help screen if a mouse driver

is present.  This will produce a scrollable window containing a list of the

these stations, their times of operation on that frequency (in English),

and the target area for the broadcast.  The window will appear along the

bottom left half of the full screen.  Stations with a previously marked

frequency are highlighted in the list.  If the listing contains more sta-

tions than are shown in the visible window, arrows will be displayed inside

the left edge of the window.  Navigation of the schedule window can be ac-

complished with the up and down cursor, up and down page, home, and end

keys.

If a mouse driver is present, the window may be paged by clicking in the

upper or lower portions of the scroll bar.  It may be scrolled, one line at

a time, by clicking on the arrows of the scroll bar.  The scroll bar is lo-

cated on the left hand side of the window.  Tabbing from this window to ei-

ther the station or frequency windows can be accomplished by clicking on

the bottom line of the screen.

Mouse users may now automatically invoke this function, by clicking on the

left-hand portion of any undisplayed station ID line in the faceplate win-

dow (where the time block would normally be shown).

Smart R8 Control  version 1.50                                Page 15 of 22

Intuitive Database Searches.

In certain situations, a particular frequency may be tuned to for which the

station is not identified in the faceplate, or tuning to a parallel fre-

quency would be desired. Smart R8 Control has the capability for mouse us-

ers to perform _Intuitive Database Searches._  By this, certain areas of

the faceplate and Who Else (F8) windows have been reserved to invoke other

functions.  By clicking on the left-hand portion (time block) of any dis-

played station ID line in the faceplate will invoke the Find Station Fre-

quencies (F6) function.  Clicking in the same area, but for blank station

ID lines, will invoke the Who Else (F8) function.  Clicking on any time

block in the Who Else (F8) window will invoke the the Station's Full Pro-

gram Schedule (F7) function.  In each instance the functions are automati-

cally invoked with either the proper frequency or station name.  This alle-

viates the user from having to type them in, or press another key.

F9 - Save Changed Database.

Marking frequencies will change the database, as evidenced by a '*' dis-

played in the upper left corner of the faceplate window.  It is important

to note that the entire shortwave database is contained in memory (rather

than direct disk access) to make operations on it more efficient.  During

some operations, a changed database will automatically be saved to disk

(e.g., a shell to DOS and exiting the program).  This can be done manually,

by pressing the F9 function key.  If a mouse driver is present, clicking on

the '*' icon in the upper left hand corner will accomplish the same thing.

While the database is being saved, a small bar scale display will show the

relative progress of the operation.

Shift-F9 - Swap Databases (Registered Users Only).

Subscribers to FineWare's SWBC Schedules can import that database into a

format that Smart R8 Control can use.  Once converted, the user can swap

databases by pressing the Shift and F9 keys.  Upon pressing Shift-F9, the

user will be prompted to input whether the Smart R8 Control distribution

database (SRC) or SWBC Schedules (SKD) database is to be chosen.  Answering

the prompt with 'SRC' or 'SKD' will potentially swap that database into the

program.  The user may cancel this action by either hitting the 'Esc' key,

or clicking on 'Esc', while the input prompt is displayed.  If a mouse

driver is present, clicking on the 'Sh' symbol in the help screen will ac-

complish the same thing as pressing Shift-F9.  Also, the prompt may be an-

swered by clicking on 'SRC' or 'SKD'.

In many instances, the user may swap the database while either the DX Shows

(F3) or Available Stations (F5) windows are displayed.  Since each would

invariably change upon a database swap, these windows are automatically up-

dated.

Alt-F9 - Dump Database To Disk (Registered Users Only).

Major modifications to the database are performed by using a text editor.

In order to make major modifications to the database, the database must be

dumped in a text file format.  Pressing the Alt-F9 dumps the database to

disk in a text format.  This feature is restricted to registered users that

have the Smart R8 Control Database Utility, which then compiles this text

file back into a run-time format.  While the database is being dumped, a

small bar scale display will show the relative progress of the operation.

Smart R8 Control  version 1.50                                Page 16 of 22

If a mouse driver is present, clicking on the '@' symbol in the help screen

will accomplish the same thing.

If a dumped database already exisits, the user will be prompted with the

option to overwrite the text version of the database.  The user may con-

tinue by replying with 'Y' and hitting the 'Enter' key.  The overwrite ac-

tion may be cancelled by replying with an 'N' and pressign 'Enter', or by

hitting the 'Esc' key.  If a mouse driver is present, clicking on the 'Y',

'N' or 'Esc' will produce the desired result.

F3 - List Selected DX Shows.

The R8 Communications Receiver is especially designed for listening to pro-

grams.  Pressing the F3 function key displays a scrollable window along the

bottom half of the full screen, containing a list of selected programming

for the next 24 hours.  If a mouse driver is present, the DX Shows can be

presented by clicking on the 'F3' symbol on the help screen.  Each entry

displays the name of the station and UT time that the program is on.  A

header marking the start of a UT day is provided as a reference.  If the

station happens to be on at the time displayed in the faceplate window it

will be highlighted in the list, regardless of whether the show is actually

on.  This was implemented in this manner, because stations often shift pro-

gram times during the schedule season.  If the listing contains more pro-

grams than are shown in the visible window, arrows will be displayed inside

the left edge of the window.  Navigation of the program window can be ac-

complished with the up and down cursor, up and down page, home and end

keys.  If a mouse driver is present, the window may be paged by clicking in

the upper or lower portions of the scroll bar.  It may be scrolled, one

line at a time, by clicking on the arrows of the scroll bar.  The scroll

bar is located on the left hand side of the window.

By moving the pointer bar to a highlighted station and pressing the 'Enter'

key, the station's frequency list will be presented.  If a mouse driver is

present, clicking on a highlighted station will accomplish the same thing.

Selecting a station from the DX Shows window has the same effect as either

using the F5 key (and selecting a station) or using the F6 key to bring up

a station's frequency list.  The DX Shows window can be re-entered from any

other window by pressing the Shift and Tab (or Tab Left) key.  This will

allow the user to quickly jump back and forth between the DX Shows window

and the station frequency list window.

The database of programs is contained in the file 'DXSHOWS.TXT'.  As an ex-

ample, DXSHOWS.TXT mostly contains listings for radio oriented programming,

such as Glen Hauser's 'World of Radio'.  This file is a field-delimited

flat file that may be edited with a simple text editor.  The user may edit

this file, provided that the general format of the examples is adhered.

Another restriction is that the station name must match exactly what is in

the main shortwave database.

Note:  For queries prompting for a station name, the user may enter the

full or partial name of the station.  The entry must consist of a discrimi-

nating string.  For example, the user could enter 'BBC' or 'London' to

query 'BBC, London'.  However, using terms like 'Radio' or 'Voice' alone

should be avoided, because there are hundreds of stations that use these

terms.  This would result in a listing of thousands of entries, for which

Smart R8 Control  version 1.50                                Page 17 of 22

the software cannot support.  In all of these queries (except for F6-Find

Station Frequencies), the station name may be entered in either upper or

lower case.

OTHER FUNCTIONS.

F1 - Display Help Screen.

Most of the key commands used in Smart R8 Control will be easily memorized.

In the event that some reminder is required, a help screen containing all

of the key commands is provided.  The help screen is organized by keys per-

taining to radio operation, database query and other utilities.  In the

event that the F1 key cannot be associated with displaying the help screen,

a small reminder is positioned at the bottom of the receiver's faceplate

window.  If a mouse driver is present, the help screen doubles as a main

menu.  Any of the functions listed in the help screen can be invoked by

clicking on the key combination symbol.  In some instances (F5, F7 and F9),

single help screen entries refer to multiple similar functions.  Clicking

on the discriminating part of the key combination (Sh, @ or the function

key symbol alone) will invoke the most likely key combination.  For exam-

ple, clicking on the '@' part of the F5 (What is on Now) entry will invoke

the Alt-F5 key combination for displaying Utility Stations.

F2 - Display Shortwave Band Plan.

Despite the rulings of the World Administrative Radio Conference (WARC) and

the International Telecommunications Union (ITU), there are a plethora of

stations broadcasting outside the authorized broadcast bands.  By pressing

the F2 function key, a fixed window of the de facto band plan will be dis-

played in the lower right corner of the full screen.  If a mouse driver is

present, clicking on the F2 symbol in the help screen will accomplish the

same thing.  New in version 1.40, this feature can be alternatively invoked

by clicking on the faceplate's band indicator.  The frequency limits dis-

played are not necessarily hard and fast.  The VOIRI, Iran, still broad-

casts on the odd ball frequency of 9022 kHz, which is nowhere near the lim-

its of the 31 meter band.  However, the limits displayed represent where

the majority of shortwave broadcasters may be found.  It serves as a help-

ful reminder of what frequencies to search while using the stepped method

of tuning within Smart R8 Control.

Smart R8 Control has the capability to tune the receiver from the Band Plan

window.  This should be especially helpful when doing band scans.  After

invoking the Band Plan window, the user can tune the receiver to any fre-

quency in the window by clicking on it.  The user can then proceed to tune

either up or down the shortwave band as usual.

Alt-F - Run HF Propagation Prediction (Registered Users Only).

HF propagation is a varying medium.  Of the frequency entries provided in

the database, many are not to the listener's target area but may be audible

there.  In order to better predict which frequency will be the best audible

in a particular area, a link to the Smart R8 Control HF Propagation Predic-

tion Utility is provided within the main program.  This utility can be in-

voked by pressing the 'Alt-F' key combination.  If a mouse driver is pre-

Smart R8 Control  version 1.50                                Page 18 of 22

sent, clicking on the '@F' symbol in the help screen will accomplish the

same thing.

If 'Alt-F' is pressed when the user is either pointing to a particular sta-

tion in one of the 'Available Stations' windows, the HF Propagation Predic-

tion Utility will attempt to find that station, for an expedient calcula-

tion.  If 'Alt-F' is pressed while in the 'Frequency List' window, the

utility will point to this station, as well.

This feature is restricted to registered users that have the Smart R8 Con-

trol HF Propagation Prediction Utility.

NOTE:  Using this utility in this fashion will require an additional over-

head of approximately 127k base memory RAM, bringing the total memory re-

quirement to 532kb.  Without this additional overhead, the utility may not

run correctly.

Alt-U - Publish Database on Printer (Registered Users Only).

The Smart R8 Control Printing Utility has been integrated within the main

program.  This feature will allow users to publish the database, in a two-

column newspaper style fashion.  The database will automatically published

on the default printer, set within the Setup Utility.  This utility can be

invoked by pressing the `Alt-U' key combination.  If a mouse driver is pre-

sent, clicking on the `@U' symbol in the help screen will accomplishg the

same thing.

This feature is restricted to registered users that have the Smart R8 Con-

trol Printing Utility.

NOTE:  Using this utility in this fashion will require an additional over-

head of approximately 103k base memory RAM, bringing the total memory re-

quirement to 508kb.  Without this addiitonal overhead, the utility may not

run correctly.

Alt-L - Enter Log Information.

One of the mundane tasks of the listening hobby is the logging of activity.

Smart R8 Control automatically stamps the time, date and frequency on all

entries.  Upon pressing Alt-L, Smart R8 Control prompts the user for a log

entry.  If a mouse driver is present, clicking on the '@L' symbol in the

help screen accomplishes the same thing.  Any additional information may be

typed here, such as service, call, etc.  The entry is concluded with the

'ENTER' key.  All entries are posted in the file 'SMARTCON.LOG'.

Smart R8 Control prompts users to enter the station name and SINPO rating,

before any other notes or comments.  If a mouse driver is present, the user

can automatically invoke the logging facility.  This can be accomplished by

clicking on the right-hand (station name) portion of the station ID in ei-

ther the faceplate or Who Else (F8) windows.  This will automatically in-

sert the appropriate station name when prompted.  If the user clicks on the

right-hand portion of a blank station ID line in the faceplate, no station

name will be inserted.  In this case, the logging facility will perform

normally.

Smart R8 Control  version 1.50                                Page 19 of 22

F4 - Display Log Information.

It may be desirable to re-display old log information for additional refer-

ence.  By pressing the F4 function key, a scrollable window is displayed,

containing all information accumulated in the log file 'SMARTCON.LOG'.  If

a mouse driver is present, clicking on the 'F4' symbol in the help screen

will accomplish the same thing.  The window can then be paged by clicking

in the upper or lower portions of the scroll bar.  It may be scrolled, one

line at a time, by clicking on the arrows of the scroll bar.  The scroll

bar is located on the left hand side of the window.

Alt-D - Demo Mode.

Because the serial interface of the R8 requires a hardware handshake, it is

imperative that the receiver be hooked up to the computer while the program

is running.  If Smart R8 Control is to be run for database access only, the

program can be put into 'Demo Mode' from within the program.  This will

disengage the hardware handshake, and is invoked by pressing Alt-D.  The

receiver must be off and the program must recognize that the receiver is

off before this function can be invoked.  Smart R8 Control has the addi-

tional capability to automatically sense if the receiver (or any serial de-

vice) is present on the serial port.  If a receiver is not present when ei-

ther starting the program, or returning from a DOS Shell, the program will

automatically enter Demo Mode.

Alt-Q - Clear Screen.

Sometimes it is desirable to unclutter the screen.  Pressing Alt-Q will

clear all windows from the screen, with the exception of the receiver face-

plate window.  If a mouse driver is present, clicking on the '@Q' symbol in

the help screen will accomplish the same thing.

F10 - Shell to DOS.

Pressing the F10 function key will automatically save the database (if it

has changed) and temporarily put the user in hands of DOS.  This allows the

user to edit the log file with a small text editor of his/her choosing.

Pressing F10 could also be used to free up the serial line to use a termi-

nal unit (TU) or terminal node controller (TNC) with a separate terminal

program.  It is recommended that the user remember that this is only a

child process of DOS and Smart R8 Control is still dominating a chunk of

memory.  While shelled out to DOS, the user must type 'EXIT' to return to

Smart R8 Control.  A small reminder of this has been inserted into the DOS

prompt while in the DOS shell.  If a mouse driver is present, the DOS Shell

can be invoked by clicking on the 'F10' symbol in the help screen.

NOTE:  Shelling to DOS will require an additional overhead of base memory

RAM, equivalent to the memory requirements of your particular DOS version.

Without this additional memory overhead, shelling to DOS is not possible.

The memory left over when shelling may not be enough to run some software.

If the user is planning to run software from within the shell, please note

the remaining memory using the MEM command (DOS 5.0, 4DOS).  Refer to the

software's documentation it's memory requirements for a comparison.

Alt-X - Exit Program.

Pressing Alt-X will automatically save the database (if it has changed),

save the current settings to 'SMARTCON.DAT' and return the user to DOS.

Since this program alters certain computer parameters, it is important that

Smart R8 Control  version 1.50                                Page 20 of 22

the user ONLY exit the program in this manner.  If for some reason the pro-

gram hangs, the best solution is to reboot the machine with Ctrl-Alt-Del.

Pressing Ctrl-Break to 'break' out of Smart R8 Control should be avoided.

If a mouse driver is present, the program can be exited by clicking on the

'@X" symbol in the help screen.

EXPECTED FUTURE ENHANCEMENTS.

Capability to create, save and print profiles of 100 or 440 active frequen-

cies.  This will allow users to retrieve profiles from disk, and load

the radio's memories with it.

Capability to produce a sequential list of all frequencies that a particu-

lar station uses, and the capability to tune the receiver from this

list.

Capability to store and recall VHF frequencies from the integrated fre-

quency database.

CONCLUSION.

The author would appreciate any comments regarding the improvement of this

software or any bug reports (nobody's perfect).

The author may be contacted via email at mfine@crosslink.net.  The author

also maintains a World Wide Web page at http://www.crosslink.net/~mfine for

users of Smart R8 Control, as well as other FineWare products.  It is an-

ticipated that users will be able to use the page for rapidly acquiring

software updates, information on Smart R8 Control developments as well as a

means to exchange up-to-date shortwave data.

In addition, the author may also be contacted by mail at his home address

(phone calls are discouraged):

MARK J. FINE

c/o FineWare

11252 Cardinal Drive

Remington, Virginia 22734-2032

Comments should be constructive and should contain detailed descriptions of

what the user would like to see included in this software.  If a bug report

is in order, please include information such as computer make and model,

version and make of DOS used, a hard copy of the user's AUTOEXEC.BAT and

CONFIG.SYS files, as well as detailed descriptions of the nature of each

problem.  All constructive comments and bug reports will result in a

prompt, courteous response.

Smart R8 Control  version 1.50                                Page 21 of 22

ACKNOWLEDGMENTS.

* The Drake R8 Communications Receiver is a product of the R. L. Drake Com-

pany (Drake), Miamisburg, Ohio 45342.

* The format of the shortwave database used in Smart R8 Control is based

upon the Shortwave Guide column found in Monitoring Times magazine.  Moni-

toring Times is a monthly publication of Grove Enterprises, Inc., 140 Dog

Branch Road, Brasstown, North Carolina, 28902-0098.  Special credit is

given to Bob Grove, Publisher, for permission to reproduce this format.

Smart R8 Control  version 1.50                                Page 22 of 22